| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C154 |
| --- | --- |

| Title: | Reference samples sub-sampling for SDC and DMM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides a reference samples sub-sampling mechanism for the calculation of "predDCVal" used at SDC and DMM. Both test cases at common test condition and All intra condition are verified. Experimental results show that the proposed method does not have the impact on the coding performance.

# Introduction

At the depth map coding framework, DMM and SDC are two effective coding tools for intra coding. Both tools need to calculate the DC value ("predDCVal") from the reference samples (or so-called neighbouring samples). The value is calculated by the sum of the neighbouring samples depth values (sumNeigh) and the sum of the neighbouring samples (numNeigh). For DMM case, reference samples from the current coding block's above row and left row is used to derive sumNeigh and numNeigh that are calculated as pseudo code below.

    - For x = 0..nT− 1 the following applies.

      - When partitionPattern[ x ][ 0 ] is equal to X, the following applies:

sumNeigh += p[ x ][ −1 ]	
numNeigh += 1

    - For y = 0..nT− 1 the following applies.

      - When partitionPattern[ 0, y ] is equal to X, the following applies:

sumNeigh += p[ −1 ][ y ]	
				   numNeigh += 1

For SDC case, the prediction samples (predSamples) of the current coding block are used to derive sumNeigh and numNeigh that are calculated as pseudo code below.

    - For y = 0..nT− 1

    - For x = 0..nT− 1 the following applies.

      - When partitionPattern[ x ][ 0 ] is equal to X, the following applies:

sumNeigh += predSamples[ x ][ −1 ]	
 numNeigh += 1

To reduce the computing complexity, this contribution proposes a sub-sampling method for the calculation of sumNeigh and numNeigh in SDC and DMM.

# Sub-sampling for reference samples in SDC and DMM

According to DMM and SDC designs, the number of computing operations for 32x32 and 64x64 coding blocks are considerable huge. The table 1 summaries the computing operations for those blocks.

Table 1: Summary of computing operations for DMM and SDC

|  | DMM |  | SDC |  |
| --- | --- | --- | --- | --- |
|  | addition | "if..else" judgement | addition | "if..else" judgement |
| 32 x 32 block | 128 | 64 | 2048 | 1024 |
| 64 x 64 block | 256 | 128 | 8192 | 4096 |

Considering the large coding blocks, e.g. 32x32 or 64x64 block, always contain smooth area with less texture information, this contribution proposes a sub-sampling method to derive the samples from the neighbouring blocks and use those data to calculate sumNeigh and numNeigh in SDC and DMM. Figure 1 illustrates the sub-sampling method. Only the samples marked as grey colour is used for the calculation in DMM and SDC. Table 2 summaries the computing operations when 2:1 sub-sampling applied is applied. As it can be seen from Table 2, the number of the computing operations is reduced significantly.

Figure 1: 2:1 sub-sampling for the reference samples

Table 2: Summary of computing operations for DMM and SDC (2:1 sub-sampling is applied)

|  | DMM |  | SDC |  |
| --- | --- | --- | --- | --- |
|  | addition | "if..else" judgement | addition | "if..else" judgement |
| 32 x 32 blocks | 64 | 32 | 512 | 256 |
| 64 x 64 blocks | 128 | 64 | 2048 | 1024 |

# Experimental results

The proposed method is implemented on HTM5.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. The 2:1 sub-sampling is applied to the neighbouring samples at the calculation of DC value ("predDCVal") in DMM and SDC when block size is equal to 32 or 64. The table 1 and table 2 show the results that only apply the sub-sampling to SDC. Table 3 and table 4 show the results that apply the sub-sampling to both SDC and DMM.

Table 1: 2:1 sub-sampling for SDC (CTC)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99.7% | 96.2% | 97.7% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.04% | -0.03% | 100.5% | 99.8% | 100.7% |
| Newspapercc | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 100.3% | 101.6% | 99.3% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 0.00% | -0.04% | -0.03% | 99.9% | 99.2% | 99.5% |
| PoznanHall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.06% | 0.04% | 100.2% | 99.9% | 100.4% |
| PoznanStreet | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.01% | 95.3% | 102.5% | 102.1% |
| UndoDancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.06% | -0.04% | 95.2% | 98.9% | 98.3% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 100.2% | 99.2% | 99.3% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 97.6% | 100.1% | 100.0% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 98.7% | 99.7% | 99.7% |

Table 2: 2:1 sub-sampling for SDC (All - intra)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.03% | 100.2% | 97.7% | 98.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.02% | 99.7% | 99.8% | 99.2% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 100.1% | 99.9% | 99.9% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 101.1% | 98.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.01% | 100.0% | 97.7% | 99.8% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.00% | 99.7% | 100.3% | 101.2% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.4% | 100.4% | 101.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 100.0% | 99.1% | 99.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.01% | 99.8% | 99.8% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.01% | 99.9% | 99.5% | 99.8% |

Table 3: 2:1 sub-sampling for SDC and DMM (CTC)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 100.0% | 98.3% | 98.1% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.05% | -0.03% | 100.6% | 102.6% | 100.1% |
| Newspapercc | 0.00% | 0.00% | 0.00% | 0.00% | -0.05% | -0.04% | 100.5% | 104.1% | 99.7% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 0.00% | -0.02% | -0.01% | 99.9% | 101.6% | 99.7% |
| PoznanHall2 | 0.00% | 0.00% | 0.00% | 0.00% | -0.19% | -0.13% | 100.1% | 102.9% | 101.4% |
| PoznanStreet | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 95.7% | 105.1% | 103.6% |
| UndoDancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.06% | -0.04% | 95.5% | 98.8% | 99.2% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | -0.02% | 100.4% | 101.6% | 99.3% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.07% | -0.05% | 97.8% | 102.1% | 101.0% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.05% | -0.04% | 98.9% | 101.9% | 100.3% |

# Changes to the description of Test Model

- Depth partition value derivation and assignment process

Inputs to this process are:

- neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- an binary array partitionPattern[ x ][ y ], with x, y =0..nT−1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

- a variable nT specifying the transform size,

- a flag dcOffsetAvailFlag, specifying whether DC Offset values are available

- the variables quantDcOffsetP0 and quantCcOffsetP1, specifying the DC offsets for the block partitions

Output of this process is:

- predictedsamples predSamples[ x ][ y ], with x, y =0..nT−1.

For X being replaced by 0 and 1, the values of theprediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - If nT is equal to 32 or nT is equal to 64, nStep is set to 2; otherwise, nStep is set to 1.

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..nT− 1, x += nStep the following applies.

      - When partitionPattern[ x ][ 0 ] is equal to X, the following applies:

- sumNeigh += p[ x ][ −1 ]		(G-1)
numNeigh += 1		(G-2)

    - For y = 0..nT− 1, y += nStep the following applies.

      - When partitionPattern[ 0, y ] is equal to X, the following applies:

- sumNeigh += p[ −1 ][ y ]		(G-3) 
numNeigh += 1		(G-4)

# Conclusion

This contribution proposes a reference sample sub-sampling method that can significant reduce the number of computing operations for DMM and SDC. The 2:1 sub-sampling is used at the experiments. The number of computing operations for SDC can be reduced to one forth of the original's. The computing operations for DMM can be reduced by half. The test results for CTC and All-Intra configurations show that no impact on the coding performance. Therefore, it is recommended to incorporate the proposed sub-sampling solution into next version of 3D-HEVC Test Model and HTM6.

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).