| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0155 |
| --- | --- |

| Title: | CE6.H related: Simplification of SDC signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution suggests to unify and simplify the bitstream structure for SDC syntax and conventional intra mode syntax. The unified signalling structure can reduce decoder parsing burden, clean-up the syntax description in the 3D-HEVC specification, and reduce the number of context model for SDC. Both test cases at common test condition and All intra condition are verified. Experimental results show that the proposed method does not have the impact on the coding performance for CTC, and have minor negative impact for All-intra case.

Besides the simplification of SDC, this contribution also fixes a encoder bug for SDC that can reduce encoding time and does not change test results.

# Introduction

Simplified Depth Coding (SDC) was adopted at the second JCT3V meeting [1]. It exploits the features of depth map that only code one residual value for one segment and use DLT (Depth Lookup Table) to code the residual value. Four optimal prediction modes are selected for SDC design. To signal SDC syntax, the current design need to code "sdc_flag" to indicate the usage of SDC, and "sdc_pred_mode" to specify the intra mode for SDC decoding process, and related syntax for DMM when DMM is used at SDC. The details of syntax description is list below (marked as blue colour).

| if( PredMode = = MODE_INTRA &&DepthFlag ) |  |
| --- | --- |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( sdc_pred_mode = = 2) { |  |
| dmm_delta_end_flag[ x0 ][ y0 ] | ae(v) |
| if( dmm_delta_end_flag[ x0 ][ y0 ] ) { |  |
| dmm_delta_end_abs_minus1[ x0 ][ y0 ] | ae(v) |
| dmm_delta_end_sign[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| .............. |  |

The current SDC signalling method is independent to the signalling for the conventional intra mode, e.g. DC, planar, intra_angular modes and DMM. It adds additional "if...else..." clause to the syntax. This contribution suggests to unify the syntax of SDC and conventional intra modes that can simplify the bitstream structure. Moreover, an encoder bug is also fixed in this document that can reduce the encoding time.

# Simplification of SDC signalling

The four selected intra modes for SDC are exactly belong to the conventional intra modes. Moreover, SDC process is performed after the derivation of intra prediction samples by the specified intra mode. Therefore, it is unnecessary to independent signal SDC syntax from the view of the decoding process. This contribution suggests to move SDC signalling flag "sdc_flag" to the place behind the signalling of the intra mode. The modifications of the syntax can be found at the part "Changes to the description of Test Model".

Based on the move of the position of "sdc_flag", the syntax "sdc_pred_mode" can be removed as the specified intra mode used at SDC has been signalled by the intra mode syntax. Therefore, the related contexts for "sdc_pred_mode" can also be removed.

Moreover, the current context selection for "sdc_flag" is based on the left and above coding blocks' "sdc_flag" values. From the preliminary tests, the usage of SDC is highly depended on the selected intra mode. Therefore, this contribution also changes the context selection for "sdc_flag". The new selection is based on the intra modes that is described as the following pseudo code. Base on the design describe in the pseudo code, the number of context model for "sdc_flag" can be reduced to 2.

UInt uiCtxSDCFlag = (intraPredMode == DC_IDX || intraPredMode == PLANAR_IDX) ? 0 : 1;

# Encoder bug-fixed for SDC

In HTM5.0, RDO search for the conventional intra prediction and SDC is performed in the function TEncSearch::estIntraPredQT. The pseudo code is list below:

for( UInt uiSDC=0; uiSDC<=uiUseSDC; uiSDC++ )

{

for( UInt uiRes = 0; uiRes<=uiUseSDC; uiRes++ )

{

if(uiSDC == 1)

{

.............

xIntraCodingSDC(pcCU, uiPartOffset, pcOrgYuv, pcPredYuv, uiPUDistY, dPUCost, (uiRes==1));

.............

}

else

{

.............

xRecurIntraCodingQT( pcCU, uiInitTrDepth, uiPartOffset, bLumaOnly, pcOrgYuv, pcPredYuv,  pcResiYuv, uiPUDistY, uiPUDistC, true, dPUCost, bZeroResi );

.............

}

}

}

"uiUseSDC" equals to "0" means that the conventional intra prediction is used. The encoding of the residual value for SDC contains two cases: 1. the value isn't equal to 0 (uiRes == 1); 2. the value is equal to 0 (uiRes == 0). In the RDO process for the conventional intra prediction, this methodology isn't applied. However, the current software has also tested these two cases (uiRes == 1 and uiRes == 0) and thereby caused the redundant searches.

Based on the above analysis, this contribution suggests to add the below condition (marked by the yellow colour) to remove the redundant searches.

for( UInt uiSDC=0; uiSDC<=uiUseSDC; uiSDC++ )

{

for( UInt uiRes = 0; uiRes<=uiUseSDC; uiRes++ )

{

if ( uiSDC == 0 && uiRes == 1 )

{

continue;

}

if(uiSDC == 1)

{

.............

xIntraCodingSDC(pcCU, uiPartOffset, pcOrgYuv, pcPredYuv, uiPUDistY, dPUCost, (uiRes==1));

.............

}

else

{

.............

xRecurIntraCodingQT( pcCU, uiInitTrDepth, uiPartOffset, bLumaOnly, pcOrgYuv, pcPredYuv,  pcResiYuv, uiPUDistY, uiPUDistC, true, dPUCost, bZeroResi );

.............

}

}

}

# Experimental results

The proposed method is implemented on HTM5.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. During the test, the encoder bug-fixed is also applied. The table 1 and table 2 show the results for CTC and All-Intra configurations. The results show that the proposed method has no impact on coding performance for CTC, and minor negative impact on coding performance for All-Intra. The reduction of encoding time is due to the encoder bug-fixed.

Table 1: Simplification of SDC signalling + encoder bug-fixed (CTC)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.06% | 0.06% | 97.8% | 97.3% | 98.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.02% | 98.2% | 100.5% | 100.7% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.00% | 0.09% | 0.06% | 97.8% | 104.0% | 99.7% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.00% | -0.06% | -0.03% | 98.1% | 100.7% | 99.8% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.05% | -0.04% | 99.0% | 99.6% | 101.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 93.6% | 101.9% | 103.8% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.17% | -0.12% | 93.5% | 98.4% | 98.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.04% | 0.04% | 97.9% | 100.6% | 99.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.06% | -0.04% | 96.0% | 100.1% | 100.8% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.01% | 96.8% | 100.3% | 100.2% |

Table 2: Simplification of SDC signalling + encoder bug-fixed (All-Intra)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.55% | 0.42% | 81.3% | 97.3% | 102.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.68% | 0.52% | 82.3% | 95.7% | 101.8% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.00% | 0.50% | 0.37% | 79.0% | 99.9% | 101.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.00% | 0.26% | 0.21% | 80.2% | 95.0% | 102.0% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.42% | 0.35% | 81.2% | 98.6% | 102.3% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.00% | 0.23% | 0.17% | 78.0% | 96.5% | 104.8% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.11% | 0.08% | 78.8% | 101.6% | 103.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.58% | 0.44% | 80.9% | 97.6% | 101.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.25% | 0.20% | 79.6% | 97.9% | 103.2% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.39% | 0.30% | 80.1% | 97.8% | 102.6% |

# Changes to the description of Test Model

#1 Modification:

| if( PredMode = = MODE_INTRA &&DepthFlag ) |  |
| --- | --- |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( sdc_pred_mode = = 2) { |  |
| dmm_delta_end_flag[ x0 ][ y0 ] | ae(v) |
| if( dmm_delta_end_flag[ x0 ][ y0 ] ) { |  |
| dmm_delta_end_abs_minus1[ x0 ][ y0 ] | ae(v) |
| dmm_delta_end_sign[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( enable_DMM_flag && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + i ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 )<= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( PredMode = = MODE_INTRA &&DepthFlag && (IntraPredMode[ xB ][ yB ] == 0 ¦¦ IntraPredMode[ xB ][ yB ] == 1 ¦¦ IntraPredMode[ xB ][ yB ] == 35 ¦¦ IntraPredMode[ xB ][ yB ] == 41) ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if (sdc_flag[ x0 ][ y0 ] == 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |

#2 Modification:

sdc_pred_modeshall be one of the values shown in Table G-1. sdc_pred_modespecifies the intra prediction mode used for simplified depth coding.

Table G-1–Interpretation of sdc_pred_mode

| sdc_pred_mode | Associated Intra Prediction Mode |
| --- | --- |
| 0 | Intra_DC |
| 1 | Intra_DepthPartition( 35 ) |
| 2 | Intra_DepthPartition( 41 ) |
| 3 | Intra_Planar |

#3 Modification:

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

- If sdc_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows.

  - If sdc_pred_mode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 1.

  - Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 35.

  - Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 41.

  - Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 3, IntraPredMode[ xB ][ yB ] is set equal to 0.

- Otherwise, if dmm_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows:

IntraPredMode[ xB ][ yB ] = dmm_mode[ xB ][ yB ] + 35	(G-1)

Otherwise....................

#4 Modification

Table G-2 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | dmm_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_1_abs dmm_dc_2_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | res_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | mvp_l0_idx mpv_l1_idx | Error: Reference source not found |  | 0..1 | 2..3 |
|  | sdc_flag | Error: Reference source not found | 0..1 | 2..4 | 5..6 |
|  | sdc_residual_flag | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Error: Reference source not found | 0..19 | 20..39 | 40..59 |
|  | sdc_pred_mode | Table G-3 | 0..2 | 3..5 | 6..8 |

Table G-3 – Values of variable initValue for sdc_pred_mode ctxIdx

| Initialization variable | sdc_pred_mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 9 | 255 | 70 | 0 | 9 | 255 | 70 | 0 | 9 |

# Conclusion

This contribution proposes a SDC signaling simplification solution that unify the bitstream structure for SDC and conventional intra mode syntax. The syntax of "sdc_pred_mode" and related contexts are removed. The context selection for "sdc_flag" is also changed. Those modifications can reduce the "if...else..." clause that reduce the decoder parsing burden. The text for 3D-HEVC specification can also be condensed and tidy. Moreover, the encoder bug-fixed can reduce the encoding run time and doesn't change the test results

The experimental results show that no impact on the coding performance for CTC configuration. The encoding run time is significant reduced for All-Intra case. Therefore, it is recommended to incorporate the proposed SDC signaling solution into next version of 3D-HEVC Test Model and HTM6, and incorporate the encoder bug-fixed to HTM6.

# Reference

[1] Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, Shanghai, CN, Oct. 2012.

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).