| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0156r1 |
| --- | --- |

| Title: | MVC+D HLS: On unpaired MVD and field coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela and Dmytro Rusanovskyy Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes asserted clean-ups and bug fixes related to the unpaired multiview-video-plus-depth (MVD) support and field coding. It also proposes that when an access unit contains coded depth field view components, they shall be of the same parity.

Revision 1 of the document attempts to reflect the decisions of JCT-3V made 19th January, 2013.

# Proposals

## Proposed change 1

Motivation for the proposed change: As a consequence on the adoption of the unpaired MVD support from contribution JCTVC-B0147, a subset sequence parameter set RBSP structure no longer includes a seq_parameter_set_mvc_extension for the Multiview Depth High profile (having profile_idc equal to 138), but includes a seq_parameter_set_3dvc_extension syntax structure as can be observed from the syntax in 7.3.2.1.3 of the MVC+D specification. The following sentence in the semantics accidentally referred to the subset sequence parameter set RBSP syntax before the support for unpaired MVD was added and is therefore proposed to be removed.

- I.7.3.2.1.4	Sequence parameter set MVC extension syntax

The syntax table is specified in subclause H.7.3.2.1.4.

When profile_idc is equal to 138 in active sequence parameter, the seq_parameter_set_mvc_extension( ) is applicable to the depth views and 3DV operation points.

## Proposed change 2

Motivation for the proposed change: We attempted to clarify the paragraph by formulating the constraints using the same structure for the sentences. Phrasing “the depth view component with the same view_id value, if present in the access unit” attempts to capture the fact that, due to unpaired MVD, texture views are not always accompanied by depth views.

- I.7.4.1.2.5	Order of VCL NAL units and association to coded pictures

The specification of subclause H.7.4.1.2.5 applies with following modifications.

Each VCL NAL unit is part of a coded picture.

Let voIdx be the value of VOIdx of any particular VCL NAL unit. The order of the VCL NAL units within a coded picture is constrained as follows:

- For all VCL NAL units following this particular VCL NAL unit, the value of VOIdx shall be greater than or equal to voIdx.

- All VCL NAL units for a depth view component, if present, shall follow any VCL NAL unit of a texture view component with a same value of VOIdx.

For each set of VCL NAL units within a texture or depth view component, the following applies:

–	If arbitrary slice order, as specified in Annex Error: Reference source not found, subclause Error: Reference source not found or subclause I.10, is allowed, coded slice NAL units of a view component may have any order relative to each other.

- Otherwise (arbitrary slice order is not allowed), coded slice NAL units of a slice group shall not be interleaved with coded slice NAL units of another slice group and the order of coded slice NAL units within a slice group shall be in the order of increasing macroblock address for the first macroblock of each coded slice NAL unit of the same slice group.

If a coded texture field view component with a particular view_id is either the first field view component of a complementary field pair or a non-paired field, the first depth field view component of complementary field pair, the first depth field view component of non- paired field or a depth frame view component with the same view_id shall be present in the access unit of that coded texture field view component. If a coded texture field view component with a particular view_id is the second field view component of a complementary field pair or non-paired field, a first depth field view component of complementary field pair or a non-paired field with the same view_id shall not be present in the access unit of that coded texture field view component. If a coded texture view component with a particular view_id is a frame view component, a depth frame view component with the same view_id shall be present in the access unit of that coded texture frame view component.

The following applies:

–	If a coded texture view component with a particular view_id is the first field view component of a complementary field pair, the depth view component with the same view_id value, if present in the access unit, shall be a coded frame view component or the first field view component of a complementary field pair.

–	Otherwise, if a coded texture view component with a particular view_id is the second field view component of a complementary field pair, the depth view component with the same view_id value, if present in the access unit, shall be the second field view component of a complementary field pair.

–	Otherwise, if a coded texture view component with a particular view_id is a non-paired field, the depth view component with the same view_id value, if present in the access unit, shall be a coded frame view component or a non-paired field.

–	Otherwise (a coded texture view component with a particular view_id is a coded frame), the depth view component with the same view_id value, if present in the access unit, shall be a coded frame view component.

NAL units having nal_unit_type equal to 12 may be present in the access unit but shall not precede the first VCL NAL unit of the primary coded picture within the access unit.

NAL units having nal_unit_type equal to 0 or in the range of 24 to 31, inclusive, which are unspecified, may be present in the access unit but shall not precede the first VCL NAL unit of the primary coded picture within the access unit.

NAL units having nal_unit_type in the range of 22 to 23, inclusive, which are reserved, shall not precede the first VCL NAL unit of the primary coded picture within the access unit (when specified in the future by ITU-T | ISO/IEC).

## Proposed change 3

Motivation for the proposed change: Phrasing “and VOIdx equal to i” is added to clarify the relation of the loop entries (view_id[ i ], depth_view_present_flag[ i ], and texture_view_present_flag[ i ]) and the entry counter i to the VOIdx value.

- I.7.4.2.1.4	Sequence parameter set 3DVC extension semantics

The semantics specified in subclause H.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component and with the following additions:

depth_view_present_flag[ i ] equal to 0 specifies that there is no depth view having a view_id equal to view_id[ i ] and VOIdx equal to i. depth_view_present_flag[ i ] equal to 1 specifies that there is a depth view having a view_id equal to view_id[ i ] and VOIdx equal to i.

texture_view_present_flag[ i ] equal to 0 specifies that there is no texture view having a view_id equal to view_id[ i ] and VOIdx equal to i. texture_view_present_flag[ i ] equal to 1 specifies that there is a texture view having a view_id equal to view_id[ i ] and VOIdx equal to i. If depth_view_present_flag[ i ] is equal to 0, texture_view_present_flag[ i ] shall be equal to 1.

## Proposed change 4

Motivation for the proposed change: MVC+D does not require the field_pic_flag and bottom_field_flag to be identical among all depth view components of the same access unit. In other words, it is currently allowed by MVC+D to have some depth view components coded as frames and others coded as fields and/or some depth view components coded as top fields and some others as bottom fields within the same access unit. We do not see a need for such flexibility. Moreover, it is unclear to how inter-view prediction would operate in such a situation. Furthermore, as far as we are aware, experiments have been performed only with a coding configuration where depth view components have the same values of field_pic_flag and bottom_field_flag within an access unit. It is proposed to require the values of the field_pic_flag and bottom_field_flag to be unchanged for all depth view components of an access unit.

- I.7.4.3	Slice header semantics

The semantics specified in subclause H.Error: Reference source not found apply with the substitution of texture view component (for nal_unit_type 1, 5, and 20) or depth view component (for nal_unit_type 21) for view component and with the following modifications.

When nal_unit_type is equal to 1, 5, or 20, all constraints specified in subclause H.Error: Reference source not found apply only to the texture view components with the same value of VOIdx. When nal_unit_type is equal to 21, all constraints specified in subclause H.Error: Reference source not found apply only to the depth view components with the same value of VOIdx.

The value of the following 3DVC sequence parameter set syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, and 20 of an access unit: chroma_format_idc.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, and 20 of an access unit: field_pic_flag and bottom_field_flag.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type 21 of an access unit: field_pic_flag and bottom_field_flag.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).