| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0157 |
| --- | --- |

| Title: | 3D-CE5.h: Cross-check report of JCT3V-C0045 on Merge candidates derivation from disparity vector shifting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Christine Guillemot Laurent Guillo | Tel: Email: | +33 299847100 Christine.guillemot@inria.fr Laurent.guillo@inria.fr |
| Source: | INRIA, France. |  |  |

_____________________________

# Abstract

This document is a crosscheck of the proposal JCT3V-C0045. In this proposal, up to two new candidates can be added in the candidate merge list. They are derived from disparity vectors with a fixed horizontally shift. The software provided by the proponents appears to match the description of the method detailed in JCT3V-C0045. Tests have been run and the results match those provided by the proponents.

# Introduction

The authors of JCT3V-C0045 [1] propose to modify the candidate merge list in adding up to two new candidates. They are derived from the disparity motion vector candidate in either adding 4 (for the left candidate) or subtracting 4 (for the right candidate) to the horizontal component of the disparity vector. The vertical component is always set to 0.

The left candidate is inserted before the temporal merging candidate whereas the right candidate is inserted after the temporal candidate. In this way, the temporal candidate is preserved.

# Source code verification

The proponents provided INRIA with the source code to be tested. It is based on HTM 5.0.1 [4] and the new or modified source code is activated when the macro ”QC_MRG_CAN_SHIFTEDDV”  is set to 1.

Two methods are modified:

- TComDataCU::getInterMergeCandidates in the file TComDataCU.cpp

- TComDepthMapGenerator::getPdmMergeCandidate in the file TComDepthMapGenerator.cpp

Modifications are clearly identified and correspond either to new code or to extension of source codes related to the proposal JCT3V-B0048 [2].

The software matches the description of the proposal.

The source codes have been successfully compiled using Linux (Fedora Core 13 64 bits) with gcc 4.4.5.

Tests were run on a cluster grid with the following characteristics:

- CPU: 2xIntel(R) Xeon (R) E5640 @ 2.67GHz, 4 cores

- RAM : 12Go per core.

No memory leaks were found.

# Results

This section provide results of the proposed method in comparison with the 3D-HTM anchor. The 3D-HTM 5.0.1 [4] is utilized and the method of the proponent is implemented on top of it. All tests are run based on the common tests conditions [5].

Table 1: Coding gains of JCT3V-C0045 with respect to anchor for 3-view case

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | -0,3% | -0,1% | 0,0% | -0,1% | 100,5% | 101,9% | 101,4% |
| Kendo | 0,0% | -0,8% | -0,8% | -0,4% | -0,3% | -0,3% | 100,6% | 100,7% | 99,9% |
| Newspapercc | 0,0% | -0,6% | -0,4% | -0,2% | -0,2% | -0,3% | 100,6% | 97,5% | 105,9% |
| GhostTownFly | 0,0% | -0,4% | -0,3% | -0,1% | -0,1% | -0,1% | 101,1% | 101,9% | 99,9% |
| PoznanHall2 | 0,0% | 0,0% | -0,6% | -0,1% | -0,1% | -0,1% | 101,2% | 98,0% | 103,9% |
| PoznanStreet | 0,0% | -0,4% | -0,3% | -0,1% | -0,1% | -0,1% | 101,6% | 99,7% | 100,8% |
| UndoDancer | 0,0% | -0,5% | -0,5% | -0,2% | -0,4% | -0,3% | 100,8% | 100,5% | 99,9% |
| 1024x768 | 0,0% | -0,5% | -0,5% | -0,2% | -0,2% | -0,2% | 100,6% | 100,0% | 102,4% |
| 1920x1088 | 0,0% | -0,3% | -0,4% | -0,2% | -0,2% | -0,2% | 101,2% | 100,0% | 101,1% |
| average | 0,0% | -0,4% | -0,4% | -0,2% | -0,2% | -0,2% | 100,9% | 100,0% | 101,7% |

Comments about the results:

- The obtained bit rate gains match those provided by the proponent.

- Coding time is a bit higher than the one provided by the proponent but added complexity remains negligible.

- This proposal performs better with the class C than with the class A. Gains with Kendo are relevant (0.8% for both side views and 0.4% reduction for the overall bitrate).

Additional comments:

- The size of the candidate merge is still at the most equal to 6.

- Preserving the temporal candidate when new candidates are added is relevant especially with videos of the class C which have mainly an horizontal shift ( it is probably less relevant with Ghost Town Fly which zooms in).

- It would have been interested to know what would have been performances if the two shifted disparity vectors (LDV and RDV) were derived from the disparity vector with other values than 16 or were even equal to the DV.

# Conclusion

The proposal JCT3V-C0045 provides an overall bit rate gain of 0.2% with no added complexity.

# References

- L. Zhang, Y. Chen, M. Karczewicz, "3D-CE5.h: Merge candidates derivation from disparity vector shifting", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0045, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- L. Zhang, Y. Chen, "3D-CE5.h: Merge candidates derivation from disparity vector", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0048, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, ", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1005, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.

- 3DV-HTM version 5.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1110, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.