| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0158r1 |
| --- | --- |

| Title: | MVC+D HLS: depth sampling characteristics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela and Dmytro Rusanovskyy Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to enable indications of depth sampling grid location as well as depth sample size relative to those of the texture as video usability information (VUI). It is also proposed to enable indicating, as supplemental enhancement information (SEI) message, a suggested depth output time relative to the output time of the texture view components of the same access unit. It is suggested that these pieces of information can be assist a decoder side post-processing unit to perform depth-image-based rendering (DIBR) properly.

Revision 1 attempts to reflect the JCT-3V decision made 19th January, 2013. The VUI information proposed in the previous release is converted into an SEI message in this revision.

# Introduction

Depth information may be generated using a specific depth sensor rather than generated in a per-pixel depth estimation process based on texture views. In such a camera setup, the depth sensor(s) can be either collocated with some of utilized image sensor(s) or located at a different position than the color camera(s), hence representing a different view. A visualization of this concept is shown below:

Figure 1. Stereo camera with a single ranging device.

It is asserted that such camera setups for paired or unpaired MVD (i.e. respectively having the same or a different number of depth and color cameras) might cause the following characteristics:

- As part of pre-processing operations in the encoder side, the texture and depth view components can be rectified and aligned to the same viewpoint. However, due to different camera parameters (e.g. focal length and field of view), the sample arrays of texture view components and depth view components for the same viewpoint might not be perfectly overlapping. In other words, the top-left corner of the depth sampling grid might not correspond to the top-left corner of the texture sampling grid of the same view, and the sample extents (i.e. physical size) in the depth sampling grid might not have any integer ratio relative to sample extents in the texture sampling grid.

- The capture times of texture view components and depth view components might not be perfectly aligned.

The depth sampling grid position relative to the texture sampling grid and the depth sample size relative to the texture sample size are essential pieces of information to perform depth-image-based rendering (DIBR).

The capture times of the depth view components may be used in the decoder side for example for temporal interpolation of depth view components to match the capture times of texture view components. The temporally interpolated depth view components can then be used for DIBR.

It is therefore proposed to enable indications of depth sampling grid location as well as depth sample size relative to those of the texture. It is also proposed to enable indicating a suggested depth output time relative to the output time of the texture view components of the same access unit. The proposals are described in the following sub-sections.

# Depth sampling grid location and depth sample extents

## Motivation for the design decisions

As the depth_vui_parameters_extension( ) only contains information related to the depth sampling grid, all the syntax elements of the structure are unconditionally present. If it turns out that some other depth VUI information would also be needed, the presence of depth sampling grid information within the depth_vui_parameters_extension( ) syntax structure could be made conditional similarly to many other pieces of information included in VUI structures. E.g. depth_sampling_grid_info_present_flag could be added to the syntax.

The horizontal and vertical sample size of depth view components relative to the horizontal and vertical sample size of luma texture view components, respectively, is indicated as a horizontal and vertical ratio (“depth to texture sample ratio”), respectively. These ratios are referred to as dttsrX and dttsrY, respectively. In other words, the width of one depth sample covers dttsrX luma texture sample widths and the heights of one depth sample covers dttsrY luma texture sample heights. dttsrX and dttsrY are indicated with fixed-point values with signalled positions of the decimal point. Note that dttsrX and dttsrY relate to sample aspect ratio, but for simplicity it was decided not to involve texture and depth sample aspect ratios to the coding of the relative depth sample sizes.

As the rectification and alignment of depth views relative to respective texture views might differ for each viewpoint, we allow signaling the depth sampling grid offset separately for each depth view. The depth sampling grid is not required to reside completely within the texture sampling grid of the same view. Depth samples outside the texture sampling grid of the same view may be helpful for example in extending the texture field of view by synthesizing sample values. The depth sampling grid location offset relative to the texture sampling grid, both horizontally and vertically, is indicated with fixed-point values with signalled positions of the decimal point.

## Proposed specification text

### The changes to syntax are indicated in red font color:

    - Depth sampling information SEI message

| depth_sampling_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| dttsr_x_mul | 5 | u(16) |
| dttsr_x_dp | 5 | u(4) |
| dttsr_y_mul | 5 | u(16) |
| dttsr_y_dp | 5 | u(4) |
| per_view_depth_grid_pos_flag | 5 | u(1) |
| if( per_view_depth_grid_pos_flag ) { |  |  |
| num_video_plus_depth_views_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_video_plus_depth_views_minus1; i++ ) { |  |  |
| depth_grid_view_id[ i ] | 5 | ue(v) |
| depth_grid_position( ) |  |  |
| } |  |  |
| else |  |  |
| depth_grid_position( ) |  |  |
| } |  |  |

The depth sampling information SEI message specifies the depth sample size relative to luma texture sample size. In addition, the depth sampling information SEI message specifies the depth sampling grid position of one or more depth view components of the associated access unit relative to the sampling grid of the texture view components of the same access unit with the same view_id value. When present, the depth sampling information SEI message shall be associated with an IDR access unit. The semantics of the message are valid for the current coded video sequence.

NOTE – The depth sample size and the depth sampling grid position are indicated for frame or field view components that are present in the associated IDR access unit. In subsequent access units in the coded video sequence the depth view components may have different values of field_pic_flag and bottom_field_flag compared to those of the depth view components of the IDR access unit. Likewise, in subsequent access units in the coded video sequence the texture view components may have different values of field_pic_flag and bottom_field_flag compared to those of the texture view components of the IDR access unit. The depth sample size and depth sampling grid position should be modified according to the values of field_pic_flag and bottom_field_flag of the texture and depth view components of an access unit compared to those of the IDR access unit.

dttsr_x_mul and dttsr_x_dp specify that the width of a depth sample relative to the width of a luma texture sample is approximately dttsr_x_mul ÷ 2dttsr_x_dp. When dttsr_x_mul is not present, it is inferred to be equal to 1. When dttsr_x_dp is not present, it is inferred to be equal to 0. The value of 0 for dttsr_x_mul is reserved.

dttsr_y_mul and dttsr_y_dp specify that the height of a depth sample relative to the height of a luma texture sample is approximately dttsr_y_mul ÷ 2dttsr_y_dp. When dttsr_y_mul is not present, it is inferred to be equal to 1. When dttsr_y_dp is not present, it is inferred to be equal to 0. The value of 0 for dttsr_y_mul is reserved.

per_view_depth_grid_pos_flag equal to 0 specifies that the depth sampling grid position information is the same for all depth views for which there is a texture view with the same view_id present. The single occurrence of the depth_grid_position( ) syntax structure indicates the depth sampling grid position.

per_view_depth_grid_pos_flag equal to 1 specifies that a depth_grid_position( ) syntax structure is present for indicated depth views.

num_video_plus_depth_views_minus1 (when present) + 1 specifies the number of views for which the depth sampling grid position information is present in this SEI message.

depth_grid_view_id[ i ] specifies the i-th view_id value for which the depth sampling grid position information is specified with the depth_grid_position( ) structure following in the syntax structure.

### Depth sampling grid position

| depth_grid_position( ) { | C | Descriptor |
| --- | --- | --- |
| depth_grid_pos_x_fp | 0 | u(20) |
| depth_grid_pos_x_dp | 0 | u(4) |
| depth_grid_pos_x_sign | 0 | u(1) |
| depth_grid_pos_y_fp | 0 | u(20) |
| depth_grid_pos_y_dp | 0 | u(4) |
| depth_grid_pos_y_sign | 0 | u(1) |
| } |  |  |

depth_grid_pos_x_fp, depth_grid_pos_x_dp and depth_grid_pos_x_sign specify that the horizontal position of the top-left sample in the sampling grid for a depth view component relative to the top-left sample in the sampling grid for the luma component of the texture view component of the same view_id is equal to the output value of the function gridPos(depth_grid_pos_x_fp, depth_grid_pos_x_dp, depth_grid_pos_x_sign) as specified below.

When depth_grid_pos_x_fp is not present, it is inferred to be equal to 0. When depth_grid_pos_x_dp is not present, it is inferred to be equal to 0. When depth_grid_pos_x_sign is not present, it is inferred to be equal to 0.

depth_grid_pos_y_fp, depth_grid_pos_y_dp and depth_grid_pos_y_sign specify that the vertical position of the top-left sample in the sampling grid for a depth view component relative to the top-left sample in the sampling grid for the luma component of the texture view component of the same view_id is equal to the output value of the function gridPos(depth_grid_pos_y_fp, depth_grid_pos_y_dp, depth_grid_pos_y_sign) as specified below.

The function gridPos(fp, dp, sign) is specified to return ( 1 – 2 * sign ) * ( fp ÷ 2dp ).

When depth_grid_pos_y_fp is not present, it is inferred to be equal to 0. When depth_grid_pos_y_dp is not present, it is inferred to be equal to 0. When depth_grid_pos_y_sign is not present, it is inferred to be equal to 0.

# Relative depth output time

## Motivation for the design decisions

The current specification of the HRD for MVC+D uses a single output time for all decoded view components of an access unit. We thought that it would be a too drastic change in such a late stage in the MVC+D standardization to change this design choice. Hence, we propose no changes to the HRD, but propose a new SEI message (called depth timing SEI), which can be used in the post-processing phase after output of the view components of the associated access unit.

The depth timing SEI message may be present in any access unit and it pertains until the end of the coded video sequence or until the next depth timing SEI message, whichever is earlier in decoding order. If the acquisition of coded depth views was synchronized, a single value of timing offset may be provided that applies to all coded depth views. Otherwise, a value of timing offset may be provided for each coded depth view separately. The latter case appears to be quite typical in non-monoblock camera arrays.

The timing offset values are provided as fixed-point with a selectable position of the decimal point. They are relative to the tick duration of the texture view components (as signaled in the sequence parameter set for texture view components). Only non-negative offsets are allowed, as negative offsets would have contradicted with the output from the DPB – i.e., if negative offsets were allowed, a post-processor might receive depth view components from the decoder which are already outdated for the latest decoded texture view components that the decoder has output.

## Proposed specification text

### Payload type specification

| else if( payloadType = = 48 ) |  |  |
| --- | --- | --- |
| 3dvc_scalable_nesting( payloadSize ) |  |  |
| else if ( payloadType = = 49 ) |  |  |
| 3dvc_view_scalability_info( payloadSize ) |  |  |
| else if (payloadType = = 50 ) |  |  |
| depth_acquisition_info ( payloadSize ) |  |  |
| else if (payloadType = = 51 ) |  |  |
| depth_representation_information ( payloadSize ) |  |  |
| else if (payloadType = = 52 ) |  |  |
| 3d_reference_displays_info ( payloadSize ) |  |  |
| else if( payloadType = = 53 ) |  |  |
| depth_timing( payloadSize ) |  |  |
| else if( payloadType = = 54 ) |  |  |
| depth_sampling_information( payloadSize ) |  |  |

### Depth timing SEI message

| depth_timing( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| per_view_depth_timing_flag | 5 | u(1) |
| if ( per_view_depth_timing_flag ) |  |  |
| for( i = 0; i < NumDepthViews; i++ ) |  |  |
| depth_timing_offset( ) |  |  |
| else |  |  |
| depth_timing_offset( ) |  |  |
| } |  |  |

The depth timing SEI message indicates the acquisition time of the depth view components of one or more access units relative to the DPB output time of the same access units. The depth timing SEI message may be present in any access unit and it pertains until the end of the coded video sequence or until the next depth timing SEI message, whichever is earlier in decoding order. The access units that the message pertains to are referred to as the target access unit set.

per_view_depth_timing_flag equal to 0 specifies that all the depth view components within the target access unit set have the same acquisition time offset relative to the DPB output time of the respective access unit in the target access unit set. The single occurrence of the depth_timing_offset structure specifies this acquisition time offset.

per_view_depth_timing_flag equal to 1 specifies that a depth_timing_offset syntax structure is present for each depth view in ascending order of view order index values for the depth views and specifies the acquisition time offset for that view.

### Depth timing offset syntax structure

| depth_timing_offset( ) { | C | Descriptor |
| --- | --- | --- |
| offset_len_minus1 | 5 | u(5) |
| depth_disp_delay_offset_fp | 5 | u(v) |
| depth_disp_delay_offset_dp | 5 | u(6) |
| } |  |  |

offset_len_minus1 specifies the length of the depth_disp_delay_offset_fp syntax element.

depth_disp_delay_offset_fp and depth_disp_delay_offset_dp specify that the acquisition offset of the respective depth view component or components relative to the DPB output time of the access unit containing the depth view component or components is equal to depth_disp_delay_offset_fp ÷ 2depth_disp_delay_offset_dp) in units of clock ticks as specified in Annex C.

The length of depth_disp_delay_offset_fp syntax element is equal to offset_len_minus1 + 1.

If depth_disp_delay_offset_fp is not present, it is inferred to be equal to 0. If depth_disp_delay_offset_dp is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).