| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0160 |
| --- | --- |

| Title: | 3D-CE6.h related: Results on modified depth coding in random access units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heribert Brust Karsten Mueller Thomas Wiegand | Email: | heribert.brust@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to change the coding of slices in random access units in non-base depth views. The changes are applied to HTM5.0.1. With the first change Intra modes are always tested at the encoder for depth views in random access units similar to the encoding of I-slices. With the second change quadtree limitation and predictive coding, which is described in JCT3V-B0068, is switched off in all random access units in addition to I-slices. It is reported that both changes combined have a BD-rate of -0.8% for synthesized views and -0.4% for coded and synthesized views with an encoding time of 105.5% and decoding time of 102.2%.

# Introduction

In HTM5.0.1 Intra modes are tested in the encoding process in I-slices for all coding units. Additionally Intra modes are tested for all possible coding unit depths in I-slices because quadtree limitation and predictive coding (JCT3V-B0068[1]) is switched off for I-slices.

In random access units only Intra prediction and inter-view prediction is allowed, but no temporal prediction. Slices in non-base views of random access units are coded as P-slices in the common test conditions (see Figure 1).

Figure 1: Coding structure of HTM common test conditions

The depth maps of different views are not always consistent. This means that inter-view references are not always good predictors for dependent depth views. In HTM5.0.1 non-base view depth slices in random access units have very limited prediction possibilities because temporal prediction is not allowed and Intra modes are only tested for some coding units (CUs) depending on the coding results of previous CUs. The quadtree limitation further reduces the prediction possibilities.

# Proposed changes

- The first change is that Intra modes are always tested for depth slices in random access units at the encoder.

- The second change is that quadtree limitation and predictive coding is switched off for all depth slices in random access units.

The changes have been implemented into HTM5.0.1. They only affect slices in non-base depth views. In HTM5.0.1 with common test conditions non-base views in random access units have NAL unit type: NAL_UNIT_CODED_SLICE_IDV. The NAL unit type is used to identify the correct slices. In HTM5.1 and common test conditions the NAL unit type of all random access unit slices is NAL_UNIT_CODED_SLICE_IDR.

In the 3D-HEVC Test Model Description JCT3V-B1005_d0 the normative changes can be applied with the check RapPicFlag is equal to 0 or 1, which signals the random access unit. The normative changes affect only predictive coding.

Based on the HEVC text specification draft 8 JCTVC-J1003_d7 RapPicFlag is 1 if nal_unit_type is between 7 and 12 inclusive.

Based on the HEVC text specification draft 9 JCTVC-K1003_v13 RapPicFlag is 1 if nal_unit_type is between 16 and 23 inclusive.

Depending on the software version these NAL unit type ranges can be used to identify the random access slices.

# Experimental Results

Table 1 shows the results for the first change: Intra modes are always tested at the encoder for all CUs in random access unit depth-slices. A coding gain of -0.4% BD-rate for synthesized views and -0.2% BD-rate for coded and synthesized views can be observed with an encoding time of 101.0% and decoding time of 100.6%.

Table 1: Results for experiment 1: Full encoder Intra search for depth-RAU-slices

Table 2 shows the results for the second change: Quadtree limitation and predictive coding is switched of for random access unit (RAU) slices.

Table 2: Results for experiment 2: Quadtree limitation and predictive coding switched off for RAU-slices

Table 3 shows the combination of the first and the second change: Intra modes are always tested at the encoder for all CUs in random access unit depth-slices and quadtree limitation and predictive coding is switched off for random access unit slices. A coding gain of -0.8% BD-rate for synthesized views and -0.4% BD-rate for coded and synthesized views can be observed with an encoding time of 105.5% and decoding time of 102.2%.

Table 3: Results for experiment 3: Full Intra encoder search for RAU depth-slices and quadtree limitation and predictive coding switched off for RAU-slices

Table 4 shows for comparison the coding results for quadtree limitation and predictive coding switched off completely. An encoding time of 143.8% can be seen, which is significantly higher than the 105.5% of the proposed changes.

Table 4: Results for experiment 4: Quadtree limitation and predictive coding switched off for all slices

# Conclusion

This contribution proposes to change the coding of non-base view depth slices in random access units.

With the first change Intra modes are always tested in depth views of random access units. The second change is that quadtree limitation and predictive coding is switched off for all slices in random access units. Both changes combined have a gain of -0.8% BD-rate for synthesized views and -0.4% BD-rate for coded and synthesized views with an encoding time of 105.5% and decoding time of 102.2%.

We propose to adopt both changes to the HTM software and to the working draft.

# Patent rights declaration

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Jung, E. Mora (Orange Labs), " 3D-CE3.h: Depth Quadtree Prediction for 3DHTM 4.1", JCT3V-B0068, October 2012, Shanghai, CN.