| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0163 |
| --- | --- |

| Title: | 3D-CE1.h related: Results on the VSP-mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heribert Brust Gerhard Tech Karsten Mueller Thomas Wiegand | Email: | heribert.brust@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a view synthesis prediction (VSP) mode for dependent texture views which is signaled by a flag without coding additional information for the VSP mode. It is reported that the coding gain for non-base texture views is -25.9% BD-rate and -25.3% BD-rate, for Video PSNR versus total bitrate it is -0.9% BD-rate, for synthesized PSNR versus total bitrate it is -4.5% BD-rate and for coded and synthesized PSNR versus total bitrate it is -3.4% BD-rate. The reported encoding time is 98.7% and the decoding time is 201.5%.

# Introduction

View Synthesis prediction (VSP) has been proposed and studied in AVC compatible 3D video extension (ATM) and HEVC compatible 3D video extension (HTM). The method, proposed in this document is similar to the algorithms proposed in [1][2] and [3]. It focuses especially on reducing the additional bit rate required by the non-base texture views.

The proposed method gives the possibility to render views with high quality and reduce the additional bitrate of the MVD components compared to the texture base view. The bitrate of dependent views is reduced by predicting most parts from the extrapolated base view without coding additional information for these parts.

# Proposed Algorithm

The proposed VSP mode was implemented into HTM5.0.1 and is only used for dependent texture views. In addition to MODE_SKIP, MODE_INTER and MODE_INTRA, MODE_VSP was added. After decoding the texture base view and the depth base view, a picture is extrapolated from the decoded texture and depth to the dependent view position (Figure 1). The renderer in HTM5.0.1 was used with line wise background extension.

Figure 1: Extrapolated view at position 5 using decoded texture and depth of position 4 (base view); sequence poznanstreet

For each coding unit (CU) it is signaled in the bit stream whether the CU is predicted using the synthesized picture or intra/inter prediction.

An additional flag is inserted before the skip flag to signal the VSP mode. If the VSP flag is true no additional information is sent for this CU.

The encoder decision whether to use the VSP mode or Intra/Inter prediction is done with the conventional cost calculation.

Figure 2 shows as illustration decoded CUs which use conventional Intra/Inter prediction.

Figure 2: Decoded CUs using Intra and Inter prediction; sequence poznanstreet

Figure 3 shows the decoded dependent texture view using view synthesis predicted CUs and Intra/Inter CUs.

Figure 3: Decoded texture view 5 using view synthesis predicted CUs and intra/inter predicted CUs; sequence poznanstreet

# Configuration

All tests have been carried out with the VSP mode implemented into HTM5.0.1 as described above.

The configuration differences to the common test conditions are the following:

For all tests the parameter “LambdaScaleVSO” was set to “2”.

The sample adaptive offset (SAO) filter for non-base texture views was switched on or off according to table 1.

In HTM5.0.1 and common test conditions the QP-difference between base texture view and non-base texture view is 3. This QP-difference was also varied according to table 1.

| Experiment | SAO filter for non-base texture views | QP-difference between base texture view and non-base texture views |
| --- | --- | --- |
| 1 | on | 5 |
| 2 | off | 5 |
| 3 | off | 6 |
| 4 | off | 7 |

Table 1: Configuration differences for VSP-experiments

# Experimental results

In the description of Core Experiment 1 on View Synthesis Prediction JCT-3V-B1101 it is noted that all PSNR values should be reported against the total bitrate (including the depth rate). The video PSNR versus video bitrate (without depth) is meaningless for VSP as the video cannot be decoded without the depth bit stream. Because of that old and new excel files are attached and the appropriate columns are reported here.

Table 2: Results for experiment 1

Table 3: Results for experiment 2

Table 4: Results for experiment 3

Table 5: Results for experiment 4

# Conclusion

The results for experiment 4 show a significant coding gain for HTM5.0.1 with the proposed VSP mode. Bitrate savings of -25.9% BD-rate and -25.3% BD-rate for non-base texture views can be seen. The video PSNR versus total bitrate has a gain of -0.9% BD-rate, synthesized PSNR versus total bitrate has a gain of -4.5% BD-rate and coded and synthesized PSNR versus total bitrate has a gain of -3.4% BD-rate. The decoding time however is significantly increased to 201.5%. To achieve the reported bitrate savings with the VSP mode it was necessary to change the coding configuration. The SAO filter was switched off for non-base texture views, the QP-difference between base texture view and non-base texture views was changed from 3 to 7 and the Lambda Scale for depth views was set to 2.

We propose to study the described algorithm further in the CE.

# Patent rights declaration

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- Dong Tian, Danillo Graziosi, Anthony Vetro, "3D-AVC-CE1 results on VSP skip /direct mode by Mitsubishi", ISO/IEC JTC1/SC29/WG11 MPEG2011/M23915, February 2012, San Jose, USA.

- Yang Yu, Ying Chen, Li Zhang, Marta Karczewicz, "3D-CE1.a related: Generalized view synthesis prediction (GVSP) mode", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24940, April 2012, Geneva, Switzerland.

- Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata, (NTT), Dong Tian, Feng Zou, Anthony Vetro, (MERL), "3D-CE1.h Results on View Synthesis Prediction ", JCT3V-A0018, July 2012, Stockholm, Sweden.