| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0164 m27926 |
| --- | --- |

| Title: | Recommendation on MVC+D reference software |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olgierd Stankiewicz, Krzysztof Wegner Dmytro Rusanovskyy | Tel: Email: Email: | +48 61 665 3893 ostank@multimedia.edu.pl kwegner@multimedia.edu.pl Dmytro.Rusanovskyy@nokia.com |
| Source: | Poznan University of Technology, Poland Nokia Corporation |  |  |

_____________________________

# Abstract

In this document we discuss advantages and disadvantages of available candidates for MVC+D reference software. It is observed that neither of available candidates (JMVC or 3DV-ATM) are currently fully complies with general requirements on MPEG standard reference software and both candidates need additional development efforts to comply with it. However, considering design architecture and stability of software packages, it is recommended to utilize 3DV-ATM software as a reference test model for MVC+D specification. The list of recommended changes is provided in m27809 and m27811 MPEG documents.

# Introduction

The need for reference software reconsideration has been raised on BoG on reference software for MVC+D, at Shanghai meeting (102th MPEG, 2nd JCT3V meeting), in the context of conformance testing.

According to recommendation of BoG, Poznan University (Poland) performed an evaluation of cross-platform compatibility of the JMVC software in configuration with interlace depth coding. Evaluation showed, that JMVC software has severe problems with cross-platform compatibility and cannot be executed properly in Windows platforms. Complete report on this evaluation to be presented in MPEG/JCT-3V document [3], however it is reported that JMVC+D software does not support cross-platform compatibility. For example, JMVC+D software crashes when coding in interleaved modes in Windows and Linux platform.

According to recommendation of BoG, Industrial Technology Research Institute (Republic of Korea), performed an evaluation of 3DV-ATM with respect to its compliance with MVC+D specification, and started integration of interlace coding support to 3DV-ATM.

As a result of this work ITRI reported in m27811 MPEG/JCT-3V document some mismatched in High Level Syntax in 3DV-ATM bitstreams comparing to MVC+D specification. Detected mismatch is localized in Sequence Parameter Set and bug-fix to be provided during next integration round.

Results of ITRI’s work on interlace depth coding support in 3DV-ATM are reported in details in m27809 MPEG/JCT-3V document. The work on the encoder compliance is in its final stage and decoder compliance to be started in the next integration round. It is believed, that this work can be finalized in next meeting round, if sufficient support from software coordinators to be provided.

Moreover, it is observed and worth of noting that JMVC software utilizes a sequence level multipass approach for coding of multiview video data (applies for MVD as well) and has significantly restricted encoder functionality. Consider these architecture elements, utilization of JMVC as a reference for an industrial standard is not recommended.

# Recommendation

As a result of on-site work with available candidates for MVC+D reference software, it is observed that neither of available candidates (JMVC or 3DV-ATM) are currently fully complies with general requirements on MPEG standard reference software and both candidates need additional development efforts to comply with it.

However, considering design architecture and stability of software packages, it is recommended to utilize 3DV-ATM software as a reference test model for MVC+D specification. The list of recommended changes is provided in m27809 and m27811 MPEG documents.

# Patent rights declaration(s)

Poznan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Teruhiko Suzuki, Miska M. Hannuksela, Ying Chen, Gary Sullivan, Shinobu Hattori, ”Study Text of ISO/IEC 14996-10:2012/DAM2 MVC extension for inclusion of depth maps“, ISO/IEC JTC1/SC29/WG11 MPEG2012/N13140, Shanghai, China, October 2012.

- Olgierd Stankiewicz, Krzysztof Wegner, " Report on results of cross-check on coding with JMVC+Depth software in interlace mode", Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JCT3V-C0036, m27759, 3rd Meeting, Geneva, January 2013.

- Ching-Chieh Lin, Fang-Chu Chen “AHG4: MVC+D syntax mismatch with ATM 6.0 (frame coding)”, 3rd JCT-3V meeting, JCT3V-C0071, Geneva. Switzerland, Jan. 2013

- Ching-Chieh Lin, Fang-Chu Chen “AHG4: Interlace coding on ATM Encoder”, 3rd JCT-3V meeting, JCT3V-C0069, Geneva. Switzerland, Jan. 2013