| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0168 |
| --- | --- |

| Title: | AHG4: Reduced complexity depth coding in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

3D-AVC [1] specifies several coding that imply low level processing for efficient depth map coding. In this contribution we studied a compression efficiency of such tools. Simulation results produced with the most recent 3DV-ATM v6.1 and simplified depth map coding (all low level depth coding tools were disabled) show the penalty of 1.5% of dDBR on average for coded views and 1.7% of dBR for synthesised views with 5% of decoding complexity reduction on average. It proposed to consider removal of low level depth coding tools for purposes of coding architecture simplification.

# Proposal

The most recent 3DV-ATM v6.1 software executed under the CTC and following parameters:

InLoopJVF                =   0	         #Enable In Loop Joint View Depth Filtering (Default:0)

IVMP                     =   0                       # Enable motion vector prediction from texture to depth (Default:0)

DISP                     =   0                       # Enable depth intra skip prediction (Default:0)

PSIP                     =   0                       # Enable depth plane segmentation intra prediction (Default:0)

Introduced changes disable inter-component dependency (texture to depth) and low level changes to Motion Vector Prediction processes for depth map coding (IVMP). In addition, these changes disable two intra coding modes utilized for depth map coding (DISP and PSIP).  With given changes, 3D-AVC would fully benefit from texture coding and from high level tools of depth map coding (Slice Header Prediction).

Simulation results produced with this configuration are shown in Table 1.  Introduced changes resulted in 1.5% of penalty for coded views on average, and in 1.7% of dBR for synthesized views. Complexity estimates for the tested scheme compared against the the anchor are shown in Table 2.

This contribution proposes to consider removal of the following tools from the 3D-AVC specification [1]:

Inside View Motion Prediction:	J.8.3.3	Derivation process for inside view motion prediction

Depth intra prediction:		J.8.7	Depth intra prediction process

Joint view in-loop depth filtering:	J.8.6	In-loop depth filtering process

Table 1. Simulation results of 3DV-ATM v6.1 in tested configuration against the v6.1 anchor results.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesis PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -0.15 | 0.01 | 12.17 | -0.69 | 2.44 | -0.08 | 2.82 | -0.09 |
| S02 | -0.04 | 0.00 | -9.41 | 0.44 | 0.80 | -0.02 | 0.84 | -0.03 |
| S03 | -0.29 | 0.01 | 1.37 | -0.11 | 0.41 | -0.01 | 0.98 | -0.03 |
| S04 | -0.28 | 0.01 | 3.88 | -0.22 | 1.33 | -0.05 | 1.79 | -0.06 |
| S05 | -0.05 | 0.00 | 8.90 | -0.41 | 3.16 | -0.14 | 3.12 | -0.13 |
| S06 | 0.02 | 0.00 | 1.56 | -0.07 | 1.35 | -0.06 | 1.41 | -0.06 |
| S08 | 0.03 | 0.00 | -2.29 | 0.10 | 1.23 | -0.05 | 1.56 | -0.05 |
| Average | -0.11 | 0.00 | 2.31 | -0.14 | 1.53 | -0.06 | 1.79 | -0.06 |

Table 2. Simulation results of 3DV-ATM v6.1 in tested configuration against the v6.1 anchor results.

|  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- |
|  | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 1.00 | 0.98 | N/A |
| S02 | 1.00 | 1.01 | N/A |
| S03 | 0.99 | 1.02 | N/A |
| S04 | 0.99 | 0.97 | N/A |
| S05 | 1.00 | 0.89 | N/A |
| S06 | 1.00 | 0.89 | N/A |
| S08 | 0.98 | 0.91 | N/A |
| Average | 1.00 | 0.95 | #DIV/0! |

# References

- M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, G. J. Sullivan, " 3D-AVC draft text 4," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1002, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).