| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C1069 |
| --- | --- |

| Title: | CE1.a-related: Simplification of BVSP in 3DV-ATM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution presents simulation results produced with 3DV-ATM v6.1 and configurable block sizes of BVSP. The BVSP implemented with 4x4 blocks produced identical average results as the 2x2 implementation (-0.01% of dBR), whereas BVSP with 8x8 block sizes reduced coding performance by 0.15% of dBR on average. In the tested configurations, a single disparity vector was derived for a target block size through the normative 3D-AVC disparity derivation method (i.e., the maximum out of 4 corner samples).

As a result of this study, it is proposed to adopt BVSP implementation with the 8x8 block size in the 3D-AVC specification [1] and in the common test conditions.

# Proposal

3D-AVC currently specifies VSP SKIP and VSP DIRECT modes which are implemented through disparity compensated blocks size of 2x2. In this mode a single disparity value is derived for every texture block 2x2 from associated depth map data and converted to a disparity vector.

The 3DV-ATM v6.1 software allows VSP configuration of different block sizes. In this document VSP_SKIP and VSP_DIRECT implementation with larger block sizes are evaluated. In conducted experiments, a single disparity vector was derived for texture blocks of given size with normative in 3D-AVC derivation method (max out of 4 corner candidates).

It is proposed to adopt BVSP implementation with 8x8 block size in order to align with existing in H.264/AVC SKIP mode functionality.

# Simulation results

Simulation results produced with BVSP with block sizes 4x4 is shown in Table 1. Usage of a single disparity vector for texture block size of 4x4 produce identical results to the 2x2 BVSP anchor scheme.

Simulation results produced with BVSP with block sizes 8x8 is shown in Table 2. Usage of a single disparity vector for texture block size of 8x8 introduce a penalty of 0.15% of dBR whereas introduce significant complexity reduction.

Table 1. Simulation results produced with BVSP implemented with 8x8 blocks

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -0.07 | 0.00 | 0.00 | 0.00 | -0.07 | 0.00 | -0.07 | 0.00 |
| S02 | -0.06 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.06 | 0.00 |
| S03 | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | -0.01 | 0.00 |
| S04 | -0.15 | 0.01 | 0.00 | 0.00 | -0.13 | 0.00 | -0.15 | 0.01 |
| S05 | 0.06 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.06 | 0.00 |
| S06 | 0.12 | -0.01 | 0.00 | 0.00 | 0.11 | -0.01 | 0.12 | -0.01 |
| S08 | 0.05 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.05 | 0.00 |
| Average | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | -0.01 | 0.00 |

Table 2. Simulation results produced with BVSP implemented with 8x8 blocks

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.08 | 0.00 |
| S02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.02 | 0.00 |
| S03 | 0.48 | -0.02 | 0.00 | 0.00 | 0.46 | -0.02 | 0.41 | -0.01 |
| S04 | -0.27 | 0.01 | 0.00 | 0.00 | -0.25 | 0.01 | -0.27 | 0.01 |
| S05 | 0.22 | -0.01 | 0.00 | 0.00 | 0.15 | -0.01 | 0.29 | -0.01 |
| S06 | 0.37 | -0.02 | 0.00 | 0.00 | 0.32 | -0.02 | 0.42 | -0.02 |
| S08 | 0.17 | -0.01 | 0.00 | 0.00 | 0.15 | -0.01 | 0.23 | -0.01 |
| Average | 0.14 | -0.01 | 0.00 | 0.00 | 0.12 | -0.01 | 0.17 | -0.01 |

3

# Proposed Working Draft Text

The section below provides recommended WD text for clause J.8.3.2.1. Introduced text is marked in yellow colour (text to be reomoved) and with red colour (new text).

J.8.3.2.1	Decoding process for view synthesis prediction samples

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as specified by the following ordered steps:

- The variables dX, dY, depthSample, and disparityValue are derived with use of temporal variables depthSample1,…, depthSample4		 :

dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
depthSample = currDepthPic[ dY ][ dX ]

blockVspT = 8

blockVspD = blockVspT >> reduced_resolution_flag

dX = (x-x% blockVspT) >> reduced_resolution_flag

dY = (y-y% blockVspT) >> reduced_resolution_flag

depthSample1 = currDepthPic[ dY][ dX ]

depthSample2 = currDepthPic[ dY][ dX + blockVspD - 1]
depthSample3 = currDepthPic[ dY + blockVspD - 1][ dX]
depthSample4 = currDepthPic[ dY + blockVspD - 1][ dX + blockVspD - 1]
depthSample = max(max(max(depthSample1, depthSample2), depthSample3), depthSample4)

disparityValue = Disparity( depthSample, currIndex, refIndex )

For each chroma sample location (0 <= xC < partWidthC, 0 <= yC < partHeightC) inside the prediction chroma sample arrays predPartLXCb and predPartLXCr, the corresponding prediction chroma sample values predPartLXCb[ xC, yC ] and predPartLXCr[ xC, yC ] are derived as specified by the following ordered steps:

- The variables dX, dY, depthSample, and disparityValue are derived by:

dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
depthSample = currDepthPic[ dY ][ dX ]

blockVspT = 8

blockVspD = blockVspT >> reduced_resolution_flag

dX = (x-x% blockVspT) >> reduced_resolution_flag

dY = (y-y% blockVspT) >> reduced_resolution_flag

depthSample1 = currDepthPic[ dY][ dX ]

depthSample2 = currDepthPic[ dY][ dX + blockVspD - 1]
depthSample3 = currDepthPic[ dY + blockVspD - 1][ dX]
depthSample4 = currDepthPic[ dY + blockVspD - 1][ dX + blockVspD - 1]
depthSample = max(max(max(depthSample1, depthSample2), depthSample3), depthSample4)

disparityValue = Disparity( depthSample, currIndex, refIndex )