| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0170 |
| --- | --- |

| Title: | CE2.h-related: Flexible Coding Order (FCO) and depth-first-based disparity derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Srikanth Gopalakrishna Dmytro Rusanovskyy Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

Flexible Coding Order (FCO), which is integrated in 3D-HTM, allows the encoder side to choose whether a texture view component follows or precedes the depth view component of the same non-base view. FCO also enables an additional approach for disparity derivation. When the encoder side chooses to code a depth view component before the associated texture view component, FCO enables derivation of disparity information directly from the depth view component, which reportedly has low computational complexity and causes no additional memory usage. The contribution presents simulation results on FCO and simplified disparity derivation in the “depth first” coding mode of the 3D-HTM. The simulation results show that proposed solution outperformed the anchor scheme of the common test conditions by 0.8% of dBR for the coded texture views. However, change in the VSO settings, which in turn introduced a penalty on depth coding results in 1.2% of dBR increase for synthesised views.

At the same time, the proposed disparity derivation solution reduced the decoding time by 10% and may be found useful in certain 3DV applications. The contribution proposes to adopt the FCO and presented disparity derivation method in the 3D-HEVC specification.

# Introduction

3D-HEVC utilizes disparity information for improving the coding efficiency of non-base views of MVD and multiview video data. The draft 3D-HEVC specifies that disparity information can be either derived from the depth of the base view, which requires projections, or derived through a search over spatio-temporal neighbors of the current CU. This contribution analyzes the current and some proposed disparity derivation methods in terms of compatibility with HEVC version 1 and MV-HEVC, amount of changes relative to HEVCv1/MV-HEVC, computational complexity, required data access (e.g. which and how many depth and/or texture view components and blocks have to be accessed), and additional memory use (e.g. to store derived disparity vectors). One presented conclusion of the analysis is that in terms of the decoding compatibility with HEVCv1 and MV-HEVC all the disparity derivation methods are identical.

The 3D-HEVC [] specifies utilization of disparity information for coding of dependent views of multiview video and MVD data. For example, the following coding tools can use disparity information:

- Adaptive Motion Vector Prediction (AMVP).

- Interview Residual Prediction.

- View Synthesis Prediction.

All of these tools takes derived disparity information as input and utilizes it for coding of an dependent texture view. It should be noted, that neither disparity derivation nor utilization of disparity for coding of video are included in HEVCv1/MV-HEVC. Thus, the given tools and the disparity derivation process shall be evaluated as 3D-HEVC-specific technologies and analyzed accordingly.

3D-HTM codec provides four possibilities to derive disparity information. However these methods may be categorized in two categories, referred here as disparity derivation types. Method 1 derives the disparity vector from a depth map (Derivation Type 1). Methods 2, 3 and 4 derive a disparity estimate from motion information extracted from coded texture information. These methods can be classified as Derivation Type 2. There are two additional methods, that currently not in the 3D-HEVC specification text, but they have been presented earlier either for 3D-HEVC or for 3D-AVC. Method 5 proposed in [JCT3V-B0090] is a hybrid MV-depth based derivation and method 6 [FCO] is a depth based derivation, analysis of which in the context of 3D-HEVC is presented in this paper. In this paper, these methods are categorized as Type 3 and Type 4 respectively.

The section below assesses the details of each method.

Method 1 specifies the projection of depth map from the base view to view point of dependent view and use this projected depth map for disparity derivation. The projection of depth map is followed by a hole filling algorithm, and holes is filled with a largest depth value among existing neighboring depth samples. Following hole filing, depth map is converted to disparity through the pre-computed look up tables.

A frame-level implementation of Method 1 would feature the least per-pixel processing complexity but would requires an additional frame buffer with random access and multi-pass processing for every row of the virtual depth map. Operations of projection and depth to disparity conversion are straightforward and executed through the pre-computed look up tables.

Local (on-fly) implementation of Method 1 would have minimal memory requirements amount alternatives, but would feature higher worse-case complexity and preserve random memory access and multi-pass processing. In such implementation, depth map samples of the base view located in a window specified by minimal and maximal disparity values are projected to fill a variable buffer-line.

The processing window for depth map projection as well as size of the buffer are derived from minimal/maximal disparity values that shall be pre-computed for a current sequence/frame/or line of the depth data in the base view, or signaled. The process of selection minimal/maximal disparity values shall be normative in order to avoid mismatch between encoder and decoder.

As short summary on this method is given below:

Derivation Type:		1 (Projection)

Content/applicability:		MVD (un-paired MVD)

Dependency:			Depth (within access unit)

Codec compatibility: 		2D HEVC

Amount of changes to HEVC:	Moderate, high/frame level

Computational complexity:	7*deltaDisparity/block_size (block based implementation, single value per block). Approximately 350 ops/block.

Additional memory:		Local buffer, size of deltaDisparity: approximately ~50 samples.

Method 2 is different from Method 1 due to the origin of disparity information and this method targets texture-only multiview coding. The source of disparity information is motion information of the previously coded picture in dependent view. The motion information is stored, analyzed and every inter-view motion vector is treated as a disparity for a certain portion. Disparity information for intra-coded partitions are derived from a local neighborhood through a process similar to intra prediction. Disparity information for following pictures of dependent views are derived through combination of motion information of inter-view prediction and disparity samples fetched through motion vectors of inter-prediction. Computational complexity and memory requirements of this method are very significant, and its applicability for depth-based coding of texture component is very limited.

Method 3 specifies derivation of the disparity information from spatial neighborhood of current Cb. For every Cb, totally 5 spatially neighboring blocks and 4 temporally neighboring blocks are parsed in priority order and their motion information is classified with respect to prediction type. If a neighbor is coded with inter-view prediction, its motion vector is interpreted as disparity and inherited for coding of current Cb.

Parsing of temporal neighbors includes selection a collocated reference picture in TMVP and a picture derived from a reference picture through prioritization process:

- Reference  picture list is searched for a RAP picture,

- If no RAP picture is found, the picture with lowest temporal ID

- If more than 1 picture with equal temporal ID exist, than select candidate with minimal POC.

Within each temporal neighboring picture, a collocated block provides 2 candidate blocks for which motion information is parsed.

Summarizing the shortly presented above, Method 3 specifies for derivation a single disparity value to access in memory 9 neighboring blocks and analyse 3-4 variable parameter for each candidate and perform copying of motion vectors. Besides the relatively demanding processing, Method 3 requires that noticeable amount of motion information and coding parameters to be stored and accessible for quite a certain amount of time, e.g. RAP ~0.5-1 second.

Method 4

Similarly to Method 3, this method assumes that disparity information is derived from motion information of spatio-temporal neighborhood. However, this method reduces number of parameters required for validation during the disparity derivation.

The algorithm specifies the following procedure:

- Parse spatio-temporal neighbors (5 spatial and 4 temporal), evaluate if DCP is used.

  - If DCP found, use its MV as disparity

- Parse spatio-temporal neighbors, evaluate if DV-MCP is used.

  - If block is coded in SKIP mode, use its pre-stored inter-view MV as disparity.

To reduce the amount of memory required for the derivation of the disparity from DV-MCP blocks, blocks B0, B1 and B2 are only utilized when they are located in the current LCU (A1 and A0 are skipped). However, this leads to additional variable to be processed.

A short summary for least complex solution among methods 2-4 is given below:

Derivation Type:		2 (Spatio-Temporal neighborhood analysis)

Content/applicability:		Multiview video

Dependency:			Spatio-temporal, across access units

Codec compatibility: 		2D HEVC

Amount of changes to HEVC:	High, low level changes

Computational complexity:	Worst case: >36 ops / block

Additional memory:	Long term memory: 2-4 variables per each PU, x number of PU in frame, x number of reference frames.

Method 5

This method is applicable for more accurate disparity derivation from neighboring blocks (methods 2-5). Disparity vector derived with NBDV can be further improved by more accurate disparity derivation from the depth map of the base view. In this approach, initial disparity vector is utilized to locate reference depth map data in the base view and the block of depth data is used to produce refined disparity value. The Method 5 requires presence of the depth information of the base view, thus it will fit to MVD-coding scenarios only.

Derivation Type:		3 (Spatio-Temporal neighborhood analysis, depth)

Content/applicability:		MVD

Dependency:			Spatio-temporal, across access units, depth

Codec compatibility: 		2D HEVC

Amount of changes to HEVC:	High, low level changes

Computational complexity:	Worst case: >40 ops / block

Additional memory:	Long term memory: 2-4 variables per each PU, x number of PU in frame, x number of reference frames.

Method 6

The Method 6, which is proposed in this document, performs disparity derivation from depth map data associated with currently coded texture view. The disparity derivation is straightforward: accessing depth samples which being coded earlier and converting them to disparity through a look up table search. This method has virtually no computation complexity and no additional memory requirements, since all required data is available within the same access unit. However, similarly to methods 1 and 5, this derivation algorithm is applicable in the case of MVD data scenarios in addition, it requires depth component being coded prior to the texture component. This coding mode is enabled in 3D-HTM with FCO=1 and leads to reconfiguration of some coding tools.

Derivation Type:		4 (Depth, FCO)

Content/applicability:		MVD

Dependency:			Depth

Codec compatibility: 		2D HEVC

Amount of changes to HEVC:	Minimal, pixel access

Computational complexity:	~2 ops / block

Additional memory:	No additional memory.

The summary of said above is given in Tables below. Table 1 shows major features of listed methods, dependency and applicability. Table 2 provides computational and implementation complexity estimates as well as memory requirements.

Table 1. Available alternative for disparity derivation, major features

| Disparity Derivation | Feature | Content | Dependency | Compatibility | Changes to HEVC |
| --- | --- | --- | --- | --- | --- |
| Type 1 | Projection | MVD | Depth | HEVC | Medium, high level (frame) |
| Type 2 | MV analysis | Multiview video | Spatio-temporal-interview | HEVC | Very large, low level |
| Type 3 | MV analysis + projection | MVD | Spatio-temporal-interview, depth | HEVC | Very large, low level |
| Type 4 | Depth comes first | MVD | Depth | HEVC | Minimal, high level |

Table 2. Available alternative for disparity derivation, implementation parameters

| Disparity Derivation | Complexity | Memory | Changes to HEVC |
| --- | --- | --- | --- |
| Type 1 | >7 * dDisparity /block: 350/block | Medium, local buffer | Medium, high level (frame) |
| Type 2 | Variable, worse case: > 36 / block | Medium, long term | Very large, low level |
| Type 3 | Variable, worse case: > 40 / block | Medium, long term | Very large, low level |
| Type 4 | Fixed: 2 ops / block | No additional memory | Minimal, high level |

# Proposal

In this contribution we propose to acknowledge, that 3D-HEVC specifies several tools that utilize disparity information for compression of dependent texture views. None of these tools are normative in HEVC/MV-HEVC and thus do not provide compatibility in multiview video content decoding with MV-HEVC decoders. The available disparity-based coding tools are agnostic toward the way disparity information is derived.

Further, we propose to acknowledge, that multiple alternative techniques for disparity derivation are available. These techniques have different characteristics e.g. in terms of compression efficiency, computational demands, and implementation complexity. None of these techniques are normative for HEVC/MV-HEVC, thus they shall be equally assessed in terms of rate-distortion gain vs. computation complexity and implementation complexity.

It is noted that the choice of the disparity derivation procedure can depend on the 3DV use case. For example, Type 2 can be used for when depth data is not available in the encoding side. Type 1 and Type 4 derivations may be used in low-complexity MVD applications, where presence of the depth information would allow a significant gain in MVD compression with small changes to the HEVC base coding scheme.

Said flexibility on letting the en encoder side to select the available disparity derivation technique can be facilitated by adopting the FCO signaling in the 3D-HEVC specification. The proposed signaling allows the encoder to specify the decoding order of texture and depth view components, including texture-first and depth-first coding configuration as well as unpaired MVD.

Section 4 includes the proposed specification text changes.

# Simulation results

To support analysis provided in this document, we integrated FCO and the proposed disparity derivation technique to 3D-HTM and executed it with CTC. In our work we replaced the disparity derivation procedure utilized in the 3D-HTM. The disparity was computed from depth map samples associated with current PU of video component.

To enable this method, 3D-HTM was executed under the CTC, with following changes applied:

FCO:					1

FCOCodingOrder		T0D0D1D2T1T2

VSOConfig:

[ox0 B(cc1) I(s0.25 s0.5 s0.75)][cx1 B(oo0) B(oo2) I(s0.25 s0.5 s0.75 s1.25 s1.5 s1.75)][ox2 B(cc1) I(s1.25 s1.5 s1.75)]

Depth to disparity conversion has been conducted through a lookup table mapping, table was constructed with equation (1), where D is disparity, d is depth map ranging from 0 to 255, w, o and n are parameters of conversion.

D = (w * d + o ) >> n						(1)

Equation (1) is identical equivalent to  full scale depth to disparity conversion, which may be performed as shown in (2):

D=f⋅l⋅d(22−1)1Znear−1Zfar+1Zfar					(2)

where f – is focal length, l is translation between two views, Znear/zfar are parameters of depth map quantization.

A single disparity value was computed for every PU, depth value associated with central sample of PU was utilized.

If Depth_PU is the depth block associated with PU then,

X = (PU_width>>1)-1

Y = (PU_height>>1)-1

d = Depth_PU(X,Y) is used in equation (1) to get the disparity vector for the given PU block.

This implies computational complexity of tested disparity derivation equal to 2 operations: read & map. Resulting disparity value was utilized in AMVP and Residual prediction, as it specified in 3D-HEVC.

Table 3 shows simulation results produced with tested scheme and compared against the 3D-HTM anchor 5.1.

Table 3. 3D-HTM with proposed disparity derivation compared against the HTM 5.1 anchor results.

|  | video 0 | video 1 | video 2 | video only | synthized only | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.9% | -1.2% | -0.6% | 0.4% | 100.0% | 96.6% | 100.0% |
| Kendo | 0.0% | -1.9% | -1.0% | -0.6% | 2.1% | 100.0% | 96.8% | 100.0% |
| Newspaper | 0.0% | -1.0% | -0.4% | -0.3% | 0.1% | 100.0% | 98.4% | 100.0% |
| GT_Fly | 0.0% | -4.7% | -3.8% | -1.2% | 1.3% | 100.0% | 92.2% | 100.0% |
| Hall | 0.0% | -1.5% | -2.8% | -0.9% | 1.9% | 100.0% | 80.7% | 100.0% |
| Street | 0.0% | -2.1% | -1.7% | -0.7% | 1.2% | 100.0% | 89.2% | 100.0% |
| Dancer | 0.0% | -3.3% | -2.5% | -1.1% | 1.3% | 100.0% | 88.4% | 100.7% |
| 1024x768 | 0.0% | -1.6% | -0.9% | -0.5% | 0.9% | 100.0% | 97.3% | 100.0% |
| 1920x1088 | 0.0% | -2.9% | -2.7% | -1.0% | 1.4% | 100.0% | 87.5% | 100.2% |
| average | 0.0% | -2.3% | -1.9% | -0.8% | 1.2% | 100.0% | 91.6% | 100.1% |

To validate the FCO in “texture-first coding order” and Type 2 disparity derivation, 3D-HTM was executed under the CTC, the following changes applied:

FCO:					1

FCOCodingOrder		T0D0T1D1T2D2

Simulation results validating that given configuration is identical to the 5.1 HTM anchor is shown in Table 4.

Table 4. 3D-HTM with FCO=1 and CTC specified coding order, compared against the HTM 5.1 anchor results.

|  | video 0 | video 1 | video 2 | video only | synthized only | Video & synthized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| Hall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 100.0% |

# Proposed specification text

## Signalling for flexible coding order

The video parameter set design based on JCTVC-K1007 is used below with the exclusion of standards scalability support (which was believed to be less mature than the other features). In the video_parameter_set_rbsp syntax structure the changes syntax elements relative to HEVCv1 are indicated with highlights.

- Video parameter set RBSP

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_extension_offset | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| vps_op_sets_num_minus1 | ue(v) |
| for( i = 1; i <= vps_num_op_sets_minus1; i++ ) { |  |
| operation_point_set( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_set_idx[ i ] |  |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| vps_extension( ) |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- Video parameter set RBSP semantics

The specifications in subclause 7.4.2.1 apply with following modifications and additions.

max_num_layers_minus1 plus 1 specifies the maximum number of layers in the coded video sequences referring to the video parameter set. The value of vps_max_layers_minus1 shall be in the range of 0 to 63, inclusive.

vps_extension_offset specifies the byte offset of the next set of fixed-length coded information in the video parameter set NAL unit, starting from the beginning of the NAL unit.

NOTE –Video parameter set information for non-base layer or view starts from a byte-aligned position of the video parameter set NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset specified by vps_extension_offset would then help to locate and access those essential information in the video parameter set NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the video parameter set that is essential for session negotiation and/or capability exchange.

vps_extension_flag equal to 0 specifies that no vps_extension( ) syntax structure is present in the video parameter set RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension( ) syntax structure is present in the video parameter set RBSP syntax structure. When max_num_layers_minus1 is greater than 0, vps_extension_flag shall be equal to 1.

vps_extension2_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension2_flag shall be equal to 1 in bitstreams conforming to annex Error: Reference source not found of this Recommendation | International Standard. The value of 1 for vps_extension2_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension2_flag in a video parameter set NAL unit.

- Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0, numScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| numScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <numScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < numScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

- Video parameter set extension semantics

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

scalability_mask[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table XX are present. scalability_mask[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table XX. Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | reference index based spatial or quality scalability | DependencyId |
| 1 | depth | DepthFlag |
| 2 | multiview | ViewId |
| 3-15 | Reserved |  |

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

vps_nuh_layer_id_present_flag specifies whether the layer_id_in_nuh[ i ] syntax is present.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. When not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

The variable LayerIdInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

The variables ScalabilityId[ layerIdInVps ][ scalabilityMaskIndex ], DependencyId[ layerIdInNuh ], DepthFlag[ layerIdInNuh ], and ViewOrderIdx[ layerIdInNuh ] are derived as follows:

for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  !=  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

DependencyId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

DepthFlag[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ]

ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 2 ]

}

num_direct_ref_layers[ i ] specifies the number of layers the i-th layer directly references. [Ed.(YK): Add the exact meaning of a layer directly references another layer.] When not present, the value of num_direct_ref_layers[ i ] is inferred to be equal to 0.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly references.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).