| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0171 |
| --- | --- |

| Title: | CE1.h-related: Backward VSP for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Srikanth Gopalakrishna Dmytro Rusanovskyy Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

In this proposal we propose to utilize the disparity information derived from the depth view component of the current view for Backward View Synthesis Prediction with a block-based implementation. The provided simulation results show that the proposed scheme, using a 4x4 block size, provided a coding gain for texture of 1.1% of dBR on average and of about 3% bitrate reduction for synthetic sequences. Using a pixel-based implementation of proposed BVSP brought about 1.4% bit-rate reductions for the coded texture.

However, it should be noted that the reconfiguration of the coding order lead to a change in the VSO settings, which in turn introduced a penalty on depth coding and of 0.6% of dBR increase on average  for synthesised views. It is believed that a proper adjustment of VSO parameters would resolve this problem.

Considering the negligible computational complexity and the observed gain for texture coding, the contribution proposes to integrate proposed BVSP solution to 3D-HTM for its FCO configuration.

# Introduction

Flexible Coding Order (FCO), which is integrated in 3D-HTM, allows the encoder side to choose whether a texture view component follows or precedes the depth view component of the same non-base view. FCO also enables an additional approach for disparity derivation. When the encoder side chooses to code a depth view component before the associated texture view component, FCO enables derivation of disparity information directly from the depth view component, which reportedly has low computational complexity and causes no additional memory usage.

The 3D-HEVC [] specifies utilization of disparity information for coding of dependent views of multiview video and MVD data. For example, the following coding tools can use disparity information:

- Adaptive Motion Vector Prediction (AMVP).

- Interview Residual Prediction.

- View Synthesis Prediction.

There are several disparity derivation approaches which are specified in 3D-HEVC available. However, these methods features quite significant computational complexity, memory requirements and would lead to significant implementation efforts. In our development we utilized derivation technique proposed in [].

Disparity derivation method that is proposed in [] performs disparity derivation from depth map data associated with currently coded texture view. The disparity derivation is straightforward: accessing depth samples which being coded earlier and converting them to disparity through a look up table search. This method has virtually no computation complexity and no additional memory requirements, since all required data is available within the same access unit.

In this method, depth to disparity conversion conducted through a lookup table mapping, table was constructed with equation (1), where D is disparity, d is depth map ranging from 0 to 255, w, o and n are parameters of conversion.

D = (w * d + o ) >> n						(1)

Equation (1) is identical equivalent to  full scale depth to disparity conversion, which may be performed as shown in (2):

D=f⋅l⋅d(22−1)1Znear−1Zfar+1Zfar					(2)

where f – is focal length, l is translation between two views, Znear/zfar are parameters of depth map quantization.

Considering implementation (2), this disparity derivation method requires 2 basic operations: read & map, require no additional memory and require minimal changes to spec text.

# Proposal

In this contribution we consider View Synthesis Prediction in 3D-HEVC and present a very low complexity solution, which requires no additional memory, implemented with minimal number of operations, and thus would introduce minimal changes to specification text. The visualization of the proposed technique given in Figure 1.

Let us assume that the following coding order is utilized: (T0, D0, D1, T1). Texture component T0 is a base view and T1 is dependent view coded with the VSP. Depth map components D0 and D1 are respective depth map associated with T0 and T1. 3D-HTM is able to code MVD data in given coding order with enable FCO=1.

In dependent view T1, sample values of currently coded block Cb are predicted from reference area R(Cb) that consists of sample values of the base view T0. The displacement vector (motion vector) between coded and reference samples is found as a disparity between T1 and T0 from a depth map value associated with a currently coded texture sample. In such implementation, B-VSP is comparable with a tradition Motion Compensated Prediction (MCP) where displacement (motion) vectors are replaced with disparity vectors.

In proposed solution, the BVSP is implemented as a separate coding mode and signaled at the CU level.

Encoder thus considers the BVSP coding mode while performing rate distortion optimization for CU along with inter and intra modes. CU is divided into blocks of size 4x4, disparity vector for each block is estimated by considering the maximum depth out of corners of the block and the center depth pixel in the block.

d1 = d(0,0));		d2 = d(0,size_x-1));

d3 = d(size_y-1,0));	d4 = d(size_y-1,size_x-1));	d5= d (size_y>>1 - 1,size_x>>1 - 1);

D = Disp(max(d1,max(d2,max(d3,max(d4,d5)))))

where d1-d5 are depth map samples representing corner locations of CU and D is resulting disparity vector.

Figure 1. Visualization of Block-based VSP (B-VSP) based on backward warping

Signaling of the BVSP coding mode is performed at the CU level.

coding_unit( x0, y0, log2CbSize ) {

if( transquant_bypass_enable_flag ) {

cu_transquant_bypass_flag

}

if( slice_type != I )

If(((texture == 1) && (depth_map_coded_flag == 1) && (view_id != 0)) skip_flag_or_bvsp_flag[ x0 ][ y0 ] else skip_flag[x0][y0]

if( skip_flag_or_bvsp_flag[ x0 ][ y0 ] ) If(((texture == 1) && (depth_map_coded_flag == 1) && (view_id != 0)){ Bvsp_flag[x0][y0] If(Bvsp_flag[x0][y0]) Encode_residual() Else

prediction_unit( x0, y0, log2CbSize )} else { prediction_unit( x0, y0, log2CbSize )}

# Simulation results

We integrated the proposed BVSP method to 3D-HTM. To enable this method, 3D-HTM was executed under the CTC, with following changes applied:

FCO:					1

FCOCodingOrder		T0D0D1D2T1T2

VSOConfig:

[ox0 B(cc1) I(s0.25 s0.5 s0.75)][cx1 B(oo0) B(oo2) I(s0.25 s0.5 s0.75 s1.25 s1.5 s1.75)][ox2 B(cc1) I(s1.25 s1.5 s1.75)]

Simulation results produced with pixel-based BVSP and compared against the 3D-HTM anchor 5.1 are shown in Table 1

Table 1. 3D-HTM with pixel-based BVSP compared against the HTM 5.1 anchor results.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.5% | -1.2% | -0.5% | 0.4% |
| Kendo | 0.0% | -1.3% | -0.4% | -0.3% | 2.2% |
| Newspaper_CC | 0.0% | -0.6% | -0.1% | -0.1% | 0.2% |
| GT_Fly | 0.0% | -12.3% | -11.4% | -3.0% | -0.2% |
| Poznan_Hall2 | 0.0% | -0.8% | -2.4% | -0.6% | 2.1% |
| Poznan_Street | 0.0% | -3.9% | -3.1% | -1.1% | 0.8% |
| Undo_Dancer | 0.0% | -14.8% | -13.5% | -4.1% | -1.3% |
| 1024x768 | 0.0% | -1.1% | -0.6% | -0.3% | 0.9% |
| 1920x1088 | 0.0% | -8.0% | -7.6% | -2.2% | 0.4% |
| Average | 0.0% | -5.0% | -4.6% | -1.4 % | 0.6 % |

As it seen from Table 1, proposed in this document technique provides about 1.4% of coding gain for texture data on average, and 12% bitrate reduction is achieved for synthetic sequences. However, the change in coding order modifies settings of depth coding tools (mostly VSO) which lead to an increase in depth map bitrate, and respectively to penalty in dBR of synthesised views by 0.6%. It is believe that proper adjusting of VSO parameters would resolve this problem.

Simulation results produced with block-based BVSP and compared against the 3D-HTM anchor 5.1 are shown in Table 2

Table 1. 3D-HTM with block-based (4x4) BVSP compared against the HTM 5.1 anchor results.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.3% | -0.7% | -0.4% | 0.5% |
| Kendo | 0.0% | -1.1% | -0.3% | -0.2% | 2.4% |
| Newspaper_CC | 0.0% | -0.5% | 0.2% | 0.0% | 0.3% |
| GT_Fly | 0.0% | -11.5% | -10.8% | -2.8% | 0.0% |
| Poznan_Hall2 | 0.0% | -0.6% | -2.5% | -0.6% | 2.2% |
| Poznan_Street | 0.0% | -3.9% | -3.2% | -1.1% | 0.8% |
| Undo_Dancer | 0.0% | -9.8% | -8.3% | -2.6% | 0.0% |
| 1024x768 | 0.0% | -1.0% | -0.3% | -0.2% | 1.1% |
| 1920x1088 | 0.0% | -6.5% | -6.2% | -1.8% | 0.7% |
| Average | 0.0% | -4.1% | -3.6% | -1.1% | 0.9% |

Considering the negligible computational complexity and observed gain for texture coding, we propose to integrate BVSP solution to 3D-HTM for FCO configuration.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).