| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0172 |
| --- | --- |

| Title: | CE1.h: Cross-check of BVSP results from MERL (JCT3V-C0152) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Srikanth Gopalakrishna, Dmytro Rusanovskyy | Email: | firstname.secondname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document is a crosscheck of JCT3V-C0152 proposal from MERL.

Current document reports results of Nokia cross-check for Pixel-based BVSP and block-based BVSP size of 4x4. Cross checker confirmed that the software and the text provided by the proponent are consistent and simulation results produced by cross-checker are matched to those reported by proponents in C0152.

Encoder times recorded by the cross-checker are matches to those reported by proponents, decoding time reported by cross-checker are somewhat higher (due to a significant cluster load). Decoding time estimates are expected to be updated during the meeting.

# Introduction

The document C0152 proposes a Backward View Synthesis Prediction for adoption to for 3D-HTM and CTC. Proposed BVSP utilizes disparity vectors derived from spatio-temporal neighbourhood of currently coded block and signaled as a new candidate in the merge candidate list. Pixel-based and block based implementation of BVSP are proposed.

C0152 reports the following results:

- Pixel-based BVSP produces 1% of dBR on average for coded&synthesed views with 3% of increase in the decoding time.

- Block based BVSP of size 2x2 produces 0.9% of dBR on average for coded & synthesised  views with 4% increase in the decoding time. (not cross-checked in this document)

- Block based BVSP of size 4x4 produces 0.8% of dBR on average for coded & synthesised  views with 5% increase in the decoding time.

# Cross-check results

- Coding and view synthesis results produced with MERL software for 1x1 and 4x4 implementation of BVSP was found to be identical to results reported in C0152

- All coded bitstreams were found to be decodable and no mismatch was observed between reconstructed and decoded YUV files.

- Encoding time estimates are well matched to results reported by proponent, decoding time estimates reported by cross-checker are somewhat higher due to used cluster load.

# Experimental results

Table 1. Cross-checker’s results on C0152 proposal, BVSP with 1x1 implementation

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time |  | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.9% | -0.5% | -0.2% | -0.2% | -0.3% | 104.6% |  | 100.0% |
| Kendo | 0.0% | -1.6% | -1.8% | -0.5% | -0.4% | -0.6% | 104.1% |  | 100.0% |
| Newspapercc | 0.0% | -0.2% | -0.7% | -0.1% | -0.1% | -0.2% | 95.3% |  | 100.0% |
| GhostTownFly | 0.0% | -9.0% | -8.7% | -2.4% | -2.2% | -1.7% | 96.3% |  | 100.0% |
| PoznanHall2 | 0.0% | -0.4% | -2.8% | -0.6% | -0.5% | -0.7% | 99.5% |  | 100.0% |
| PoznanStreet | 0.0% | -2.9% | -3.0% | -0.9% | -0.8% | -0.7% | 103.3% |  | 100.0% |
| UndoDancer | 0.0% | -12.4% | -10.8% | -3.4% | -3.0% | -2.2% | 100.9% |  | 100.7% |
| 1024x768 | 0.0% | -0.9% | -1.0% | -0.3% | -0.2% | -0.4% | 101.2% |  | 100.0% |
| 1920x1088 | 0.0% | -6.2% | -6.3% | -1.8% | -1.6% | -1.3% | 100.0% |  | 100.2% |
| average | 0.0% | -3.9% | -4.1% | -1.2% | -1.0% | -0.9% | 100.5% |  | 100.1% |

Table 2. Cross-checker’s results on C0152 proposal, BVSP with 4x4 implementation

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time |  | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.8% | -0.6% | -0.2% | -0.2% | -0.3% | 97.9% |  | 100.0% |
| Kendo | 0.0% | -1.4% | -1.7% | -0.5% | -0.4% | -0.6% | 104.5% |  | 100.0% |
| Newspapercc | 0.0% | -0.1% | -0.5% | 0.0% | -0.1% | -0.2% | 101.5% |  | 100.0% |
| GhostTownFly | 0.0% | -8.0% | -8.0% | -2.1% | -1.9% | -1.6% | 101.9% |  | 100.0% |
| PoznanHall2 | 0.0% | -0.1% | -2.4% | -0.5% | -0.3% | -0.5% | 96.3% |  | 100.0% |
| PoznanStreet | 0.0% | -2.4% | -2.7% | -0.8% | -0.7% | -0.6% | 101.6% |  | 100.0% |
| UndoDancer | 0.0% | -8.2% | -7.0% | -2.2% | -1.9% | -1.6% | 104.5% |  | 100.7% |
| 1024x768 | 0.0% | -0.8% | -0.9% | -0.2% | -0.2% | -0.3% | 101.2% |  | 100.0% |
| 1920x1088 | 0.0% | -4.7% | -5.0% | -1.4% | -1.2% | -1.1% | 101.0% |  | 100.2% |
| average | 0.0% | -3.0% | -3.3% | -0.9% | -0.8% | -0.8% | 101.1% |  | 100.1% |

# Reference

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”