| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0172 |
| --- | --- |

| Title: | CE1.a: Cross-check of BVSP configurations in 3D-ATM proposed by Mediatek (JCT3V-C0130) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Payman Aflaki, Dmytro Rusanovskyy | Email: | firstname.secondname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document is a crosscheck of test configurations Test1,2,5 and 7 proposed by Mediatek in JCT3V-C0130. Cross checker confirmed that simulation results produced by cross-checker are matched to those reported by proponents in C0130, except of execution time. Cross-check was performed on the linux cluster, thus execution time estimates may not be reliable.

# Introduction

Proposal C0130 introduces and test several BVSP configurations in 3DV-ATM software.

Test 1 studies 3DV-ATM v6.1 reference software with BVSP size of 4x4.

No coding penalty is reported (~0.01% of dBR) on average in comparison to the anchor configuration 2x2. Maximal penalty of 0.12% of dBR is observed for Ballon sequence.

Test 2 studies 3DV-ATM v6.1 reference software with BVSP size of 4x4 and average-based disparity derivation method for block BVSP.

Insignificant coding penalty is reported (~0.07% of dBR) on average in comparison to the anchor configuration 2x2. Maximal penalty of 0.16% of dBR is observed for Ballon sequence.

Test 5 modifies BVSP size of 4x4 with not-transmitting reference index. Disparity derivation algorithm is maximal out of 4 corner candidates.

Coding gain of -0.14% of dBR on average in comparison to the anchor configuration 2x2 is reported. Maximal gain of -0.44% of dBR is observed for GT_Fly sequence.

Test 7 is different from Test 5 in the disparity derivation algorithm. An average of depth samples within a block is used for disparity derivation.

Coding penalty of 0.55% of dBR on average in comparison to the anchor configuration 2x2 is observed Maximal penalty of 1.16% of dBR is observed for the Dancer sequence.

# Cross-check results

- Coding and view synthesis results produced with proponent’s software were found to be identical to results reported in C0130.

- All coded bitstreams were found to be decodable and no mismatch was observed between reconstructed and decoded YUV files.

- Encoding time estimates are well matched to results reported by proponent, decoding time estimates reported by cross-checker are somewhat higher due to used cluster load.

# Experimental results

Table 1. Cross-checker’s results on C0130 proposal, test 1 configuration.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -0.07 | 0.00 | 0.00 | 0.00 | -0.07 | 0.00 | 0.02 | 0.00 |
| S02 | -0.06 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.05 | 0.00 |
| S03 | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.00 | 0.00 |
| S04 | -0.15 | 0.01 | 0.00 | 0.00 | -0.13 | 0.00 | -0.15 | 0.01 |
| S05 | 0.06 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.07 | 0.00 |
| S06 | 0.12 | -0.01 | 0.00 | 0.00 | 0.11 | -0.01 | 0.12 | -0.01 |
| S08 | 0.05 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.06 | 0.00 |
| Average | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.01 | 0.00 |

Table 2. Cross-checker’s results on C0130 proposal, test 2 configuration.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -0.13 | 0.00 | 0.00 | 0.00 | -0.12 | 0.00 | -0.05 | 0.00 |
| S02 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.04 | 0.00 |
| S03 | 0.11 | 0.00 | 0.00 | 0.00 | 0.11 | 0.00 | 0.12 | 0.00 |
| S04 | -0.02 | 0.00 | 0.00 | 0.00 | -0.02 | 0.00 | -0.02 | 0.00 |
| S05 | 0.13 | -0.01 | 0.00 | 0.00 | 0.10 | 0.00 | 0.14 | -0.01 |
| S06 | 0.16 | -0.01 | 0.00 | 0.00 | 0.13 | -0.01 | 0.16 | -0.01 |
| S08 | 0.09 | 0.00 | 0.00 | 0.00 | 0.07 | 0.00 | 0.10 | 0.00 |
| Average | 0.05 | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 | 0.07 | 0.00 |

Table 3. Cross-checker’s results on C0130 proposal, test 5 configuration.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -0.15 | 0.00 | 0.00 | 0.00 | -0.15 | 0.00 | -0.05 | 0.00 |
| S02 | -0.36 | 0.01 | 0.00 | 0.00 | -0.33 | 0.01 | -0.28 | 0.01 |
| S03 | -0.23 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.17 | 0.01 |
| S04 | -0.51 | 0.02 | 0.00 | 0.00 | -0.48 | 0.02 | -0.44 | 0.02 |
| S05 | -0.14 | 0.01 | 0.00 | 0.00 | -0.12 | 0.01 | -0.05 | 0.00 |
| S06 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.08 | 0.00 |
| S08 | -0.06 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.04 | 0.00 |
| Average | -0.21 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 | -0.14 | 0.00 |

Table 4. Cross-checker’s results on C0130 proposal, test 7 configuration.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | 0.19 | 0.00 | 0.00 | 0.00 | 0.16 | 0.00 | 0.29 | -0.01 |
| S02 | 0.42 | -0.01 | 0.00 | 0.00 | 0.37 | -0.01 | 0.41 | -0.01 |
| S03 | 1.26 | -0.04 | 0.00 | 0.00 | 1.17 | -0.04 | 1.14 | -0.03 |
| S04 | 0.23 | -0.01 | 0.00 | 0.00 | 0.21 | -0.01 | 0.23 | -0.01 |
| S05 | 0.51 | -0.02 | 0.00 | 0.00 | 0.32 | -0.01 | 0.69 | -0.03 |
| S06 | 0.70 | -0.03 | 0.00 | 0.00 | 0.56 | -0.03 | 0.89 | -0.04 |
| S08 | 0.22 | -0.01 | 0.00 | 0.00 | 0.16 | -0.01 | 0.29 | -0.01 |
| Average | 0.50 | -0.02 | 0.00 | 0.00 | 0.42 | -0.02 | 0.56 | -0.02 |

# Reference

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”