| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0175 |
| --- | --- |

| Title: | 3D-CE5.h: Cross check on Additional merge candidates derived from shifted disparity candidate predictors (JCT3V-C0148) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document is a crosscheck of the proposal JCT3V-C0148. In this proposal, two new candidates are added which are derived from the first available disparity compensated candidate in the constructed merge candidate list with a horizontal shift of -4 or +4. Simulations have been conducted and the PSNR and bitrate results are identical with those provided by the proponents.

# Introduction

The proponents of JCT3V-C0148 [1] propose to modify the candidate merge list in adding up to two new candidates. They are derived from the first candidate of the constructed merge candidate list in either adding 4 (for the left candidate) or subtracting 4 (for the right candidate) to the horizontal component of the disparity motion vector. The newly introduced candidates are then inserted to the merge candidate list with shifting the existing candidate to a later position.

# Source code verification

In contribution JCT3V-C0148, the candidate list construction process is as follows with the newly introduced part described in step 4:

- A disparity vector is derived and the corresponding block of current PU based on the disparity vector is identified. The inter-view predicted motion vector candidate derived from the corresponding block, if available, is added to the merge candidate list.

- Spatial merging candidates (SMC), disparity motion vector converted from the disparity vector, and temporal merging candidate are added to the merge candidate list after pruning.

- Other motion vector merging candidates are added to the merge candidate list if the number of valid candidates is less than the maximum number.

- Candidates from shifted disparity vector predictors:

  - Loop the first four available candidates in the candidate list constructed by step 1-3 to find the first disparity compensated predictor by checking the reference picture list 0 and 1, if available, of each candidate in order;

  - Create two new candidates in order, with the found disparity compensated predictor horizontally shifted by 4 and -4 for view 1, -4 and 4 for view 2. The motion information of the other reference picture list for B slices is kept unchanged.

  - Change the position of one existing merge candidate and then insert the new candidates into the candidate merge list and the inserted positions are dependent on the merge candidate list size.

The proposed method is implemented to HTM 5.0.1 [2] with the macro ‘INRIA_NEW_MERGE_CAND’ set to 1.

# Results

This section provides the simulation results of the proposed method in comparison with the 3D-HTM anchor. The 3D-HTM 5.0.1 is utilized and the method of the proponent is implemented on top of it. All tests are run based on the common tests conditions [3].

Table 1: Coding gains of JCT3V-C0148 with respect to anchor for 3-view case

|  | Video 0 | Video 1 | Video 2 | Video only | synthesized only | coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 95.6% | 101.6% |
| Kendo | 0.0% | -0.4% | -0.7% | -0.3% | -0.1% | -0.1% | 97.6% | 98.4% | 94.6% |
| Newspapercc | 0.0% | -0.7% | -0.6% | -0.3% | -0.2% | -0.2% | 104.2% | 99.7% | 103.9% |
| GhostTownFly | 0.0% | -0.5% | -0.9% | -0.2% | -0.2% | -0.2% | 98.3% | 105.2% | 103.2% |
| PoznanHall2 | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 92.2% | 100.5% | 101.1% |
| PoznanStreet | 0.0% | -0.7% | -0.6% | -0.3% | -0.2% | -0.2% | 94.5% | 104.4% | 98.5% |
| UndoDancer | 0.0% | -0.3% | -0.4% | -0.1% | -0.2% | -0.2% | 99.6% | 112.6% | 106.2% |
| 1024x768 | 0.0% | -0.3% | -0.4% | -0.2% | -0.1% | -0.1% | 100.5% | 97.9% | 99.9% |
| 1920x1088 | 0.0% | -0.4% | -0.5% | -0.2% | -0.2% | -0.2% | 96.1% | 105.6% | 102.2% |
| average | 0.0% | -0.4% | -0.5% | -0.2% | -0.1% | -0.2% | 97.9% | 102.2% | 101.2% |

Comments about the results:

- The obtained bit rate gains match those provided by the proponent.

- Coding time is somewhat different from the one provided by the proponent due to different platforms used.

# References

- C. Guillemot, L. Guillo, " 3D-CE5.h: Additional merge candidates derived from shifted disparity candidate predictors", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0148, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- 3DV-HTM version 5.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1110, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.