| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0176 |
| --- | --- |

| Title: | 3D-CE3.h: Cross check on removal of parsing dependency and picture buffers for motion parameter inheritance (JCT3V-C0137) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document is a crosscheck of the proposal JCT3V-C0137. In this proposal, an additional merging candidate to inherit the motion information of the corresponding texture block is proposed for merge and skip mode to replace the original motion parameter inheritance (MPI) method. Simulations have been conducted and the PSNR and bitrate results are identical with those provided by the proponents.

# Introduction

The proponents of JCT3V-C0137 [1] propose to replace MPI by adding one merge candidate which is derived from the corresponding texture block. For each depth prediction unit (PU), the corresponding texture block is identified by the center position of current PU. The block that covers the center position of current PU in the texture view is used as the corresponding texture block and its associated motion information is reused as an additional candidate. The additional candidate is inserted to the merge candidate list before spatial merging candidates.

# Source code verification

The proposed method is implemented to HTM 5.0.1 [2] with the macro ‘MTK_DEPTH_MERGE_TEXTURE_CANDIDATE’ set to 1.

The software matches the description of the proposal and it has been successfully compiled in Linux environment.

# Results

This section provides the simulation results of the proposed method in comparison with the 3D-HTM anchor. The 3D-HTM 5.0.1 is utilized and the method of the proponent is implemented on top of it. All tests are run based on the common tests conditions [3].

Table 1: Coding gains of JCT3V-C0137 with respect to anchor for 3-view case

|  | Video 0 | Video 1 | Video 2 | Video only | synthesized only | coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% | 94.7% | 98.0% | 94.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 93.0% | 100.1% | 94.9% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 94.4% | 96.0% | 95.7% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% | 91.0% | 99.8% | 97.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.5% | 97.1% | 102.3% | 95.9% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 89.8% | 102.9% | 89.3% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 91.3% | 95.8% | 93.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 94.0% | 98.1% | 95.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 92.3% | 100.2% | 93.8% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 93.0% | 99.3% | 94.3% |

Comments about the results:

- The obtained bit rate gains match those provided by the proponent.

- Coding time is somewhat different from the one provided by the proponent due to different platforms used.

# References

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, "3D-CE3.h results on removal of parsing dependency and picture buffers for motion parameter inheritance," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0137, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- 3DV-HTM version 5.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1110, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.