| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0177 |
| --- | --- |

| Title: | 3D-CE3.h: Cross check on Improved Motion Parameter Inheritance (JCT3V-C0095) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document is a crosscheck of the proposal JCT3V-C0095. In this proposal, the inheritance of coding unit (CU) subdivision information in current motion parameter inheritance (MPI) method is removed when the size of depth CU is larger than that of texture CU. Simulations have been conducted and the PSNR and bitrate results are identical with those provided by the proponents.

# Introduction

In current design of MPI, when the size of current depth CU is larger than that of texture CU, both the motion information and the CU subdivision information are both inherited. The proponents of JCT3V-C0095 [1] propose to just reuse the motion information to avoid the parsing issue. When the size of current depth CU is smaller or equal to that of texture CU, the current design is kept unchanged, i.e., only motion information used.

# Source code verification

The proposed method is implemented to HTM 5.0.1 [2] with the macro ‘SAIT_MPI’ set to 1.

The source code has been successfully compiled in Linux environment.

# Results

This section provides the simulation results of the proposed method in comparison with the 3D-HTM anchor. The 3D-HTM 5.0.1 is utilized and the method of the proponent is implemented on top of it. All tests are run based on the common tests conditions [3].

Table 1: Coding gains of JCT3V-C0095 with respect to anchor for 3-view case

|  | Video 0 | Video 1 | Video 2 | Video only | synthesized only | coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 99.4% | 99.7% | 97.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 95.1% | 95.4% | 94.5% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.1% | 97.8% | 100.1% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 88.2% | 101.0% | 92.2% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -0.4% | 99.8% | 100.1% | 99.4% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 88.8% | 99.4% | 90.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 91.6% | 97.6% | 96.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 97.5% | 97.6% | 97.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 92.0% | 99.5% | 94.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 94.3% | 98.7% | 95.8% |

Comments about the results:

- The obtained bit rate gains match those provided by the proponent.

- Coding time is somewhat different from the one provided by the proponent due to different platforms used.

# References

- B. T. Oh, K.-J. Oh, H.-C. Wey, D.-S. Park, "3D-CE3.h results on Improved Motion Parameter Inheritance," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0095, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- 3DV-HTM version 5.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1110, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.