| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0193 |
| --- | --- |

| Title: | 3D-HEVC: Crosscheck of Alignment of inter-view vector scaling (JCT3V-C0115) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of inter-view vector scaling modification. Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It was confirmed by the cross-checker that there is some discrepancy between HTM and Test Model Description [JCT3V-B1005] where HTM uses view order index and Test Model uses view_id.  JCT3V-C0115 fix this discrepancy by using view_id in HTM.  It is reported that the case of IV scaling on with this view_idx implementation has coding gain of 0.6 % in coded & synthesized compared with the case of IV scaling off.  Considering a possibility that view idx design might not be useful in some cases, JCT3V-C0115 also proposes to add a control flag iv_vector_scaling_flag in SPS extension.

- It was recommended to solve this discrepancy by either way HTM follow Text or Text follows HTM.

# Verification

Test Model Description uses view idx in inter-view vector scaling as shown bellow, but HTM uses view order index for implementation.

ViewId( picX ) = view_id of the picture picX		(G 97)

Note: in algorithm description, Test Model Description also uses view order index as shown below.

td = Clip3( -128, 127, ColViewOrderIdx – ColRefViewOrderIdx )

tb = Clip3( -128, 127, CurrViewOrderIdx – CurrRefViewOrderIdx )

The variables in the above equations are specified as follows:

CurrViewOrderIdx :ViewOrderIdx of current picture

ColViewOrderIdx :ViewOrderIdx of co-located picture

It was asserted by the JCT3V-C0115’s proponent that HTM should follow the Test Model Description.

Table 1 Test condition

|  | Anchor | Test |
| --- | --- | --- |
| TMVP inter view scaling | No (not used in CTC) | Yes |
| new SPS flag | No (same as CTC) | Yes |
| Prediction Structure | V2 refer two pictures: V0 and V1 (In CTC, only one picture can be refer as inter-view picture) |  |

# Experimental results

The experiment result on a condition by cross-checker is shown in Table 1.

Table 1 The results of inter-view vector scaling (anchor: inter-view vector scaling: off)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -3.4% | -0.6% | -0.6% | -0.6% | 100.3% | 97.9% | 100.0% |
| Kendo | 0.0% | 0.0% | -4.8% | -0.9% | -0.7% | -0.7% | 100.5% | 99.5% | 99.6% |
| Newspapercc | 0.0% | 0.0% | -3.7% | -0.7% | -0.7% | -0.7% | 99.7% | 99.5% | 100.3% |
| GhostTownFly | 0.0% | 0.0% | -6.4% | -0.7% | -0.6% | -0.6% | 98.9% | 100.8% | 101.0% |
| PoznanHall2 | 0.0% | 0.0% | -3.3% | -0.7% | -0.6% | -0.6% | 99.2% | 99.9% | 99.6% |
| PoznanStreet | 0.0% | 0.0% | -5.9% | -0.8% | -0.7% | -0.7% | 98.9% | 99.1% | 98.8% |
| UndoDancer | 0.0% | 0.0% | -4.5% | -0.6% | -0.5% | -0.6% | 99.6% | 101.0% | 99.9% |
| 1024x768 | 0.0% | 0.0% | -3.9% | -0.7% | -0.7% | -0.7% | 100.2% | 99.0% | 100.0% |
| 1920x1088 | 0.0% | 0.0% | -5.0% | -0.7% | -0.6% | -0.6% | 99.2% | 100.2% | 99.8% |
| average | 0.0% | 0.0% | -4.6% | -0.7% | -0.6% | -0.6% | 99.6% | 99.7% | 99.9% |

# References

[1] Y. Takahashi, O. Nakagami, S. Hattori, T. Suzuki (Sony), “3D-HEVC: Alignment of inter-view vector scaling, “JCT3V-C0115, JCT3V 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).