| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0194 |
| --- | --- |

| Title: | 3D-HEVC: Crosscheck of Inter-view vector scaling for AMVP (JCT3V-C0116) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of inter-view vector scaling modification. Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It is reported that inter-view vector scaling in AMVP achieve 0.2% bdrate gain in one dependent view.

# Verification

Table 1 Test condition

|  | Anchor | Test | CTC |
| --- | --- | --- | --- |
| TMVP inter view scaling | Yes | Yes | No (Implemented but not used) |
| AMVP inter view scaling | No | Yes | No |
| new SPS flag | Yes | Yes | No |
| Prediction Structure | V2 refer two pictures: V0 and V1 |  | V2 refer one pictures: V0 |

# Experimental results

The experiment result on a condition by cross-checker is shown in Table 1.

Table 1 The results of AMVP inter-view vector scaling (anchor AMVP inter view scaling is off)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.7% | 99.6% | 99.5% |
| Kendo | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 99.7% | 100.2% | 98.9% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 100.4% | 98.4% | 98.7% |
| GhostTownFly | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 100.8% | 100.1% | 99.9% |
| PoznanHall2 | 0.0% | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% | 101.1% | 100.3% | 100.9% |
| PoznanStreet | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 100.9% | 101.4% | 101.8% |
| UndoDancer | 0.0% | 0.0% | -0.3% | 0.0% | -0.2% | -0.1% | 100.2% | 100.1% | 100.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 99.4% | 99.0% |
| 1920x1088 | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | 100.7% | 100.5% | 100.7% |
| average | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 100.2% | 100.0% | 100.0% |

# References

[1] Y. Takahashi, O. Nakagami, S. Hattori, T. Suzuki (Sony), “3D-HEVC: Inter-view vector scaling for AMVP, “JCT3V-C0116, JCT3V 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).