| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0195 |
| --- | --- |

| Title: | AHG13: Crosscheck of Constrained DV for inter-view data access (JCT3V-C0129) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of disparity vector constraints related proposals. Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It is reported that discarding DV constraints in some tools such as DV_MCP and residual prediction achieves coding gain on MVC non-rectified and 2D layout video but it takes small coding loss. It is also reported that imposing DV constraints in all tools leads to no coding loss on CTC but it yields significant coding loss on MVC non-rectified and 2D layout video.

# Verification

Table 1 Tested scheme

|  | HTM5.1 | Test 1 |  | Test 2 |
| --- | --- | --- | --- | --- |
| DV in DV_MCP | X component only | X and Y component |  |  |
| DV in Residual prediction | X component only | X component (DV constraint flag is 1) X and Y component (DV constraint flag is 0) |  |  |
| DV in Inter-view candidate | X and Y component (No constraint) | X component (DV constraint flag is 1) X and Y component (DV constraint flag is 0) |  |  |
| DV constraint flag | No exist | Yes in VPS (value is 0) | Yes in VPS (value is 1) |  |

# Experimental results

The experiment result on CTC by cross-checker is shown in Table 1 and Table 2

Table 1 Experimental results in CTC (DV constraint flag is 0)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.6% | 1.0% | 0.3% | 0.3% | 0.3% | 100.2% | 100.7% | 101.3% |
| Kendo | 0.0% | 0.2% | 0.4% | 0.1% | 0.1% | 0.1% | 99.9% | 102.2% | 99.8% |
| Newspapercc | 0.0% | 0.3% | 0.4% | 0.1% | 0.1% | 0.1% | 99.9% | 103.2% | 100.1% |
| GhostTownFly | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 102.2% | 99.8% |
| PoznanHall2 | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 100.1% | 102.1% | 100.0% |
| PoznanStreet | 0.0% | 0.2% | 0.2% | 0.1% | 0.0% | 0.1% | 100.1% | 102.6% | 100.4% |
| UndoDancer | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 99.9% | 102.5% | 100.3% |
| 1024x768 | 0.0% | 0.4% | 0.6% | 0.2% | 0.2% | 0.2% | 100.0% | 102.0% | 100.4% |
| 1920x1088 | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | 100.0% | 102.4% | 100.1% |
| average | 0.0% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 100.0% | 102.2% | 100.2% |

Table 2 Experimental results in CTC (DV constraint flag is 1)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 100.7% | 100.3% | 100.1% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 100.6% | 104.0% | 99.6% |
| Newspapercc | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 101.2% | 103.3% | 99.4% |
| GhostTownFly | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 102.0% | 99.5% |
| PoznanHall2 | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 100.2% | 102.3% | 100.0% |
| PoznanStreet | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 100.4% | 102.9% | 100.5% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 100.2% | 102.5% | 100.4% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.8% | 102.5% | 99.7% |
| 1920x1088 | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100.4% | 102.4% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 102.5% | 99.9% |

The experiment result on MVC by cross-checker is shown in Table 3 and Table 4

Table 3 Experimental results in MVC (DV constraint flag is 0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | -1.0% | -0.8% | -0.5% | -0.5% | #VALUE! | 100.0% | 101.9% | #NUM! |
| Flamenco2_2_1_3 | 0.0% | -2.6% | -2.3% | -0.9% | -0.9% | #VALUE! | 101.3% | 104.4% | #NUM! |
| Akko&Kayo_48_46_50 | 0.0% | 0.5% | 0.3% | 0.2% | 0.2% | #VALUE! | 100.9% | 101.6% | #NUM! |
| Akko&Kayo_48_26_70 | 0.0% | -1.5% | -1.1% | -0.7% | -0.6% | #VALUE! | 99.0% | 100.5% | #NUM! |
| Breakdancers_4_1_7 | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | #VALUE! | 99.7% | 101.9% | #NUM! |
| average | 0.0% | -1.0% | -0.8% | -0.4% | -0.4% | ###### | 100.2% | 102.1% | #NUM! |

Table 4 Experimental results in MVC (DV constraint flag is 1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | 6.8% | 6.2% | 3.5% | 3.4% | #VALUE! | 100.8% | 101.3% | #NUM! |
| Flamenco2_2_1_3 | 0.0% | 6.6% | 4.7% | 2.2% | 2.2% | #VALUE! | 101.7% | 100.8% | #NUM! |
| Akko&Kayo_48_46_50 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 101.0% | 101.6% | #NUM! |
| Akko&Kayo_48_26_70 | 0.0% | 4.8% | 6.6% | 2.9% | 2.9% | #VALUE! | 101.4% | 99.5% | #NUM! |
| Breakdancers_4_1_7 | 0.0% | 0.3% | 0.2% | 0.2% | 0.2% | #VALUE! | 100.1% | 100.2% | #NUM! |
| average | 0.0% | 3.7% | 3.5% | 1.8% | 1.7% | ###### | 101.0% | 100.7% | #NUM! |

# References

[1] Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “AHG13: Constrained DV for inter-view data access, “JCT3V-C0129, JCT3V 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).