| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0198 |
| --- | --- |

| Title: | AHG13: Test results on AHG recommended condition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports test results of AHG recommended conditions:

1) MVC sequences (Flamenco2, Akko&Kayo, Breakdancers)

2) Artificially shifted CTC sequences

- On each test conditions, vertical range constraint of 56 pixel has been tested.

- It is reported that 56 pixel constraints in vertical range of disparity vector (DV) brings no loss in MVC sequences except for one 2D array setting sequences of Akko & Kayo and also no loss in artificially shifted CTC sequence in 16 and 32 pixel shifted setting. It is also reported that 56 pixel constraints brings significant loss in a 2D array setting MVC sequence and artificially shifted CTC sequences in 64 pixel shifted setting. It is observed that the amount of loss by the DV constraints of 3D-HEVC results is larger than that of MV-HEVC results.

It is recommended to review the results to discuss the feasibility of DV constraints.

# Test materials and conditions

1) MVC sequences (Flamenco2, Akko&Kayo, Breakdancers)

  - Flamenco2

    - View0 (Center), View1 (Left), View3 (Right)

    - View2 (Center), View1 (Left), View3 (Right)

  - Akko&Kayo

    - View48 (Center), View46 (Left), View50 (Right)

    - View48 (Center), View26 (Left), View70 (Right) 2D array setting sequences

  - Breakdancers

    - View4 (Center), View1 (Left), View7 (Right)

anchor:

--CodedDepthMaps=0

--MultiviewMvRegMode=0

test:

--CodedDepthMaps=0

--MultiviewMvRegMode=0

--DisparitySearchRangeRestriction=1

--VerticalDisparitySearchRange=56

2) Artificially shifted CTC sequences

With the attached AHG13 software (HTM51_SHIFT_DEPENDENT_VIEW.patch), artificially shift coding with the following parameters was conducted(for N = 16, 32, 64)

anchor:

--VerticalShiftValueinPixel=N
test:

--VerticalShiftValueinPixel=N

--DisparitySearchRangeRestriction=1

--VerticalDisparitySearchRange=56

With the above configuration, View1 is coded +N pixels shift and View2 is coded with -N pixels shift (View0 is not changed)

# Experimental results

The experiment result on CTC by cross-checker is shown in Table 1 and Table 2

## MV-HEVC condition result

Table 1 CTC with DV range 56 and VerticalShiftValueinPixel = 0 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% | #VALUE! | 0.0% | 96.6% | 99.7% | #NUM! |
| Kendo | 0.0% | 0.1% | -0.1% | 0.0% | #VALUE! | 0.0% | 96.0% | 98.6% | #NUM! |
| Newspapercc | 0.0% | 0.1% | -0.1% | 0.0% | #VALUE! | 0.0% | 94.4% | 98.0% | #NUM! |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 99.9% | 99.3% | #NUM! |
| PoznanHall2 | 0.0% | 0.2% | -0.2% | 0.0% | #VALUE! | 0.0% | 97.5% | 100.8% | #NUM! |
| PoznanStreet | 0.0% | 0.1% | 0.1% | 0.0% | #VALUE! | 0.0% | 97.0% | 99.7% | #NUM! |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | #VALUE! | 0.0% | 97.8% | 101.8% | #NUM! |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 95.7% | 98.7% | #NUM! |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% | #VALUE! | 0.0% | 98.0% | 100.4% | #NUM! |
| average | 0.0% | 0.1% | 0.0% | 0.0% | #VALUE! | 0.0% | 97.0% | 99.7% | #NUM! |

Table 2 CTC with DV range 56 and VerticalShiftValueinPixel = 16 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 96.3% | 100.2% | #NUM! |
| Kendo | 0.0% | -0.1% | -0.1% | -0.1% | #VALUE! | -0.1% | 97.3% | 100.9% | #NUM! |
| Newspapercc | 0.0% | -0.1% | 0.0% | 0.0% | #VALUE! | 0.0% | 94.4% | 98.6% | #NUM! |
| GhostTownFly | 0.0% | -0.2% | 0.1% | 0.0% | #VALUE! | 0.0% | 99.5% | 99.8% | #NUM! |
| PoznanHall2 | 0.0% | 0.3% | 0.1% | 0.1% | #VALUE! | 0.1% | 96.8% | 100.1% | #NUM! |
| PoznanStreet | 0.0% | -0.2% | 0.0% | 0.0% | #VALUE! | 0.0% | 95.5% | 100.3% | #NUM! |
| UndoDancer | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 97.1% | 100.2% | #NUM! |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | #VALUE! | 0.0% | 96.0% | 99.9% | #NUM! |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 97.2% | 100.1% | #NUM! |
| average | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 96.7% | 100.0% | #NUM! |

Table 3 CTC with DV range 56 and VerticalShiftValueinPixel = 32 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.0% | #VALUE! | 0.0% | 96.7% | 100.2% | #NUM! |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 97.0% | 100.4% | #NUM! |
| Newspapercc | 0.0% | 0.1% | -0.1% | 0.0% | #VALUE! | 0.0% | 94.5% | 102.8% | #NUM! |
| GhostTownFly | 0.0% | 0.2% | 0.0% | 0.0% | #VALUE! | 0.0% | 99.9% | 100.5% | #NUM! |
| PoznanHall2 | 0.0% | -0.1% | -0.3% | -0.1% | #VALUE! | -0.1% | 97.2% | 100.3% | #NUM! |
| PoznanStreet | 0.0% | 0.1% | 0.2% | 0.0% | #VALUE! | 0.0% | 95.3% | 100.0% | #NUM! |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | #VALUE! | 0.0% | 97.6% | 100.3% | #NUM! |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 96.0% | 101.1% | #NUM! |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 97.5% | 100.3% | #NUM! |
| average | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 96.9% | 100.6% | #NUM! |

Table 4 CTC with DV range 56 and VerticalShiftValueinPixel = 64 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 22.1% | 95.5% | 26.8% | #VALUE! | 26.6% | 97.1% | 105.0% | #NUM! |
| Kendo | 0.0% | 61.8% | 85.4% | 33.6% | #VALUE! | 33.3% | 97.2% | 104.5% | #NUM! |
| Newspapercc | 0.0% | 28.6% | 121.5% | 32.1% | #VALUE! | 31.8% | 94.7% | 102.6% | #NUM! |
| GhostTownFly | 0.0% | 97.1% | 411.7% | 63.7% | #VALUE! | 63.5% | 98.1% | 117.3% | #NUM! |
| PoznanHall2 | 0.0% | 27.1% | 91.6% | 27.3% | #VALUE! | 26.9% | 97.6% | 104.2% | #NUM! |
| PoznanStreet | 0.0% | 38.4% | 234.9% | 43.9% | #VALUE! | 43.7% | 96.2% | 105.6% | #NUM! |
| UndoDancer | 0.0% | 35.5% | 323.5% | 52.2% | #VALUE! | 52.1% | 99.5% | 107.6% | #NUM! |
| 1024x768 | 0.0% | 37.5% | 100.8% | 30.8% | #VALUE! | 30.5% | 96.4% | 104.0% | #NUM! |
| 1920x1088 | 0.0% | 49.5% | 265.4% | 46.8% | #VALUE! | 46.6% | 97.9% | 108.6% | #NUM! |
| average | 0.0% | 44.4% | 194.9% | 39.9% | #VALUE! | 39.7% | 97.2% | 106.6% | #NUM! |

Table 5 MVC with DV range 56 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | #VALUE! | 94.4% | 96.3% | #NUM! |
| Flamenco2_2_1_3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 95.9% | 100.2% | #NUM! |
| Akko&Kayo_48_46_50 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 95.8% | 100.8% | #NUM! |
| Akko&Kayo_48_26_70 | 0.0% | 18.1% | 20.1% | 10.1% | 10.0% | #VALUE! | 94.2% | 100.5% | #NUM! |
| Breakdancers_4_1_7 | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | #VALUE! | 96.9% | 98.8% | #NUM! |
| average | 0.0% | 3.6% | 4.0% | 2.0% | 2.0% | ###### | 95.4% | 99.3% | #NUM! |

## 3D-HEVC condition result

Table 6 CTC with DV range 56 and VerticalShiftValueinPixel = 0 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% | #VALUE! | 0.0% | 98.4% | 99.2% | #NUM! |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | #VALUE! | -0.1% | 97.0% | 100.8% | #NUM! |
| Newspapercc | 0.0% | 0.0% | 0.1% | 0.0% | #VALUE! | 0.0% | 97.4% | 100.7% | #NUM! |
| GhostTownFly | 0.0% | -0.1% | -0.1% | 0.0% | #VALUE! | 0.0% | 98.2% | 100.3% | #NUM! |
| PoznanHall2 | 0.0% | 0.1% | -0.6% | -0.1% | #VALUE! | -0.1% | 96.8% | 100.6% | #NUM! |
| PoznanStreet | 0.0% | 0.2% | -0.1% | 0.0% | #VALUE! | 0.0% | 98.6% | 101.4% | #NUM! |
| UndoDancer | 0.0% | -0.1% | 0.1% | 0.0% | #VALUE! | 0.0% | 98.4% | 100.4% | #NUM! |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | #VALUE! | 0.0% | 97.6% | 100.3% | #NUM! |
| 1920x1088 | 0.0% | 0.0% | -0.2% | 0.0% | #VALUE! | 0.0% | 98.0% | 100.7% | #NUM! |
| average | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 97.8% | 100.5% | #NUM! |

Table 7 CTC with DV range 56 and VerticalShiftValueinPixel = 16 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 94.2% | 99.4% | #NUM! |
| Kendo | 0.0% | -0.2% | 0.1% | 0.0% | #VALUE! | 0.0% | 94.4% | 99.8% | #NUM! |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 93.4% | 98.7% | #NUM! |
| GhostTownFly | 0.0% | 0.1% | 0.1% | 0.0% | #VALUE! | 0.0% | 93.8% | 100.3% | #NUM! |
| PoznanHall2 | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 95.7% | 99.5% | #NUM! |
| PoznanStreet | 0.0% | -0.2% | -0.4% | -0.1% | #VALUE! | -0.1% | 93.8% | 99.3% | #NUM! |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | #VALUE! | 0.0% | 94.4% | 99.9% | #NUM! |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 0.0% | 94.0% | 99.3% | #NUM! |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 94.4% | 99.7% | #NUM! |
| average | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 94.2% | 99.6% | #NUM! |

Table 8 CTC with DV range 56 and VerticalShiftValueinPixel = 32 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.0% | #VALUE! | 0.0% | 93.8% | 100.6% | #NUM! |
| Kendo | 0.0% | -0.1% | 0.1% | 0.0% | #VALUE! | -0.1% | 93.5% | 101.9% | #NUM! |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | #VALUE! | -0.1% | 93.1% | 99.3% | #NUM! |
| GhostTownFly | 0.0% | -0.1% | -0.2% | -0.1% | #VALUE! | -0.1% | 94.2% | 99.7% | #NUM! |
| PoznanHall2 | 0.0% | 0.0% | -0.2% | 0.0% | #VALUE! | 0.0% | 94.6% | 99.8% | #NUM! |
| PoznanStreet | 0.0% | 0.0% | -0.1% | 0.0% | #VALUE! | 0.0% | 92.9% | 99.2% | #NUM! |
| UndoDancer | 0.0% | -0.2% | -0.1% | 0.0% | #VALUE! | 0.0% | 95.0% | 99.7% | #NUM! |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | #VALUE! | -0.1% | 93.4% | 100.6% | #NUM! |
| 1920x1088 | 0.0% | -0.1% | -0.2% | 0.0% | #VALUE! | 0.0% | 94.2% | 99.6% | #NUM! |
| average | 0.0% | -0.1% | -0.1% | 0.0% | #VALUE! | -0.1% | 93.9% | 100.0% | #NUM! |

Table 9 CTC with DV range 56 and VerticalShiftValueinPixel = 64 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 139.5% | 128.5% | 53.9% | #VALUE! | 45.9% | 89.1% | 100.9% | #NUM! |
| Kendo | 0.0% | 116.0% | 108.8% | 47.4% | #VALUE! | 38.2% | 89.0% | 99.1% | #NUM! |
| Newspapercc | 0.0% | 143.6% | 142.0% | 56.3% | #VALUE! | 45.0% | 89.3% | 103.0% | #NUM! |
| GhostTownFly | 0.0% | 406.7% | 437.8% | 106.4% | #VALUE! | 101.4% | 91.5% | 92.3% | #NUM! |
| PoznanHall2 | 0.0% | 85.6% | 104.0% | 42.2% | #VALUE! | 37.6% | 92.8% | 95.0% | #NUM! |
| PoznanStreet | 0.0% | 224.5% | 248.7% | 76.1% | #VALUE! | 70.5% | 88.0% | 95.2% | #NUM! |
| UndoDancer | 0.0% | 295.5% | 351.2% | 96.1% | #VALUE! | 92.1% | 93.1% | 96.3% | #NUM! |
| 1024x768 | 0.0% | 133.0% | 126.4% | 52.5% | #VALUE! | 43.0% | 89.1% | 101.0% | #NUM! |
| 1920x1088 | 0.0% | 253.1% | 285.4% | 80.2% | #VALUE! | 75.4% | 91.3% | 94.7% | #NUM! |
| average | 0.0% | 201.6% | 217.3% | 68.3% | #VALUE! | 61.5% | 90.4% | 97.3% | #NUM! |

Table 10 MVC with DV range 56 (compared with no DV constraint)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | 95.2% | 101.7% | #NUM! |
| Flamenco2_2_1_3 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | #VALUE! | 96.3% | 101.2% | #NUM! |
| Akko&Kayo_48_46_50 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | #VALUE! | 98.7% | 102.2% | #NUM! |
| Akko&Kayo_48_26_70 | 0.0% | 26.9% | 28.3% | 14.0% | 13.9% | #VALUE! | 91.8% | 98.4% | #NUM! |
| Breakdancers_4_1_7 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | #VALUE! | 96.4% | 97.8% | #NUM! |
| average | 0.0% | 5.3% | 5.6% | 2.8% | 2.8% | ###### | 95.7% | 100.2% | #NUM! |

# Experimental results

It is reported that DV range constraints brings no coding loss in most of cases while significant loss is observed in some 2D array or largely shifted CTC sequences.  It is recommended to review the results to discuss the feasibility of DV constraints.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).