| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0200 |
| --- | --- |

| Title: | Study on view synthesis prediction in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jin Young Lee | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the last meeting, generalized view synthesis prediction (GVSP) mode (JCT3V-B0054) and 4x4 VSP skip mode (JCT3V-B0094) were proposed. Both the methods achieved about 0.5~0.6% bit rate reduction. The GVSP mode allows sub-block level signaling of VSP and removes VSP reference form the reference picture list. The 4x4 VSP skip mode increases the VSP block size from 2x2 to 4x4 and it does not require the reference picture list index as in the GVSP mode. It was discussed that probably the gain for both the methods are mainly coming from not sending the reference picture index. This contribution reports how much gain comes from the sub-block level signaling of VSP in JCT3V-B0054 and the VSP block size increase to 4x4 in JCT3V-B0094. The results demonstrate that benefits of the sub-block level signaling of VSP and 4x4 block size in the VSP skip mode are negligible in terms of the performance.

# Coding Experiments

Tests were based on the common test conditions and done on a clustered system with Linux OS and GCC 4.1.2 compiler. Anchor is ATM v6.1 with disabling macro QC_GVSP_B0054 (Sub-block signaling) and setting BLOCK_VSP_T (VSP block size) to 2. The VSP picture is not removed in the reference picture list. Table 1 is about the performance of the sub-block level signaling in JCT3V-B0054. The gain is very minor, but the encoding complexity increases by 10%. Table 2 is about the performance of 4x4 block size for VSP skip mode in JCT3V-B0094. It shows that the block size increase to 4x4 has no influence on the coding performance. Table 3 is the performance when the sub-block level signaling and 4x4 VSP skip mode are combined.

Table 1. Performance of the sub-block level signaling in JCT3V-B0054

Table 2. Performance of 4x4 block size for VSP skip mode in JCT3V-B0094

Table 3. Performance when the sub-block level signaling and 4x4 VSP skip mode are combined

# Conclusion

The results demonstrate that benefits of the sub-block level signaling of VSP and 4x4 block size in the VSP skip mode are negligible in terms of the performance.