| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0201 |
| --- | --- |

| Title: | Hook on temporal motion vector prediction for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | information |  |  |
| Author(s) or Contact(s): | Yongbing Lin Xiaozhen Zheng, Jianhua Zheng | Tel: Email: | +86-10-60612372 yblin@huawei.com |
| Source: | HiSilicon, Huawei |  |  |

_____________________________

[Note: this proposal is a copy of JCTVC-L0177, which targets to improve coding efficiency for MV-HEVC with modification to HEVC specification.]

# Abstract

MV-HEVC Hook on temporal motion vector derivation is proposed with 2 modifications to HEVC. The first modification to collocated MV selection is made such that the collocated MV has the same reference picture list direction as that of current MV when collocated picture  is a long-term reference picture. The second modification is made to collocated picture selection in a way that the collocated picture has different picture type (i.e. short-term or long-term) as current reference picture. The two modifications provide 1.8% average bitrate saving for 3-view coding under MV-HEVC configuration, and coding gain for single dependent view reaches up to 4.3%.

# Introduction

During the last meeting, temporal motion vector prediction hook for MV-HEVC was proposed in JCTVC-K0239 [1], showing significant gain for MV-HEVC configuration. At the stage of HEVC standard finalization, such a hook with minor text change is desirable.

The picture type (i.e. short-term or long-term) has influence on TMVP (temporal motion vector predictor) derivation. During TMVP derivation process, current block’s TMVP is derived by scaling motion vector of collocated block in a collocated picture, where the current block’s TMVP becomes unavailable when both the reference picture of the current block and the reference picture of the collocated block have different picture type. The unavailable TMVP will affect coding performance for HEVC 3D and Scalability extension.

It is observed that both inter-view and temporal pictures are used as references for dependent view coding in HEVC 3DV extension. The inter-view reference picture is treated as long-term while the temporal reference picture is usually treated as short-term. The inter-view reference picture comes from base view, where all motion vectors in base view usually point to short-term reference pictures. It means that an inter-view reference picture used as collocated picture includes almost all motion vectors pointing to short-term reference picture.

Base on the observation, two modifications to TMVP derivation are proposed as a hook for MV-HEVC.

# Modification 1: reference picture list direction of collocated MV

During TMVP derivation, the collocated block may have two motion vectors for B slice, only one of the collocated MV is used for MV scaling. The existing method in HEVC selects one collocated MV depending on the reference picture list direction of the collocated picture (i.e. collocated_from_l0_flag). It is useful when the collocated picture has different POC value to the current picture.

However, collocated_from_l0_flag doesn’t provide meaningful information when the collocated picture has same POC value as the current picture. In this case, it is proposed that the reference picture list direction for the collocated MV is set equal to the reference picture list direction specified for the current block. In this way, both the current block and its collocated block have same reference picture list direction, which is helpful to improve effectiveness of the derived TMVP of the current block.

The modification was integrated into HTM5.1 reference software [2]. Table 1 summarizes the test results under MV-HEVC configuration. At the mean time, the collocated picture at encoder is forced to an inter-view reference picture (if available), and signalled at slice level by using the existing syntaxes in HEVC (i.e. collocated_from_l0_flag and collocated_ref_idx ). About average bitrate saving of 0.7% is observed for 3-view coding case as specified in 3DV common test conditions [3].

Table 1: Performance of modification 1 with respect to MV-HEVC anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -5.7% | -4.0% | -2.5% | -2.5% | 102.2% | 97.1% |
| Kendo | 0.0% | -3.2% | -2.1% | -1.4% | -1.4% | 101.8% | 89.6% |
| Newspaper_CC | 0.0% | -0.3% | 1.8% | 0.4% | 0.4% | 102.6% | 91.8% |
| GT_Fly | 0.0% | 0.1% | 0.5% | -0.5% | -0.5% | 94.5% | 104.0% |
| Poznan_Hall2 | 0.0% | -3.2% | -0.1% | -0.7% | -0.6% | 100.6% | 97.5% |
| Poznan_Street | 0.0% | 0.1% | 0.7% | 0.1% | 0.1% | 100.1% | 93.8% |
| Undo_Dancer | 0.0% | -0.6% | 0.4% | -0.1% | -0.1% | 102.0% | 101.1% |
| 1024x768 | 0.0% | -3.1% | -1.4% | -1.2% | -1.2% | 102.2% | 92.8% |
| 1920x1088 | 0.0% | -0.9% | 0.4% | -0.3% | -0.3% | 99.2% | 99.0% |
| Average | 0.0% | -1.8% | -0.4% | -0.7% | -0.7% | 100.5% | 96.3% |

# Modification 2: collocated picture selection

A block-level collocated picture selection is proposed to improve coding efficiency for MV-HEVC. Basic idea is that a reference picture within reference picture lists is chosen as additional collocated picture for the current block, such that the additional collocated picture has different picture type as the current picture.

Depending on picture type or POC value of the current picture and current reference picture specified for the current block, the additional collocated picture for the current block is derived as follows.

Case 1)	If the current reference picture has different POC value as the current picture, a reference picture (within current reference picture lists) with same POC value as the current picture will be chosen as the additional collocated picture.

Case 2)	If the current reference picture has same POC value as the current picture, a reference picture with different POC value as the current picture will be chosen as the additional collocated picture.

The above two cases may occur for dependent view coding in MV-HEVC. As a result, the additional collocated picture and the current reference picture come from different view, which also means both the additional collocated picture and the current reference picture have different picture type.

For case 1, the current reference picture comes from the dependent view and usually used as short-term. The additional collocated picture is an inter-view reference picture from base view, where almost all reference pictures for the additional collocated picture are short-term. Thus, reference picture for the current block and reference picture for the collocated block in the additional collocated picture have same picture type, i.e., short-term, which is helpful to reduce the possibility of unavailable TMVP case caused by picture type mismatch during MV scaling process.

For case 2, the current reference picture is an inter-view reference picture coming from base view and used as long-term. The additional collocated picture comes from the dependent view and usually used as a short-term picture. In this case, a collocated picture coming from the base view is not suitable since the desired long-term reference picture for the collocated picture is usually not available from the base view. Therefore, the additional collocated picture coming from the dependent view is exploited instead.

Noted that up to two collocated pictures are allowed only for current slice in dependent view, i.e., one from base view and the other from dependent view. The two collocated pictures can also be determined at slice level.

Table 2 shows the test results for both the modifications under MV-HEVC configuration. It is observed that average bitrate saving for 3-view coding is 1.8% and coding gain for single dependent view reaches up to 4.3%.

Table 2: Performance of modification 1 plus 2 with respect to MV-HEVC anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -7.9% | -6.1% | -3.6% | -3.6% | 100.3% | 98.5% |
| Kendo | 0.0% | -6.0% | -4.7% | -2.7% | -2.7% | 100.0% | 91.0% |
| Newspaper_CC | 0.0% | -2.0% | 0.2% | -0.4% | -0.4% | 99.8% | 93.5% |
| GT_Fly | 0.0% | -4.1% | -4.2% | -2.0% | -2.0% | 92.5% | 103.4% |
| Poznan_Hall2 | 0.0% | -5.2% | -2.7% | -1.9% | -1.8% | 99.7% | 98.4% |
| Poznan_Street | 0.0% | -2.1% | -1.7% | -0.8% | -0.8% | 98.4% | 99.4% |
| Undo_Dancer | 0.0% | -2.7% | -2.1% | -1.0% | -1.0% | 99.9% | 102.4% |
| 1024x768 | 0.0% | -5.3% | -3.5% | -2.2% | -2.2% | 100.0% | 94.3% |
| 1920x1088 | 0.0% | -3.5% | -2.7% | -1.4% | -1.4% | 97.6% | 100.9% |
| Average | 0.0% | -4.3% | -3.1% | -1.8% | -1.8% | 98.6% | 98.0% |

# Proposed specification text

##### 8.5.3.1.7 Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a reference index refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The variable currPb specifies the current luma prediction block at luma location ( xP, yP ).

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If slice_temporal_mvp_enable_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the following ordered steps apply.

- Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

- If both POC values of the colPic and RefPicListX[ refIdxLX ] are equal to POC of the current picture currPic, or both are not equal to POC of the current picture, the following applies to update the collocated picture colPic.

N is defined with value of 1-collocated_from_l0_flag, M is defined with vaule of collocated_from_l0_flag;

isFound=0;

for ( rIdx = 0; rIdx <= num_ref_idx_lN_active_minus1 && !isFound; rIdx++)

if ( DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ])==0 && DiffPicOrderCnt( currPic, colPic )==0

&& DiffPicOrderCnt( currPic, RefPicListN[ rIdx ] ) != 0

|| DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) !=0 && DiffPicOrderCnt( currPic, colPic ) !=0

&& DiffPicOrderCnt( currPic, RefPicListN[ rIdx ] ) == 0 ) {

isFound=1;

colPic = RefPicList0[ rIdx ], collocated_from_l0_flag=M, collocated_ref_idx= rIdx;

}

If slice_type is equal to B, the following applies,

for ( rIdx = 0; rIdx <= num_ref_idx_lM_active_minus1 && !isFound; rIdx++)

if ( DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )==0 && DiffPicOrderCnt( currPic, colPic )==0

&& DiffPicOrderCnt( currPic, RefPicListM[ rIdx ] ) != 0

|| DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) !=0 && DiffPicOrderCnt( currPic, colPic ) !=0

&& DiffPicOrderCnt( currPic, RefPicListM[ rIdx ] ) == 0 ) {

isFound=1;

colPic = RefPicList1[ rIdx ], collocated_from_l0_flag=N, collocated_ref_idx= rIdx;

}

- The bottom right collocated motion vector is derived as follows

…

##### 8.3.1.8 Derivation process for collocated motion vectors

Inputs to this process are

- currPb specifying the current prediction block,

- colPic specifying the collocated picture,

- colPb specifying the collocated prediction block inside the collocated picture specified by colPic,

- a luma location ( xPCol, yPCol ) specifying the top-left sample of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by colPic,

- a reference index refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagLXCol[ x ][ y ], mvLXCol[ x ][ y ] and refIdxLXCol[ x ][ y ] are set equal to the corresponding arrays of the collocated picture specified by colPic, PredFlagLX[ x ][ y ], MvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

  - If predFlagL0Col[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL1Col[ xPCol ][ yPCol ], refIdxL1Col[ xPCol ][ yPCol ], and L1, respectively.

  - Otherwise if predFlagL0Col[ xPCol ][ yPCol ] is equal to 1 and predFlagL1Col[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL0Col[ xPCol ][ yPCol ], refIdxL0Col[ xPCol ][ yPCol ], and L0, respectively.

  - Otherwise (predFlagL0Col[ xPCol ][ yPCol ] is equal to 1 and predFlagL1Col[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If DiffPicOrderCnt( pic, currPic ) is less than or equal to 0 for every picture pic in every reference picture list of the current slice, or the colPic is a long-term reference picture, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xPCol ][ yPCol ], refIdxLXCol[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

      - Otherwise, mvCol, refIdxCol and listCol are set equal to mvLNCol[ xPCol ][ yPCol ], refIdxLNCol[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

and mvLXCol and availableFlagLXCol are derived as follows.

…

# References

[1] Ying Chen, Li Zhang, Vadim Seregin, Marta Karczewicz, “Temporal motion vector prediction hook for MV-HEVC,” JCTVC-K0239, 11th JCT-VC meeting, Shanghai, CN, 10-19 October, 2012.

[2] 3DV-HTM version 5.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.1/.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1100, 2nd JCT-3V meeting, Shanghai, CN, 10-19 October, 2012.

[4] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 9,” JCTVC-K1003_v13, 11th meeting, Shanghai, CN, 10-19 October, 2012

# Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologies may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).