| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0203 |
| --- | --- |

| Title: | CE1.a: Crosscheck on Qualcomm Proposal JCT3V-C0053 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang | Tel: Email: | felixzyc@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck on JCT3V-C0053. The verification task was done successfully. No mismatch is identified. The simulation result reportedly shows the average bitrate reduction of 0.21% for texture coding.

# Introduction

The contribution JCT3V-C0053 proposed to provide a bug-fixed version of Generalized View Synthesis Prediction (GVSP) to reflect the adoption of the previous proposal JCT3V-B0054 [1]. In this crosscheck report, the simulations are conducted with the same software used in [2].

# Crosscheck Results

The simlations are conducted on the common test condition [3]. The provided source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2008, and the experiments were performed on servers with 64-bit Windows 2008 R2.

No mismatch is identified between encoder and decoder. The coding results matched with those provided by the proponents. The obtained results are reported in Error: Reference source not found. Detailed results are available in the attached EXCEL files. Rendering time is not tested.

TABLE 1: Results of crosscheck simulation

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.04 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 | -0.09 | 0.00 | 97.61 | 91.04 |  |
| S02 | -0.18 | 0.01 | 0.00 | 0.00 | -0.17 | 0.01 | -0.15 | 0.00 | 98.54 | 98.89 |  |
| S03 | -0.26 | 0.01 | 0.00 | 0.00 | -0.24 | 0.01 | -0.19 | 0.01 | 95.73 | 92.38 |  |
| S04 | -0.29 | 0.01 | 0.00 | 0.00 | -0.28 | 0.01 | -0.21 | 0.01 | 98.56 | 97.97 |  |
| S05 | -0.28 | 0.01 | 0.00 | 0.00 | -0.26 | 0.01 | -0.15 | 0.01 | 99.89 | 82.15 |  |
| S06 | -0.23 | 0.01 | 0.00 | 0.00 | -0.22 | 0.01 | -0.17 | 0.01 | 97.18 | 102.04 |  |
| S08 | -0.16 | 0.01 | 0.00 | 0.00 | -0.15 | 0.01 | -0.11 | 0.00 | 98.32 | 114.50 |  |
| Average | -0.21 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 | -0.15 | 0.01 | 97.97 | 96.99 |  |

The simulation for anchor is generated by configuring the macro in the provided code as follows:

#define QC_GVSP_B0054                  1

#if QC_GVSP_B0054

#define QC_VSP_REF_REMOVE              0

#if !QC_VSP_REF_REMOVE

#define QC_VSP_SIG_BUGFIX              1

#endif

#endif

The simulation for test is generated by configuring the macro in the provided code as follows:

#if QC_GVSP_B0054

#define QC_VSP_REF_REMOVE              1

#if !QC_VSP_REF_REMOVE

#define QC_VSP_SIG_BUGFIX              1

#endif

#endif

# Conclusions

The results obtained during this crosscheck match those provided by the proponents. Experimental result reportedly shows the average bitrate reduction of 0.21% for texture coding.

# References

- X. Zhao, L. Zhang, Y. Chen, “3D-CE1.a: Generalized view synthesis prediction (GVSP) mode” JCT3V-B0054, October 2012, Shanghai, CN.

- X. Zhao, L. Zhang, Y. Chen, M. Karczewicz, “3D-CE1.a: Results on GVSP”, JCT3V-C0053, January 2013, Geneva, CH.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-B1100, October 2012, Shanghai, CN.