| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rdd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0215 |
| --- | --- |

| Title: | Cross-check of deblocking for adaptive luminance compensation of KHU(JCT3V-C0124) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Donggyu Sim Kwangwoon University 20 Gwangwun-ro, Nowon-gu Seoul, South Korea 139-701 | Tel: Email: | +82-2-940-5470 {youni333, dgsim}@kw.ac.kr |
| Source: | Kwangwoon university (KWU) |  |  |

_____________________________

# Abstract

This contribution provides a cross-check evaluation for the proposal deblocking for adaptive luminance adaptation (ALC) for 3DV-ATM from KHU, JCT3V-C0124. The verification has been successfully done, and we have confirmed that our results completely match those provided by KHU.

# Verification

The deblocking algorithms proposed by KHU are described in the document, JCT3V-C0124[1]. The verification has been done with enabling/disabling macros in source codes provided by KHU. Macros in the provided source codes are like below.

Case 1 – Proposed ALC deblocking without simplification

#define KHU_DEBLOCK             1

#define KHU_DEBLOCK_SIMP   0

Case 2 – Proposed ALC deblocking with simplification

#define KHU_DEBLOCK             1

#define KHU_DEBLOCK_SIMP   1

The algorithms have been simulated on the common test conditions [2]. Moreover, the contribution JCT3V-C0124 shows evaluation results for not only the test sequences in CTC but also MVC test sequences (Akko&Kayo, Ballroom, Breakdancers, Exit, Flamenco2, Rana, Race1, and Uli)[3]. The view coding order of both anchor and proposed methods for the MVC sequences is set as “T0T1T2D0D1D2” due to absences of depth maps. For Flamenco2 (Akko&Kayo) sequence, 1, 0, 3rd (5, 7, and 9th) views are used for three-view inputs. Lastly, all the depth values for those sequences are set to 128 for input depth map.

# Experimental results

RD performance and running time for the presented algorithms can be seen in the excel files titled as EHP_ATM6.0r1_vs_Case1_MVC_KWU.xls, EHP_ATM6.0r1_vs_Case1_3DV_KWU.xls, EHP_ATM6.0r1_vs_Case2_MVC_KWU.xls, and EHP_ATM6.0r1_vs_Case2_3DV_KWU.xls. We have confirmed that all the experimental results match.

# Conclusion

The experimental results match from each other. The cross-check is confirmed.

# Reference

[1] K. Y. Kim, H. S. Hwang, G. H. Park, “MVC deblocking for Adaptive Luminance Compensation,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0124, Jan. 2013, Geneva, CH.

[2] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, Sept. 2012, Shanghai, CN.

[3] Yeping Su, Anthony Vetro, Aljoscha Smolic, “Common Test Conditions for Multiview Video Coding,” Joint Video Team (JVT) of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, JVT-U211, Oct. 2006, Hangzhou, CN.