| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0218 |
| --- | --- |

| Title: | 3D-CE1.h related cross-check results on Adaptive method for Depth Oriented Neighboring Block Disparity Vector (JCT3V-C0112) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Lin Chang +886-3-5670766 ext. 25314 yulin.chang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check result for the proposal of LG on Adaptive method for Depth Oriented Neighboring Block Disparity Vector (DoNBDV) in 3D-HEVC. The test is cross-checked in this contribution. The verification tasks were done successfully, and the results match those provided by LG.

# Verification

LG provided the HTM source codes of their proposed codec described in JCT3V-C0112 [1]. The HTM source code is based on HTM 5.0.1. There are four test cases in JCT3V-C0112. The four test cases use different methods to derive the DoNBDV. They are cross-checked in this contribution. The simlations are conducted on the common test condition Error: Reference source not found.

# Results

The experiment result is shown in Table 1. The anchor is HTM 5.0.1 with common test condition. The BD-rates from our experimental results match those provided by LG. The encoding time/decoding time of the anchor and the tests were measured on the same platform. The detailed results can be seen in JCT3V-C0218_LGE_Method_1.xls, JCT3V-C0218_LGE_Method_2.xls, JCT3V-C0218_LGE_Method_3.xls, and JCT3V-C0218_LGE_Method_4.xls.

Table 1 Experiment results

- Proposed Method 1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.3% | -0.1% | -0.3% | -0.2% | -0.3% | 97.9% | 109.4% | 97.7% |
| Kendo | 0.0% | -1.8% | -1.0% | -0.6% | -0.4% | -0.5% | 98.2% | 97.4% | 96.3% |
| Newspapercc | 0.0% | -0.9% | -0.8% | -0.3% | -0.3% | -0.3% | 100.4% | 96.9% | 97.9% |
| GhostTownFly | 0.0% | -4.0% | -3.8% | -1.1% | -0.9% | -0.9% | 97.5% | 96.9% | 98.4% |
| PoznanHall2 | 0.0% | -1.0% | -0.5% | -0.4% | -0.1% | -0.2% | 95.0% | 97.1% | 96.5% |
| PoznanStreet | 0.0% | -1.8% | -1.8% | -0.6% | -0.5% | -0.5% | 98.9% | 92.4% | 97.1% |
| UndoDancer | 0.0% | -3.3% | -2.7% | -1.1% | -0.7% | -0.8% | 98.4% | 100.7% | 97.9% |
| 1024x768 | 0.0% | -1.3% | -0.6% | -0.4% | -0.3% | -0.3% | 98.8% | 101.1% | 97.3% |
| 1920x1088 | 0.0% | -2.5% | -2.2% | -0.8% | -0.5% | -0.6% | 97.4% | 96.7% | 97.5% |
| average | 0.0% | -2.0% | -1.5% | -0.6% | -0.4% | -0.5% | 98.0% | 98.6% | 97.4% |

- Proposed Method 2

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.1% | -0.3% | -0.3% | -0.2% | -0.3% | 99.6% | 108.6% | 102.5% |
| Kendo | 0.0% | -1.6% | -1.2% | -0.6% | -0.4% | -0.5% | 100.5% | 96.7% | 101.5% |
| Newspapercc | 0.0% | -1.0% | -0.7% | -0.4% | -0.3% | -0.3% | 101.9% | 103.8% | 99.5% |
| GhostTownFly | 0.0% | -4.0% | -3.8% | -1.1% | -0.9% | -1.0% | 98.7% | 100.8% | 99.4% |
| PoznanHall2 | 0.0% | -0.8% | -1.2% | -0.5% | -0.2% | -0.3% | 96.0% | 111.6% | 98.6% |
| PoznanStreet | 0.0% | -1.8% | -1.5% | -0.6% | -0.4% | -0.5% | 100.4% | 96.6% | 97.3% |
| UndoDancer | 0.0% | -2.7% | -1.7% | -0.8% | -0.6% | -0.6% | 99.9% | 111.7% | 99.4% |
| 1024x768 | 0.0% | -1.2% | -0.7% | -0.4% | -0.3% | -0.3% | 100.7% | 102.9% | 101.1% |
| 1920x1088 | 0.0% | -2.3% | -2.1% | -0.8% | -0.5% | -0.6% | 98.7% | 105.0% | 98.7% |
| average | 0.0% | -1.9% | -1.5% | -0.6% | -0.4% | -0.5% | 99.6% | 104.1% | 99.7% |

- Proposed Method 3

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.0% | -0.4% | -0.3% | -0.2% | -0.2% | 98.0% | 108.0% | 100.9% |
| Kendo | 0.0% | -1.6% | -1.3% | -0.6% | -0.5% | -0.5% | 99.5% | 92.6% | 98.9% |
| Newspapercc | 0.0% | -0.9% | -0.9% | -0.4% | -0.3% | -0.4% | 99.7% | 96.9% | 99.9% |
| GhostTownFly | 0.0% | -3.6% | -3.5% | -1.0% | -0.8% | -0.8% | 96.9% | 96.5% | 96.6% |
| PoznanHall2 | 0.0% | -0.8% | -1.1% | -0.4% | -0.1% | -0.2% | 94.0% | 95.0% | 98.8% |
| PoznanStreet | 0.0% | -1.5% | -1.2% | -0.5% | -0.3% | -0.4% | 97.8% | 95.5% | 98.9% |
| UndoDancer | 0.0% | -2.4% | -1.6% | -0.7% | -0.5% | -0.6% | 97.4% | 98.7% | 101.3% |
| 1024x768 | 0.0% | -1.2% | -0.9% | -0.4% | -0.4% | -0.4% | 99.1% | 98.9% | 99.9% |
| 1920x1088 | 0.0% | -2.1% | -1.8% | -0.7% | -0.4% | -0.5% | 96.5% | 96.4% | 98.9% |
| average | 0.0% | -1.7% | -1.4% | -0.6% | -0.4% | -0.4% | 97.6% | 97.5% | 99.3% |

- Proposed Method 4

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.9% | -1.3% | -0.4% | -0.4% | -0.4% | 97.9% | 100.5% | 101.0% |
| Kendo | 0.0% | -1.6% | -1.8% | -0.7% | -0.6% | -0.6% | 98.8% | 92.3% | 98.0% |
| Newspapercc | 0.0% | -0.7% | -0.9% | -0.3% | -0.3% | -0.3% | 99.9% | 98.4% | 98.8% |
| GhostTownFly | 0.0% | -3.3% | -3.5% | -1.0% | -0.8% | -0.8% | 96.9% | 98.0% | 98.0% |
| PoznanHall2 | 0.0% | -0.4% | -1.3% | -0.4% | -0.2% | -0.2% | 93.5% | 100.4% | 98.7% |
| PoznanStreet | 0.0% | -1.6% | -1.6% | -0.6% | -0.4% | -0.5% | 97.8% | 91.3% | 99.6% |
| UndoDancer | 0.0% | -2.2% | -1.5% | -0.7% | -0.5% | -0.6% | 97.7% | 101.5% | 101.1% |
| 1024x768 | 0.0% | -1.1% | -1.3% | -0.5% | -0.4% | -0.4% | 98.9% | 97.0% | 99.2% |
| 1920x1088 | 0.0% | -1.9% | -2.0% | -0.7% | -0.5% | -0.5% | 96.4% | 97.7% | 99.3% |
| average | 0.0% | -1.5% | -1.7% | -0.6% | -0.4% | -0.5% | 97.5% | 97.4% | 99.3% |

# Conclusion

The results presented by LG in JCT3V-C0112 are confirmed.

# References

- J. Jung, J. Sung, S. Yea, “3D-CE1.h related : Adaptive method for Depth Oriented Neighboring Block Disparity Vector”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0112, Jan. 2013, Geneva.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1100, October, 2012.