| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0221 |
| --- | --- |

| Title: | 3D-CE1.a cross-check on simplification of BVSP in 3DV-ATM (JCT3V-C0169) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chi-Ling Wu Yu-Lin Chang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Chi-Ling Wu +886-3-5670766 ext. 24461 chiling.wu@mediatek.com Yu-Lin Chang +886-3-5670766 ext. 25314 yulin.chang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check result for the proposal of Nokia on view synthesis prediction in 3D-AVC. The proposed methods with different block sizes were cross-checked in this contribution. The verification tasks were done successfully, and the results match those provided by Nokia.

# Verification

Nokia provided the ATMv6.1 source codes of their proposed codec described in JCT3V-C0169 [1]. In the contribution two settings of the different block sizes, 4x4 and 8x8, were cross-checked. The verification is carried out by setting the following macros in the provided code:

Test 1 ( block size 4x4):

#define BLOCK_VSP_T				4

Test 2 ( block size 8x8):

#define BLOCK_VSP_T				8

The simlations are conducted on the common test condition Error: Reference source not found.

# Results

The BD-rates from our experimental results match those provided by Nokia. The encoding time/decoding time of the anchor and the tests were measured independently, so they can only show a trend of the computational complexity. The detailed results can be seen in JCT3V-C0221_Test1.xls and JCT3V-C0221_Test2.xls..

Table 1 Experiments results

- Test 1 ( block size 4x4)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.07 | 0.00 | 0.00 | 0.00 | -0.07 | 0.00 | 0.02 | 0.00 | 100.49 | 91.34 | 96.60 |
| S02 | -0.06 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.05 | 0.00 | 103.00 | 90.74 | 106.26 |
| S03 | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.00 | 0.00 | 100.17 | 104.93 | 98.44 |
| S04 | -0.15 | 0.01 | 0.00 | 0.00 | -0.13 | 0.00 | -0.15 | 0.01 | 98.10 | 102.16 | 100.28 |
| S05 | 0.06 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.07 | 0.00 | 104.14 | 106.32 | 101.53 |
| S06 | 0.12 | -0.01 | 0.00 | 0.00 | 0.11 | -0.01 | 0.12 | -0.01 | 96.97 | 83.97 | 99.07 |
| S08 | 0.05 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.07 | 0.00 | 100.44 | 92.72 | 101.54 |
| Average | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.01 | 0.00 | 100.47 | 96.03 | 100.53 |

- Test 2 ( block size 8x8)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.08 | 0.00 | 99.81 | 92.77 | 99.67 |
| S02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.02 | 0.00 | 97.87 | 99.42 | 100.52 |
| S03 | 0.48 | -0.02 | 0.00 | 0.00 | 0.46 | -0.02 | 0.41 | -0.01 | 101.91 | 111.55 | 96.12 |
| S04 | -0.27 | 0.01 | 0.00 | 0.00 | -0.25 | 0.01 | -0.27 | 0.01 | 101.44 | 105.90 | 100.77 |
| S05 | 0.22 | -0.01 | 0.00 | 0.00 | 0.15 | -0.01 | 0.29 | -0.01 | 99.37 | 104.98 | 98.82 |
| S06 | 0.37 | -0.02 | 0.00 | 0.00 | 0.32 | -0.02 | 0.42 | -0.02 | 98.29 | 88.24 | 90.06 |
| S08 | 0.17 | -0.01 | 0.00 | 0.00 | 0.15 | -0.01 | 0.23 | -0.01 | 100.21 | 119.06 | 102.36 |
| Average | 0.14 | -0.01 | 0.00 | 0.00 | 0.12 | -0.01 | 0.17 | -0.01 | 99.84 | 103.13 | 98.33 |

# Conclusion

The results presented by Nokia in JCT3V-C0169 are confirmed.

# References

- Dmytro Rusanovskyy, Miska M. Hannuksela, “CE1.a-related: Simplification of BVSP in 3DV-ATM”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0169, Jan. 2013, Geneva.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1100, Jan., 2013, Geneva