| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0172 |
| --- | --- |

| Title: | CE3.a: Cross-check on unconstrained IVMP proposed by Mediatek (JCT-C0136) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Payman Aflaki, Dmytro Rusanovskyy | Email: | firstname.secondname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document is a crosscheck of JCT3V-C0136 proposal from Mediatek.

Cross checker confirmed coding and view synthesis results presented by proponent of C0136. Cross-checker was not able to perform a reliable time estimates due to properties of experimental environment.

# Introduction

3D-AVC specifies inside view vector prediction. Proponents discuss existing IVMP design in specification, point it drawbacks and propose a solution that improves coding gain for depth by 0.6% of dBR on average, that results in 0.06% of averaged dBR reduction for synthesised views.

# Cross-check results

- Coding and view synthesis results produced with proponent’s software were found to be identical to results reported in C0136.

- All coded bitstreams were found to be decodable and no mismatch was observed between reconstructed and decoded YUV files.

- Cross-checker was not able to produce reliable encoding and decoding time estimates due to existing cluster architecture.  However, introduced method seems do not have significant computation complexity that could be effectively observed at cluster simmulations.

# Experimental results

Table 1. Cross-checker’s results on C0136 proposal.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -0.03 | 0.00 | 0.08 | 0.00 | -0.05 | 0.00 | -0.02 | 0.00 |
| S02 | 0.00 | 0.00 | -0.37 | 0.02 | -0.02 | 0.00 | -0.01 | 0.00 |
| S03 | -0.01 | 0.00 | -1.21 | 0.09 | -0.08 | 0.00 | -0.16 | 0.01 |
| S04 | 0.01 | 0.00 | -2.03 | 0.12 | -0.07 | 0.00 | -0.04 | 0.00 |
| S05 | 0.00 | 0.00 | -0.21 | 0.01 | -0.03 | 0.00 | -0.11 | 0.00 |
| S06 | 0.02 | 0.00 | -0.41 | 0.02 | -0.01 | 0.00 | -0.03 | 0.00 |
| S08 | 0.00 | 0.00 | -0.30 | 0.01 | -0.03 | 0.00 | -0.01 | 0.00 |
| Average | 0.00 | 0.00 | -0.64 | 0.04 | -0.04 | 0.00 | -0.06 | 0.00 |

# Reference

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”