| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0228 |
| --- | --- |

| Title: | 3D-CE5.a related: Cross-check on direct MVP derivation with reduced complexity proposed by Mediatek (JCT-C0140) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Payman Aflaki, Dmytro Rusanovskyy | Email: | firstname.secondname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document is a crosscheck of JCT3V-C0140 proposal from Mediatek.

Cross checker confirmed coding and view synthesis results presented by proponent of C0140. Cross-checker was not able to perform a reliable time estimates due to properties of experimental environment.

# Introduction

Proposal C0140 introduces a harmonization of reference index settings between Spatial Skip and Spatial Direct mode. Introduced change bring penalty of 0.03% of dBR increase for coded texture view, with maximal penalty for Kendo (0.13% of dBR). Marginal execution time reduction (below 1%) is reported, however this value lay beyond of reliable time estimates.

# Cross-check results

- Coding and view synthesis results produced with proponent’s software were found to be identical to results reported in C0140.

- All coded bitstreams were found to be decodable and no mismatch was observed between reconstructed and decoded YUV files.

- Cross-checker was not able to produce reliable encoding and decoding time estimates due to existing cluster architecture. However, introduced method seems do not have significant computation complexity that could be effectively observed at cluster simulations.

# Experimental results

Table 1. Cross-checker’s results on C0140 proposal.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | 0.03 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.04 | 0.00 |
| S02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.02 | 0.00 |
| S03 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.03 | 0.00 |
| S04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 |
| S05 | 0.13 | -0.01 | 0.00 | 0.00 | 0.11 | -0.01 | 0.10 | 0.00 |
| S06 | 0.03 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.02 | 0.00 |
| S08 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.01 | 0.00 |
| Average | 0.03 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.03 | 0.00 |

# Reference

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”