| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0229 |
| --- | --- |

| Title: | 3D-CE1.h related: Cross-check on depth-oriented disparity vector predictor (JCT-C0132) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Srikanth Gopalakrishna, Dmytro Rusanovskyy | Email: | firstname.secondname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document is a crosscheck of JCT3V-C0132 proposal from Mediatek. Current document reports results of Nokia cross-check on usage of FCO-based disparity derivation for AMVP in 3D-HTM.

Cross checker confirmed coding and view synthesis results presented by proponent of C0132. Cross-checker was not able to perform a reliable time estimates due to properties of experimental environment.

# Introduction

Proposal C0132 studies disparity from depth derivation method for 3D-HTM. This approach allows a very low complexity disparity derivation if depth associated with current texture view is available in advance. Reported results show that utilization of high accuracy disparity information in AMVP improves coding of dependent texture views by 2% of dBR on average, which results in 0.7% of dVBR for texture views on average.

# Cross-check results

- Coding and view synthesis results produced with proponent’s software were found to be identical to results reported in C0132.

- All coded bitstreams were found to be decodable and no mismatch was observed between reconstructed and decoded YUV files.

- Cross-checker was not able to produce reliable encoding and decoding time estimates due to existing cluster architecture. However, introduced method obviously feature lower computation complexity than the anchor method.

# Experimental results

Table 1. Cross-checker’s results on C0132 proposal

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.0% | -2.1% | -0.6% | 0.4% | 0.8% |
| Kendo | 0.0% | -1.8% | -2.5% | -0.9% | 1.9% | 2.2% |
| Newspapercc | 0.0% | -1.0% | -1.2% | -0.4% | 0.0% | 0.8% |
| GhostTownFly | 0.0% | -4.0% | -4.2% | -1.2% | 1.2% | 1.2% |
| PoznanHall2 | 0.0% | -0.3% | -2.2% | -0.6% | 2.2% | 2.3% |
| PoznanStreet | 0.0% | -1.3% | -1.6% | -0.5% | 1.3% | 1.4% |
| UndoDancer | 0.0% | -2.6% | -2.5% | -0.9% | 1.4% | 1.3% |
| 1024x768 | 0.0% | -1.3% | -1.9% | -0.7% | 0.8% | 1.3% |
| 1920x1088 | 0.0% | -2.0% | -2.6% | -0.8% | 1.5% | 1.5% |
| average | 0.0% | -1.7% | -2.3% | -0.7% | 1.2% | 1.4% |

# Reference

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”