| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0230 |
| --- | --- |

| Title: | 3D-CE6.h related: Calculation Sample Reduction for Depth Modeling Mode 1 and 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Masaharu Sato Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution proposes further simplification of Depth Modeling Mode (DMM) 1 and 3. The proposed method skips the calculation of the non-edge region based on the Wedgelet pattern. Under the common test conditions, the proposed method reduces the decoder/render runtime by 1.0%/4.4% with no impact on the coding efficiency.

# Introduction

The additional depth intra coding modes in 3DV-HTM, referred to as DMM, are adapted to the specific characteristics of depth map by applying non-rectangular block partitions for approximating the signal of a depth block containing an edge.　Mode 1 and 3 use a minimum-distortion Wedgelet search for finding the best matching partition for a block. For this purpose a Wedgelet pattern look-up list is provided that contains possible Wedglet partition patters for a certain block size. By searching the limited list of patterns the minimum distortion Wedgelet partition is definitely found, but such a search requires a substantial amount of processing. In the previous meeting, the number of patterns was dramatically reduced [1]. In this contribution we propose the further simplification for reducing the number of calculation points.

# Proposed method

DMM 1 and 3 generate the Wedgelet pattern lists during encoder and decoder initialization. Both encoder and decoder calculate the average luma values and sum of absolute difference values for each Wedgelet partition. Wedgelet resolutions are half-pel for 4x4 and 8x8 blocks, full-pel for 16x16 blocks, and double-pel for 32x32 blocks. Consequently, the number of calculation samples becomes 8x8 points or 16x16 points.

The proposed method basically skips the calculation for the non-edge region based on the Wedgelet pattern. Collocated texture luma block (CTLB) is partitioned into four square regions. Based on the start/end position, five patterns of calculation regions such as all, top, bottom, left, and right are defined at the generation process of Wedgelet pattern lists. In Figure 1, calculation regions are shown as colored regions for pattern “left” and pattern “all.”

Figure 1 Calculation regions by Wedgelet separation line start/end position.

It is noted that the proposed method is conforming to SIMD implementation because the minimum number of calculation samples becomes 4x4 points or 8x8 points. In the following simulation, SIMD implementation is not used in order to keep the fair comparison on the encoder/decoder runtimes.

# Experimental results

The proposed method is implemented to 3DV-HTM-5.0.1. A test condition follows the common test conditions for 3D-HEVC [2].

Table 1 Result for 3-view scenario under common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 98.9% | 91.4% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 99.3% | 93.9% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 98.2% | 93.6% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 101.3% | 99.1% | 99.0% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 100.4% | 99.6% | 97.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.9% | 99.4% | 96.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.9% | 98.9% | 98.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 98.8% | 93.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.9% | 99.2% | 97.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 100.65% | 99.05% | 95.65% |

# Conclusion

This contribution proposed the simplification scheme to restrict calculation points for depth modeling mode (DMM) 1 and 3. The experimental result showed that the decoder / render runtime is reduced by 1.0% / 4.4% under the common test conditions. The BD-rate increase is negligible, being less than 0.01%.

# References

- Philipp Merkle, Karsten Müller, Xin Zhao, Ying Chen, Li Zhang, Marta Karczewicz, “CE6.H results on simplified Wedgelet search for DMM modes 1 and 3,” JCT3V-B0039, Shanghai, CN, 13–19 Oct. 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1100, Shanghai, CN, 13–19 Oct. 2012.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).