| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0232 |
| --- | --- |

| Title: | 3D-CE1.a and CE2.a related: Crosscheck results on synthesized disparity vectors for BVSP and DMVP on ATM6.0 (Table 1 & 2 in JCT3V-C0114) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li Ping Wu No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

Synthesized disparity vectors for BVSP and DMVP was proposed in JCT3V-C0114. In this document, the crosscheck results of this approach on ATM6.0, i.e. the results in Table 1 and 2 in JCT3V-C0114, are presented, and no mismatches are identified. The crosscheckers also reviewed and studied the source codes from the proponents, and verified that the proposed approach was well implemented on ATM6.0.

# Introduction

Synthesized disparity vectors for BVSP and DMVP was proposed in JCT3V-C0114 [1]. The proposed method can be used in both depth first coding order (DFCO) (CTC [2]) and texture first coding order (TFCO) (non-CTC) configurations of ATM6.0 and ATM6.1 to derive the disparity vector for BVSP and DMVP. In this document, the implementation of the proposed approach on ATM6.0 was crosschecked. Simulations were done on TFCO and DFCO configurations, and the results match the ones presented in Table 1 and Table 2 in JCT3V-C0114, respectively.

The source codes were reviewed and studied. The proposed approach was well implemented on ATM6.0.

# Crosscheck results

The source code was compiled using Microsoft Visual Studio 2008 on a server with 64-bit Window Server 2003 system. The crosscheck simulations were carried out on servers with the same OS. There are some relatively large fluctuations in the values of the encoding and decoding time.

DFCO (CTC) and TFCO (non-CTC) configurations were employed in the simulations. The non-CTC configuration was obtained by only changing the 3DVCodingOrder as “T0D0T1D1T2D2”.

The summaries are presented in Table 1 (TFCO) and Table 2 (DFCO). Detailed results are available in the attached excel documents. The anchor employed for comparisons in both tables was ATM6.0 with CTC. No mismatches were identified.

Table 1. ATM 6.0 with BVSP and DMVP using proposed DV derivation in TFCO vs. ATM 6.0 CTC anchor.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.03 | 0.00 | -2.52 | 0.17 | -0.12 | 0.00 | -0.56 | 0.02 | 91.46 | 100.38 |  |
| S02 | -0.07 | 0.00 | -3.24 | 0.15 | -0.44 | 0.01 | -0.37 | 0.01 | 99.82 | 107.00 |  |
| S03 | -0.37 | 0.01 | -1.38 | 0.11 | -0.35 | 0.01 | -1.06 | 0.03 | 101.80 | 97.33 |  |
| S04 | -0.04 | 0.00 | -6.25 | 0.36 | -0.26 | 0.01 | -0.93 | 0.03 | 92.50 | 92.12 |  |
| S05 | 1.54 | -0.07 | -1.99 | 0.09 | 0.96 | -0.04 | 0.72 | -0.03 | 99.07 | 117.69 |  |
| S06 | 1.22 | -0.06 | -6.35 | 0.30 | 0.40 | -0.02 | 0.51 | -0.02 | 104.62 | 118.17 |  |
| S08 | 0.28 | -0.01 | -2.62 | 0.11 | -0.37 | 0.02 | -0.39 | 0.01 | 111.47 | 111.57 |  |
| Average | 0.37 | -0.02 | -3.48 | 0.19 | -0.03 | 0.00 | -0.30 | 0.01 | 100.10 | 106.32 | #DIV/0! |

Table 2. ATM 6.0 with BVSP and DMVP using proposed DV derivation in DFCO vs. ATM 6.0 CTC anchor

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.31 | 0.01 | 0.00 | 0.00 | -0.28 | 0.01 | -0.08 | 0.00 | 101.51 | 98.44 |  |
| S02 | -0.26 | 0.01 | 0.00 | 0.00 | -0.22 | 0.01 | 0.00 | 0.00 | 108.39 | 100.05 |  |
| S03 | -0.46 | 0.02 | 0.00 | 0.00 | -0.42 | 0.01 | -0.16 | 0.01 | 118.75 | 94.66 |  |
| S04 | -0.14 | 0.00 | 0.00 | 0.00 | -0.13 | 0.00 | 0.05 | 0.00 | 88.32 | 91.76 |  |
| S05 | 1.24 | -0.06 | 0.00 | 0.00 | 1.12 | -0.05 | 1.14 | -0.05 | 107.29 | 124.39 |  |
| S06 | 0.91 | -0.04 | 0.00 | 0.00 | 0.78 | -0.04 | 1.05 | -0.05 | 102.91 | 129.51 |  |
| S08 | -0.04 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 | 0.08 | 0.00 | 114.76 | 91.54 |  |
| Average | 0.14 | -0.01 | 0.00 | 0.00 | 0.11 | -0.01 | 0.30 | -0.01 | 105.99 | 104.34 | #DIV/0! |

# Conclusions

The obtained results matches the ones presented in Table 1 and 2 in JCT3V-C0114 under TFCO and DFCO configurations for ATM6.0. Additionally, the source code was reviewed and identified as well implementation.

# References

- Y. Zhao, Y. Zhang, L. Yu. 3D-CE1.a and CE2.a related: synthesized disparity vectors for BVSP and DMVP, JCT-3V document JCT3V-C0114, January 2013, Geneva, Switzerland.

- D. Rusanovskyy, K. Müller, A. Vetro. Common Test Conditions of 3DV Core Experiments, JCT-3V document JCT3V-B1100, October 2012, Shanghai, China.