| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rdd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0233 |
| --- | --- |

| Title: | Cross-check of Results on Weighted Prediction for 3D-HEVC (JCT3V-C0223) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Donggyu Sim Kwangwoon University 20 Gwangwun-ro, Nowon-gu Seoul, South Korea 139-701 | Tel: Email: | +82-2-940-5470 {youni333, dgsim}@kw.ac.kr |
| Source: | Kwangwoon university (KWU) |  |  |

_____________________________

# Abstract

This contribution provides a cross-check evaluation for the proposal, results on weighted prediction for 3D-HEVC (JCT3V-C0223). The verification has been successfully done, and we have confirmed that our results completely match those provided by LG Electronics.

# Verification

The weighted prediction algorithm proposed by LG Electronics, inheriting weighting factors from different views, is described in the document, JCT3V-C0223[1]. The verification has been done with enabling/disabling macros in the provided source codes, and enabling configuration options for the weighted prediction. The encoding configuration options are like below.

weighted_pred_flag : 1

weighted_bipred_idc : 1

Also, the proposed algorithm is configurable using a macro in the provided source codes like below :

#define WP_INHERITANCE_FOR_MV           0 or 1

Note that the proposed algorithm prohibits the interview weighted prediction which calculates weighting factors using interview reference pictures. For this, one more macro is provided like :

#define DISABLE_INTERVIEW_WP               1

Moreover, the test sequences for JCT3V-C0223 have been generated and exploited to evaluate the weighted prediction. They fade in and out in 2 seconds, in the same manner of the evaluation of weighted prediction for HEVC[2]. A linear fade-out has been applied for one second, then the linear fade-in has been applied for the other one second. The fading strength is in the range of [0.25, 1]. Except for these conditions all evaluations have been performed under the common test conditions[3].

# Experimental results

Table 1 WP without parameter inheritance vs. HTM5.0.1 on fading 3DV sequences

|  | video 0 | video 1 | video 2 | video only | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -36.0% | -19.5% | -18.3% | -29.7% | 133.0% | 100.7% |
| Kendo | -18.6% | -4.6% | -2.5% | -13.5% | 163.7% | 102.8% |
| Newspapercc | -33.0% | -21.9% | -23.5% | -29.2% | 119.2% | 94.1% |
| GhostTownFly | -29.6% | -2.0% | -2.8% | -24.6% | 146.3% | 106.0% |
| PoznanHall2 | -32.1% | -9.7% | -15.9% | -24.8% | 147.2% | 100.0% |
| PoznanStreet | -27.5% | -16.3% | -14.5% | -24.6% | 122.1% | 104.5% |
| UndoDancer | -37.2% | -19.0% | -23.7% | -33.3% | 138.4% | 102.7% |
| 1024x768 | -29.2% | -15.3% | -14.8% | -24.1% | 137.4% | 99.2% |
| 1920x1088 | -31.6% | -11.8% | -14.2% | -26.8% | 138.1% | 103.3% |
| average | -30.6% | -13.3% | -14.5% | -25.7% | 137.8% | 101.5% |

Table 2 WP with parameter inheritance vs. HTM5.0.1 on fading 3DV sequences

|  | video 0 | video 1 | video 2 | video only | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -36.0% | -26.6% | -25.5% | -32.1% | 135.6% | 102.1% |
| Kendo | -18.6% | -12.9% | -11.3% | -16.4% | 164.7% | 103.2% |
| Newspapercc | -33.0% | -27.9% | -29.2% | -31.3% | 122.8% | 95.2% |
| GhostTownFly | -29.6% | -11.7% | -10.9% | -26.1% | 146.8% | 106.6% |
| PoznanHall2 | -32.1% | -17.2% | -21.7% | -27.1% | 147.3% | 100.6% |
| PoznanStreet | -27.5% | -20.0% | -19.4% | -25.6% | 122.3% | 104.7% |
| UndoDancer | -37.2% | -23.0% | -26.4% | -33.9% | 139.5% | 104.3% |
| 1024x768 | -29.2% | -22.5% | -22.0% | -26.6% | 140.0% | 100.1% |
| 1920x1088 | -31.6% | -18.0% | -19.6% | -28.1% | 138.6% | 104.0% |
| average | -30.6% | -19.9% | -20.6% | -27.5% | 139.2% | 102.3% |

Table 3 WP parameter inheritance on vs. WP parameter inheritance off

|  | video 0 | video 1 | video 2 | video only | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -8.7% | -8.8% | -3.4% | 102.0% | 101.4% |
| Kendo | 0.0% | -8.6% | -8.9% | -3.3% | 100.6% | 100.3% |
| Newspapercc | 0.0% | -7.5% | -7.3% | -2.8% | 103.0% | 101.1% |
| GhostTownFly | 0.0% | -9.7% | -8.2% | -1.9% | 100.3% | 100.6% |
| PoznanHall2 | 0.0% | -8.1% | -6.8% | -2.9% | 100.0% | 100.6% |
| PoznanStreet | 0.0% | -4.2% | -5.5% | -1.3% | 100.1% | 100.2% |
| UndoDancer | 0.0% | -4.6% | -3.2% | -0.8% | 100.7% | 101.5% |
| 1024x768 | 0.0% | -8.3% | -8.4% | -3.2% | 101.8% | 101.0% |
| 1920x1088 | 0.0% | -6.7% | -5.9% | -1.7% | 100.3% | 100.7% |
| average | 0.0% | -7.4% | -7.0% | -2.3% | 101.0% | 100.8% |

# Conclusion

The RD performance match completely from each other, and the complexity seems reasonable – the encoding complexity results from KWU is slightly lower than ones from LG Electronics, but they are in error range. The cross-check is confirmed.

# Reference

[1] J. Jung, H. Liu, “Results on Weighted Prediction for 3D-HEVC,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0225, Jan. 2013, Geneva, CH.

[2] P. Bordes, “Weighted prediction,” Joint Collaborative Team on Video Coding of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCTVC-E041, Mar. 2011, Geneva, CH.

[3] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, Sept. 2012, Shanghai, CN.