| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-Cxxxx |
| --- | --- |

| Title: | 360 degree viewable 3D Display “Holo-Table” |  |  |
| --- | --- | --- | --- |
| Status: |  |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hideyoshi Horimai #7 Incubation office, Nagoya Institute of Technology, Gokiso, Nagoya, Aichi 466-8555, Japan | Tel: Email: | +81-80-3490-3077 horimai@3dragons.jp |
| Source: | 3Dragons LLC |  |  |

_____________________________

# Abstract

Three-Dimensional (3D) Display System by using the Direct Light Scanning Method, named Holo-Table, had been developed and demonstrated. Since the holographic screen as a beam deflector sets horizontally, 3D image has a 360 degree full horizontal viewing angle. R-G-B parallax images are time-sequentially projected by only one Projector. The holographic screen rotation and the switching of projected images were synchronized by measuring the disc rotating state using a photo interrupter.

Typical specification of the Holo-Table is summarized as the following: holographic screen size is 120~300[mm], horizontal viewing angle ranges are 360 deg., ray angle pitch ~1.0 deg., number of directional images are 360 per rotation, each parallax image size is 1024x768 pixel (XGA) and resolution is 1bit per pixel. One static 3D image size (RGB color) is 1024x768x360x3(RGB) =850Mbits , movie frame rate is 60 fps., frame rate of projector is 22,000 fps, 3D image display data rate reaches 1024x768x360x60 =17Gbits per second.

The displayed image is observed by naked-eye, and sufficient motion parallax and binocular parallax was confirmed. To display real time 3D movie images, further investigations are necessary.

# Introduction

Three-Dimensional (3D) display is in great demand in many kinds of field. Development of 3D display without special glasses and smooth motion parallax is widely desired. High density directional images displaying is one of a promising glasses-free 3D display method [1][2]. This method has the following features;

Fig. 1 360 deg. viewable 3D display system, “Holo-Table”.

(a) Special glasses are not required. 
(b) The eye accommodation function may work.
(c) The smooth motion parallax is obtained.
(d) The observation position is not restricted.

In this method, however, considerably large number of spatial light modulators (SLMs) corresponding to the number of spatial beam directions are needed.

In contrast to the method mentioned above, Direct Light Scanning Method had been proposed by authors and demonstrated the high-density directional 3D motion image can easily be obtained with only one SLM[3][4][5].

In this paper, new concept of 3D Display System, so called Holo-Table as Figure 1, was introduced. By utilizing newly developed RGB display engine, full-color 3D image with 360 degree full horizontal viewing angle was observed.

Principle

Fig. 2 Depict of high density directional images composed of parallel rays.

In high density directional images displaying for 3D display, directional images are 2D graphics which is orthographic projection of 3D objects. As shown in Figure 2, this method requires projecting these directional images to corresponding directions by parallel rays.

When the ray angle pitch becomes small enough, the rays from display reconstruct the same rays from 3D objects. Therefore, this method allows us to observe reconstructed 3D images without having any special glasses. Thus displayed 3D image has then smooth motion parallax, and the viewing position is not restricted.

In this study, we used only one parallel beam with single projector to display high density directional images, where the direction of beam was scanned with a horizontally placed holographic screen by synchronously changing the projection image corresponding to the rotation angle of holographic screen.

Fig. 3 Beam scanning by rotation of hologram screen.

To change the direction of a beam, a simple hologram was set horizontally and it will rotate in plane. The incident beam was diffracted and then beam angle was changed due to bragg-matching of hologram. By rotating the hologram, the incident angle of reference beam is relatively changed the direction (Figure 3). This holographic medium acts as a screen, and hence let us refer to as a holographic screen hereafter. The amount of this change of diffraction angle may define by corresponds to vertical viewing direction. According to the holographic screen rotation angle horizontal viewing angle was changed in this 3D display system.

Set up of 3D display system

Fig. 4 Holo-Table (φ300mm)

Based upon the principle described in 2, the 3D display system was designed and constructed as Figure 4. The holographic screen was horizontally held at the top of the system by a rotation holder. The parallax images were projected normal to the holographic screen from bottom side. The optical systems were constructed as a telecentric system.

Typical specification of the Holo-Table is summarized as the following: holographic screen size is 300 mm, horizontal viewing angle ranges are 360 deg., ray angle pitch ~1.0 deg., number of directional images are 360 per rotation, each parallax image size is 1024x768 pixels (XGA) and resolution is 1bit per pixel. One static 3D image size (RGB color) is 1024x768x360x3(RGB) =850Mbits , movie frame rate is 60 fps., frame rate of projector is 22,000 fps, 3D image display data rate reaches 1024x768x360x60 =17Gbits per second. The holographic screen rotation and the switching of projected images were synchronized by measuring the disc rotating state using a photo interrupter.

The newly developed RGB display engine is shown as Figure 5. The illuminating beam (horizontal parallax images) was modulated by a Digital Micro-mirror Device (DMD). In the current system, coherent light source such as laser light was not required as a projecting illuminant, so R/G/B LED were employed as the illuminant light source.

Fig. 5 Developed RGB display engine.

# Observation of 3D display images

Figure 6 shows another Holo-Table, portable demo system with holographic screen size is 120 mm, and an example of observed 3D images. These observed 3D images were taken by a digital camera. The displayed image can be observed by naked-eye, and sufficient motion parallax and binocular parallax (the requirement 3D image seeing) was confirmed. Due to the high density directional images displaying technology, the reflection from the car body was really reconstructed.

The displayed image was made from computer graphics. Actual object images taken by photographed pictures also have demonstrated to display.

Fig. 6 Portable Holo-Table demo system (φ120mm), an example of displayed 3D images.

# Conclusion

A brand-new 3D display with high-density directional display system composed of the horizontal holographic screen and single projector had been constructed and its performance was demonstrated on site. The results revealed that this system configuration enable to display high-resolution full-color 3D images with 360 degree full horizontal viewing angle. The displayed image is observed by naked-eye, and sufficient motion parallax and binocular parallax was confirmed. R-G-B parallax images are time-sequentially projected by only one Spatial Light Modulator (SLM). Particular feature of this system is the use of single SLM, being ensuring the simple system configurations.

To display much realistic 3D movie images in real time, further investigations will be necessary.  Especially 3D movie contents coding and data transfer algorism etc. development are key to success.

# References

[1] Y. Takaki “A Novel 3D Display Using an Array of LCD Panels,” IS&T-SPIE 5003, 1-7 (Santa Clara, U.S., 2003)

[2] Y. Takaki “Natural 3D display which generates high-density directional images and human accommodation responses,” IDW’03: proceedings of the 10th International Display Workshops, 1405-1408 (Fukuoka, Japan, 2003)

[3] K. Kanemoto et al., “Construction and Properties of Projection-Type Three-Dimensional Display with Holographic Screens,” IWHM 2007, 26P22 (Penang, Malaysia, 2007)

[4] T. Kitada et al., “Three-dimensional Display with Rotation-type Holographic Screen,” IWHM 2008, 22PD-2 (Aichi, Japan, 2008)

[5] Y. Aoki et al., “Direct Light Scanning 3-D Display,” ACP 2009, WL113 (Shanghai, China, 2009)

# Patent rights declaration(s)

3Dragons LLC does not have any current or pending patent rights relating to the technology described in this contribution.