| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17-23 Jan. 2013 |  |  |  | Document: JCTVC-L0441r2 JCT3V-C0235r2 |
| --- | --- | --- | --- | --- |
| Title: | Joint BoG report on extension high-level syntax |  |  |  |
| Status: | Report Document from Chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Jill Boyce Ying Chen |  | Email: jill@vidyo.com cheny@qti.qualcomm.com |  |
| Source: | BoG |  |  |  |

# Abstract

This contribution provides a BoG report from the joint JCTVC/JCT3V BoG on extension high-level syntax.  The contributions have been grouped into the following categories

- RAP pictures and POC

- VPS extension signaling of dimensions

- Layer dependency signaling

- VPS extension profile/tier/level and operation point signaling

- VPS extension – what to signal

- Parameter set properties

- Non-parameter set

The initial version contains notes from the first meeting on the morning on 18 January.

The r1 version contains notes from the sessions on 19 January.

The r2 version contains notes from the session on 20 January.

One document remains to be presented, JCTVC-L0178, which is requested by the author (J. Boyce) to be reviewed by in the JCTVC track rather than the BoG because of the need for a different chair, and because it impacts both high-level syntax and decoding process.

The following have been recommended for adoption into the combined high-level syntax design, to be included in working drafts for both SHVC and MV-HEVC.

- JCTVC-L0039/JCTVC-C0165: several RAP picture related aspects

  - On EL CRA pictures:

    - CRA NAL unit type can be used when nuh_layer_id is greater than 0.

    - Inter-layer prediction is allowed for CRA NAL units with nuh_layer_id greater than 0, while inter prediction is disallowed

    - CRA NAL units need not be aligned across layers. In other words, a CRA NAL unit type can be used for all VCL NAL units with a particular value of nuh_layer_id while another NAL unit type can be used for all VCL NAL units with another particular value of nuh_layer_id in the same access unit.

  - On IDR and BLA pictures:

    - IDR pictures may have nuh_layer_id greater than 0 and they may be inter-layer predicted while inter prediction is disallowed.

    - IDR pictures shall be present in an access unit either in no layers or in all layers, i.e. an IDR nal_unit_type indicates a complete IDR access unit where decoding of all layers can be started.

- JCT3V-C0081/JCT3V-C0084: POC for all HEVC layers in an access unit shall be the same

- JCT3V-C0085: Specific editorial improvement as part of same adoption reflected in an aspect of JCTVC-L0039

- JCTVC-L0263: Editorial bug fix to ensure that coded picture in a layer can only reference pictures in a lower layer

- JCTVC-L0200: Add a splitting_flag to the VPS extension, which imposes a constraint that bit mapping of layer_id is supported, but otherwise doesn’t change existing syntax and semantics

- JCTVC-L0180: Profile tier level signaling per operation point, and optionally referencing the profile and tier from an earlier operation point while sending level

- JCTVC-L0446: Layer dependency signaling using mask approach

- JCTVC-L0188: Several aspects relating to combination of SHVC and MV-HEVC

  - Activation process for picture and sequence parameter sets for individual layers

  - Non-reference pictures at the highest decoded temporal sub-layer are marked as “unused for reference” immediately after their decoding to enable reduction of the DPB usage.

  - Change MV-HEVC’s view dependency change SEI to generic layer dependency SEI message, and include in combined text

The following contributions are suggested to be revisited by the JCTVC track so that additional experts are available:

- JCTVC-L0226: Aspect relating to byte alignment at end of VPS or beginning of VPS extension

- JCTVC-L0262: Signaling of required DPB size in VPS

- JCTVC-L0188: BoG recommends that the track revisit for consideration in version 1 the aspect related to marking of non-reference pictures at highest temporal sub-layer

The following contribution is suggested to be revisited by the JCT3V track so that additional experts are available:

- JCT3V-C0059: Target output views for MV-HEVC

- Meeting notes

## RAP pictures and POC

JCTVC-L0039 On SHVC RAP pictures [M. M. Hannuksela (Nokia)]

This contribution contains the following proposals related to 1) enhancement layer CRA pictures, 2) BLA and IDR pictures, and 3) cross-layer impacts of RAP pictures:

- On EL CRA pictures:

  - CRA NAL unit type can be used when nuh_layer_id is greater than 0.

Agreed.

  - Inter-layer prediction is allowed for CRA NAL units with nuh_layer_id greater than 0, while inter prediction is disallowed.

Noted that the HTM software already does this.  Agreed.

  - CRA NAL units need not be aligned across layers. In other words, a CRA NAL unit type can be used for all VCL NAL units with a particular value of nuh_layer_id while another NAL unit type can be used for all VCL NAL units with another particular value of nuh_layer_id in the same access unit.

Noted that SVC allows similar flexibility.  Allows different RAP period per layer.  Still allows same POC value for all pictures in an access unit.

For MHVC could allow gradual random access point.

Question about how splicing is impacted. An application spec could choose to restrict it.

Noted that nothing in the existing text forbids this.

Agreed.  Suggestion to add a clarifying note in the spec clarifying this point, left to editors’ discretion.

- On IDR and BLA pictures:

  - BLA pictures shall have nuh_layer_id equal to 0.

Question if it would simpler for splicers to change all layers to be BLA?  Reason is to re-use that BLA type for another purpose.  Further study.

  - IDR pictures may have nuh_layer_id greater than 0 and they may be inter-layer predicted while inter prediction is disallowed.

Noted that SVC allows similar flexibility.  Agreed.

  - IDR pictures shall be present in an access unit either in no layers or in all layers, i.e. an IDR nal_unit_type indicates a complete IDR access unit where decoding of all layers can be started.

Noted it may be useful for VPS activation specification.  May restrict some flexibility. Noted that IDR resets POC value to 0.  Agreed.

  - NAL unit types BLA_W_DLP and BLA_N_LP with nuh_layer_id greater than 0 are used to indicate a stepwise layer access (STLA) picture (STLA_W_DLP and STLA_N_LP, respectively), which is otherwise identical to an IDR picture with nuh_layer_id greater than 0 but needs not be aligned across layers.

Noted that this allows reuse of NUT values for stepwise layer switching, without requiring introduction of new NUT.  Question if CRA could accomplish same functionality. May impact definition of coded video sequence. Further study.

- On cross-layer impacts:

  - After a BLA picture at the base layer, the decoding of an enhancement layer is started when the enhancement layer contains a RAP picture and the decoding of all of its reference layers has been started.

Noted that it impacts where splicing is allowed.  Gives splicers some additional flexibility. Would impact decoding process.  Text is available in the contribution.  Further study.

  - When the decoding of an enhancement layer starts from a CRA picture, its RASL pictures are handled similarly to RASL pictures of a BLA picture.

Dependent on 2a.  Further study.

  - Layer down-switching or unintentional loss of reference pictures is identified from missing reference pictures, in which case the decoding of the related enhancement layer continues only from the next RAP picture on that enhancement layer.

Includes in the definition of a conforming bitstream one in which layer down-switching then up-switching is allowed. Question about HRD interaction. Further study.

JCT3V-C0081 AHG7: On Random access point pictures and picture order counts for MV-HEVC [B. Choi, M.W. Park, J. Yoon, C. Kim, J. Park (Samsung)]

To avoid a missing of inter-view reference pictures and make the random access and splicing easier, the contribution proposes several syntax changes on picture order count, NAL unit types, and slice types. First, it proposes a restriction on picture order counts (POCs) for all view components of a picture. If the POC values of view components in an access unit are not aligned, it may result in a decoding error or a corrupted decoded picture due to a wrong inter-view prediction. Second, it proposes a restriction on NAL unit types that all NAL units of all view components of a picture shall have the same value of nal_unit_type for a clean random access and splcing. Finally, it is suggested to change the restriction on slice types for non-base view components of RAP pictures for enabling inter-view prediction.

During discussion it was questioned if the POC restriction would be problematic for pre-encoded base layers, as it would be difficult to add a higher frame rate layer later.

POC for all HEVC layers in an access unit shall be the same.  Agreed.  Further study pre-encoded base layer with no gaps in POC and different frame rate between the layers.

JCT3V-C0085 AHG7: RAP picture alignment and slice definition [T. Ikai, Y. Yoshiya, T. Uchiumi (Sharp)]

In 3DV application, base view and dependent view is due to be presented simultaneously and this requirement is also true when random access is used.  However in the current specification, there are cases that only base picture can be decoded with random access because there are no restriction on RAP pictures usage in dependent layers.  This contribution proposes a restriction that when base layer is a RAP picture, the associated non base layers shall be also the RAP type picture.  In addition, this contribution proposes the I_SLICE restriction in RAP picture is only applied in base layer to improve coding efficiency in the case that both base and dependent layers are RAP pictures.

Already agreed as part of L0039/C0165, but provides additional Editorial improvements in MV-HEVC.

G.7.4.5 Slice header semantics

When nal_unit_type has a value in the range of 16 to 23 inclusive (RAP picture) and layer_id is equal to 0, slice_type shall be equal to 2.

Agreed.

JCT3V-C0084 AHG7: POC alignment between layers [T. Ikai, Y. Yoshiya, T. Uchiumi (Sharp)]

In 3D-HEVC Test Model 2 [JCT3V-B1005] reads “An access unit includes all video pictures and depth maps that correspond to the same time instant”.  However there is no specific way or no specific text for this condition in terms of POC.  Specifically coded slice segment NAL units of an access unit can have different POC which results in unnecessary difficulties in presentation time management in decoder.  This contribution provides a specific way to guarantee the same POC in the slices between layers.

Proposing semantic restriction to impose requirement that POC be the same for all layers.

Agreed in spirit.  A semantic restriction is needed to impose requirement that the POC be the same for all slices in all layers in the same access unit.  Left to editors’ discretion.

Second aspect proposes a restriction on pic_order_cnt_lsb.  Concern about impact on layers with different frame rates.

## VPS extension signaling of dimensions

JCTVC-L0200 On VPS extension [R. Skupin, V. George, K. Suehring, T. Schierl (Fraunhofer HHI)]

The document proposes a flag in the VPS extension syntax to indicate a sparse allocation of nuh_layer_id values that allow identification of dedicated scalable identifiers within nuh_layer_id when the so-called mapping approach for nuh_layer_id is used.

A second proposal additionally omits transmission of the mapping related syntax elements of the VPS extension when dedicated scalable identifiers within nuh_layer_id are present.

A first revision of the document reflects the changes proposed in JCTVC-L0181 that were agreed in a joint session of JCT-VC and JCT-3V with respect to the VPS extension. The proposed semantics were updated to clarify the derivation of scalable identifiers.

Question about semantics to have two different variables for the dimension ids.

In the first proposal, a flag is added, but syntax otherwise unchanged.  In second proposal, if flag is set, several syntax elements in VPS no longer sent.

An expert suggested that an application specification could perhaps be used to indicate the flag.

Expected that a middle box might benefit, if an application spec required that the proposed method be used and that the flag be mandated to be set.

Revisit option 1 after offline discussion to improve semantics.

Option 3 added to revise option 1.  splitting_flag used to indicate that bit mapping of layer_id is supported, but remaining syntax is unchanged.

Recommend to adopt option 3.

JCTVC-L0390 On layer id and dimension id mapping in VPS extension [Hendry, B. Jeon (LG)] [late]

It is observed that it may not be uncommon for scalable HEVC bitstreams to have only one type of scalability. In such case, the mapping between layer identification and scalability identification in VPS extension would be redundant layer id and dimension id are the same so that there is no need for signalling the mapping. This contribution proposes modification to avoid signalling the mapping between layer id and dimension id when a bitstream uses only one type of scalability.

Two proposals.  In first proposal, if NumScalabilityType equal to 1, do not send the dimension_id_len and dimension_id in VPS ext.  Saves some bits and some parsing in the VPS ext.  An expert noted that saving a few bits in VPS extension is not too significant.

A question raised about why vps_nuh_layer_id_present_flag is not sent if NumScalabilityType equal to 1.

In second proposal, single dimension flag added. A participant noted that this flag is a special case of functionality already present in scalability_mask.

An expert expressed concern that the proposed option 1 approach would be problematic for MV-HEVC, where the mapping to view_id is needed.

Further study to resolve potential negative impact with MV-HEVC for a single scalability dimension.

JCTVC-L0209 On Implicit Signaling of Scalability Dimension Identifiers [S Deshpande (Sharp)]

In current video parameter set extension design dimension Id values for each scalability dimension of each layer are explicitly listed and transmitted. This document additionally proposes a method for implicit signaling of scalability dimension identifiers in video parameter set extension.

Two options.  In first option, save some bits and parsing in the VPS to send dimension_ids more simply.  A derivation process replaces parsing when this implicit method is used.

In second option, also avoid sending vps_nuh_layer_id_present_flag.

Further study.

JCT3V-C0086 AHG7: On VPS extension [T. Ikai, Y. Yoshiya, T. Uchiumi (Sharp)]

In the last JCT-VC and JCT-3V meeting, NAL unit header and parameter set designs for HEVC extensions in JCTVC-K1007 and JCT3V-B1007 has been agreed. There vps_extension with Approach 1 in JCTVC-J1007 was also revised.  It is asserted that Approach 1 is suitable for general design for HEVC base extension because it has more flexibility than Approach 2.  Currently the current 3D-HEVC Test Model 2 [JCT3V-B1005] uses Approach 2 in view_id and depth_flag definition. This contribution proposes texts of Approach 1 base vps_extension.  This contribution also proposes valid_layer_flag[ i ] which is useful to indicate some specific layers do not exist in whole coded video sequences.

Proposed for MV-HEVC and 3D-HEVC, not SHVC.  Syntax element would have to be present for all extensions.  Perhaps could have mandated value for some extensions.

Want to a direct bit mapping of scalability dimensions from nuh_layer_id, but some combinations not valid, so indicate that.

Wants to mandate the linear mapping be used in all cases for of MV-HEVC and 3D-HEVC. May preclude some future extensions with combined scalability types.  A concern expressed that this reduces future flexibility.

No action.

## Layer dependency signaling

JCTVC-L0210 On Layer Dependency Signaling [S Deshpande (Sharp)]

In current video parameter set extension design layer dependency information is signaled using fixed length syntax elements which specify the number of layers each layer directly depends on followed by a list of these reference layer id values. This document proposes a new syntax for signaling layer dependency information. It is asserted that the proposed syntax is simpler and bit efficient.

4 variants proposed.

option 2 For each layer have a flag to indicate if layer is dependent or independent

option 3 Mask to indicate which layers are ref layers

option 4 combine options 1 and 3

Provides a mask that may be used by middle box and decoding process.

Some support expressed for the concept of a mask approach to signaling dependencies.

Need a process to provide an ordered list of references.

Need semantic text that fits into decoding process.  Encourage offline discussion of experts to review, led by T. Rusert.

Conditional revisit if offline discussion showed consensus between experts in offline discussion about harmonization with JCTVC-L0138.

Was revisited in JCTVC-L0446.

JCTVC-L0263 AHG 9: On inter-layer dependency signalling in VPS extension [Hendry, B. Jeon (LG)]

It is observed that the agreed signalling of layer dependency in VPS extension described in JCTVC-K1007 is still not tight enough so that it is for a layer to have reference to higher layer. Such referencing may not be desired for application that may extract bitstream based on layer information. This contribution proposes some modifications to syntax and semantic of ref_layer_id [i][j] to ensure that all coded pictures in a layer can only refer to pictures in lower layer.

Has identified a problem with the semantics that needs improvement.  Accepted in spirit.

JCTVC-L0138 On VPS extension design [Thomas Rusert (Ericsson)]

This contribution presents proposed modifications to the design of the VPS extension relative to the design in JCTVC-K1007. Two items related to claimed limitations in the VPS extension design in JCTVC-K1007 are addressed.

The first item is related to profile/tier/level signaling. It is proposed to change the number of iterations of the profile/tier/level loop in the VPS extension from vps_max_layers_minus1 to vps_num_op_sets_minus1.

The second item is related to layer dependency signaling. An alternative approach for layer dependency signaling is proposed which reportedly leads to savings of 11/30/56/60 bits for 2/4/8/16 layer configurations with 1 reference layer per layer, and savings of 11/42/92/144 bits for 2/4/8/16 layer configurations with 2 reference layers per layer. It is asserted that the worst case number of bits for dependency signaling is significantly lower than with the approach in JCTVC-K1007. It is also asserted that in cases when many layers are present in the bitstream, the proposed approach can be less bit efficient than the approach in JCTVC-K1007, however these cases are reportedly practically less relevant. It is claimed that the proposed approach of dependency signaling is conceptually similar to the “scalability_mask” approach of signaling scalability dimensions as already part of the VPS extension design in JCTVC-K1007, as well as the “operation_point_set()” signaling in JCTVC-K1003_v13.

Has some similarity to layer dependency mask of L0210.   Would need to define a variable RefLayerId that could be derived from this.

Provides bitrate savings in the VPS.  Imposes the restriction that a layer cannot reference a higher layer.  Simplifies the syntax and parsing, makes parsing not be conditional.

Some support expressed for the concept of a mask approach to signaling dependencies.

An expert expressed that the mask approach loses the ability to signal a specific default relative order of the references.

Need a process to provide an ordered list of references.

Need semantic text that fits into decoding process.  Encourage offline discussion of experts to review, led by T. Rusert.

Conditional revisit if offline discussion showed consensus between experts in offline discussion about harmonization with JCTVC-L0210.

Was revisited in JCTVC-L0446.

JCTVC-L0246 Proposal to the Extension of Video Parameter Set [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

In the last meeting, some solutions considered for NAL unit header and video parameter set for HEVC extensions in VPS extension are provided in JCTVC-K1007. In this contribution, some changes and new syntax for VPS extension are proposed. A summary of proposed items is as follows.

- Introduction of default dependency and specific dependency, where only the specific dependency between scalable layers are described in detail in VPS extension.

- Describing the maximum number of temporal sub-layers in each layer of a scalable bitstream. The reason is that each layer may have a different number of temporal sub-layer, which should be known by a middle box.

- Describing the priority of each layer of a bitstream. This information can be used as hints to adapt a bitstream, by removing the low priority layers first.

The proposed syntax appears to have some problems relating to use of default_dependency_flag[ ].

An expert mentioned that the default ordering isn’t clear for MV-HEVC.

Introducing two flags, sent per layer.

Need to describe how to derive the values of inferred dependendencies.

Would save bits in the VPS.

Further study, after consideration of mask layer dependency approaches.

JCTVC-L0226 VPS and vps_extension updates [M. Haque, A. Tabatabai]

This contribution presents some updates on VPS and syntax cleanups for vps_extension syntax structures.

Most of the cleanups for vps extension were addressed during the review of JCTVC-L0181.

Place byte alignment at end of VPS rather than start of VPS extension.

Revisit the byte alignment aspect.

- JCTVC-L0446 Layer Dependency Signaling in VPS Extension [S. Deshpande, T. Rusert] [late]

This document proposes syntax and semantics based on combining of proposals JCTVC-L0138 and JCTVC-L0210 based on feedback during BoG. Additionally semantics to derive NumDirectRefLayers[i] and RefLayerId[i][j] are added.

Recommend to adopt the second part, the bit mask signaling of layer dependency.

## VPS extension profile/tier/level and operation point signaling

JCTVC-L0180 Profile, tier, level and operation points signaling in the VPS extension [J. Boyce (Vidyo)]

Initial review chaired by K. McCann.

A method for profile_tier_level and operation point signaling in the VPS extension is proposed in this contribution, similar to the previous proposal in JCTVC-K0204.  The operation points signaled in the base VPS for HRD parameters are utilized, and additional operation points may be signaled in the VPS extension.  For each operation point, level information is sent, and profile and tier information may either be explicitly sent or may be inferred by referencing the previously sent profile and tier for another operation point.

Additional operation points, beyond those sent in the base VPS for the HRD, may be signaled in the VPS extension, using the same operation_point_set( ) syntax function.  An intended use case is when there is a pre-encoded base layer. One expert commented that it is part of the same NAL unit, so it is not so a problem that really needs to be solved.  The proponent agreed and this aspect of the contribution was withdrawn.

A second aspect of the proposal was based upon the assumption that several operation points may use the same profile and tier but different levels.  The proposal was asserted to provide an efficient way to represent different levels of same profile, so that 56 bits per layer can be saved out of 64.  One expert expressed agreement in principle, but commented that the details of the proposal could be improved.

Revisit the second aspect after offline discussion of experts interested in JCTVC-L0180, JCTVC-L0227 and JCTVC-L0138.

Revisit chaired by M. Hannuksela.

It was commented that the presented syntax would allow to refer to the profile-and-tier of the base layer. However, no other layer in the same bitstream can conform to any version 1 profile (as the other layers have nuh_layer_id greater than 0 which is not allowed in HEVC v1). It was therefore suggested to rename profile_op_ref to profile_op_ref_minus1.

The revised document was agreed with the suggested change.

JCTVC-L0227 VPS extension with updates of profile-tier-level syntax structure [M. Haque, A. Tabatabai (Sony)

Chaired by K. McCann.

This contribution presents some possible updates for the profile_tier_level syntax structure to support additional flexibility for HEVC extensions while being used in vps_extension syntax structure for each layer or operating point.

The option in the contribution presents a possibility that for general case, Level parameters may change separately from Profile/Tier Parameter changes.  One expert questioned the usefulness of this, since the existing design can deal with all scenarios and the bit-saving benefit would be limited to situations that do not frequently occur. Another expert suggested that this may occur in some form of adaptive streaming.

The second option in the contribution has some commonality with JCTVC-L0180 in signalling profile tier level per operation point rather than per layer.

Revisit the second option after offline discussion of experts interested in JCTVC-L0180, JCTVC-L0227 and JCTVC-L0138.

Revisited in the revision of JCTVC-L0180.

JCTVC-L0138 On VPS extension design [Thomas Rusert (Ericsson)]

Chaired by K. McCann.

Only the aspect related to profile_tier_level signaling was reviewed in this section.

This contribution presents proposed modifications to the design of the VPS extension relative to the design in JCTVC-K1007. Two items related to claimed limitations in the VPS extension design in JCTVC-K1007 are addressed.

The first item is related to profile/tier/level signaling. It is proposed to change the number of iterations of the profile/tier/level loop in the VPS extension from vps_max_layers_minus1 to vps_num_op_sets_minus1.

The second item is related to layer dependency signaling. An alternative approach for layer dependency signaling is proposed which reportedly leads to savings of 11/30/56/60 bits for 2/4/8/16 layer configurations with 1 reference layer per layer, and savings of 11/42/92/144 bits for 2/4/8/16 layer configurations with 2 reference layers per layer. It is asserted that the worst case number of bits for dependency signaling is significantly lower than with the approach in JCTVC-K1007. It is also asserted that in cases when many layers are present in the bitstream, the proposed approach can be less bit efficient than the approach in JCTVC-K1007, however these cases are reportedly practically less relevant. It is claimed that the proposed approach of dependency signaling is conceptually similar to the “scalability_mask” approach of signaling scalability dimensions as already part of the VPS extension design in JCTVC-K1007, as well as the “operation_point_set()” signaling in JCTVC-K1003_v13.

It was agreed that this proposal has some similarities to the common area of the previous two proposals and also includes something that is claimed to be effectively a bug fix.

Revisit after offline discussion of experts interested in JCTVC-L0180, JCTVC-L0227 and JCTVC-L0138.

Revisited in the revision of JCTVC-L0180.

JCT3V-C0059 AHG7: Target output views for MV-HEVC [Y. Chen, Y.-K. Wang (Qualcomm)]

In MVC (multiview extension of AVC), an operation point (OP) is defined by a set of target output views and a target temporal level. In the HEVC base specification, an OP is defined as a bitstream extracted from another bitstream by operation of the sub-bitstream extraction process, which takes the input as a target temporal level (tIdTarget) and a set of nuh_reserved_zero_6bits values (layerIdSetTarget), and no target output layer is defined or signalled. Consequently, in the currrent MV-HEVC wording draft, it is not possible to signal the target output views. This contributrion proposes to define a concept of OP with an assoicated list of target output views, referred to as output operation point (OOP), and to signal the target ouput views of each OOP in the VPS for MV-HEVC.

The existing signaling of operation points in the VPS in the v1 spec indicates all layers needed for decoding.  This contribution proposes additionally sending target output views in the vps extension.

Revisit in JCT3V.  Request changes based on top of JCTVC-L0188, including the output process and HRD should be provided.

VPS extension – what to signal

JCTVC-L0225 Video Sequence Characteristics Signaling in VPS and VPS Extension [M. Haque, A. Tabatabai, S. Deshpande]

This contribution proposes new syntax elements in vps_extension for the support of signaling mixed video types in the coded video sequences.

“Mixed” refers to different field/frame characteristics, or different 2D/3D characteristics.  It is asserted to be beneficial for negotiation.

The source scan type information would normally be carried in pic timing SEI message.

It was questioned if the VPS extension is the most appropriate location to include this information.

avc_base_codec_flag per layer.  This may be better handled in a systems layer, as the AVC layers would be independent of one another.

No action.

JCTVC-L0381 Parallelization indication in HEVC extensions [K. Suehring, R. Skupin, V. George, T. Schierl (Fraunhofer HHI)] [late]

This contribution proposes to add a high level syntax element to the VPS that indicates for each layer, if high level parallelization tools allow a higher level of parallelization in the enhancement layer compared to the corresponding base layer and by which factor the parallelisation opportunity differs.

Version 1 includes the min_spatial_segmentation_ratio_idc in the VUI.  For the current design, this information would be present in the VUI of each individual layer.

The asserted benefit is that it would be useful for negotiation to have this information in the VPS, without having to access the VUI for each layer.

No action.

JCTVC-L0246 Proposal to the Extension of Video Parameter Set [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

In the second aspect: Describing the maximum number of temporal sub-layers in each layer of a scalable bitstream. The reason is that each layer may have a different number of temporal sub-layer, which should be known by a middle box.

This information is present in each layer’s SPS, but having it available in the VPS may be useful for negotiation.

SVC’s layer not present SEI message could provide some of this information.

Further study for this aspect.

In the third aspect: Describing the priority of each layer of a bitstream. This information can be used as hints to adapt a bitstream, by removing the low priority layers first.

Similar to SVC’s priority_id functionality.  Question if this functionality in SVC was used in applications.

No action.

JCTVC-L0131 Multi-standard extension design [B. Choi, Y.J. Cho, M.W. Park, J. Yoon, J. Park (Samsung)]

Some aspects already discussed.

The second aspect is to signal the profile and level information of an AVC base layer in the VPS extension.

Concern expressed that hybrid codec scalability not fully thought through, and the big picture should be considered.

Some support expressed that this is compatible with the VPS design philosophy for carrying information and that it would be useful for negotiation.

Concern expressed that there should be a generic design to allow base layer from other standards in addition to AVC, e.g. MPEG-2.  Question if negotiation for the hybrid scalability case would use other parameter sets.

Question about what the SHVC requirements state regarding hybrid scalability, in terms of which other codecs are required.

Further study encouraged on a generic high level syntax design for hybrid scalability using different standards for the base layer.

JCTVC-L0262 AHG 9: Signalling of required DPB size in VPS [Hendry, B. Jeon (LG), D. Kwon, M. Budagavi, M. Zhou (TI)]

It is observed that there is no mechanism to signal required DPB size for decoding a multi-layer bitstream. This contribution proposes to signal the total number of required DPB size for each layer, which specifies the DPB size necessary to decode the layer including all lower layers that should be decoded necessarily. The proposed indications can be signalled in Video Parameter Set or in Video Parameter Set Extension.

It is asserted that if the target operation point is lower than the max capability, less memory may be required to decode.

Document to be updated to reflect inclusion in the vps extension rather than the VPS.

Revisit.

## Parameter set properties

JCTVC-L0132 Video parameter set design [B. Choi, Y.J. Cho, M.W. Park, J. Yoon, J. Park (Samsung)]

Two video parameter set (VPS) designs are proposed. The second design is a flexible design to remove duplicated syntax signalling between SPSs in multi-layers. Two flags are proposed to indicate whether the common syntaxes of SPSs in multi-layers are signalled only in VPS once, or whether the syntaxes of a specific SPS is predicted from a directly referred SPS.

Asserted benefit is bitrate savings in the SPS of enhancement layers.  Description is not complete, doesn’t indicate which syntax elements would not be duplicated in the SPS.  May introduce parsing dependency between VPS and SPS.

Third aspect adds a flag to allow parameter sets to be shared across layers.  The current design already allows that flexibility.  The benefit of signaling that it is disallowed is not clear. An identical parameter set could be copied if that was desired.

No action.

JCTVC-L0137 Inter-layer SPS prediction for HEVC extensions [Thomas Rusert (Ericsson)]

This contribution is a follow-up from JCTVC-K0276 and JCTVC-K0277, and is considering VPS design according to JCTVC-K1003_v13 and VPS extension design according to JCTVC-K1007.

The contribution addresses a redundancy problem that is asserted to be present when two or more different SPSs are present for different layers in multi-layer HEVC extensions. Since many SPS syntax elements may have identical values across SPSs, those values would be duplicated, thus causing bit overhead for parameter set transmission and storage.

First, criteria for the design of VPS and SPS for HEVC extensions are suggested. Specifically, it is suggested that (1) the VPS should (per its primary design purpose) include parameters that are useful for bitstream extraction and/or capability exchange, (2) parameters that are not required for bitstream extraction and/or capability exchange should not be present in the VPS, (3) sequence-level parameters that may change in some layers while not affecting the general layer structure should not be present in the VPS. For the design of the SPS with nuh_layer_id great than 0, it is suggested that (4) parameters present in the VPS should not be duplicated in the SPS (except for those parameters that are required for base layer decoding), and (5) sequence-level parameters that are not present in the VPS should be included in the SPS.

Then, following the design criteria, it is proposed for HEVC extensions to introduce inter-layer SPS prediction to reduce SPS-related redundancy. This implies extending the definition of SPS in the extension specifications. In the proposed SPS prediction scheme, SPS parameters are inherited at the time of SPS activation. The reference SPS for prediction is determined as the active SPS of the reference layer, which is signaled in the referencing SPS. The parameters to be inherited are indicated by flags.

Asserted benefit is bitrate savings in the SPS of enhancement layers.  Could help reduce negotiation delay.

Possible interaction with JCTVC-L0262.

Similar to JCTVC-L0132.

Have some syntax elements in the SPS that are in the VPS not be in the SPS for enhancement layers. This is consistent with the design philosophy.  Need to carefully consider each individual parameter and if the value should be identical for all layers.

Inter-SPS prediction across layers, in categories, each with a flag.  Has some relation to APS partial update.  Some support expressed for this concept.

In the activation process, refers to already active SPS.

Might want a bitstream constraint to disallow SPS prediction for a simulcast enhancement layer.

Further study in an AHG.  Add a mandate to AHG9 to study ways to reduce SPS redundancy.

## Non-parameter set

JCTVC-L0178 Legacy base layer codec support in SHVC [J. Boyce, D. Hong, W. Jang (Vidyo)]

Not reviewed.

JCTVC-L0197 Indication of tile boundary alignment [K. Suehring, R. Skupin, T. Schierl (Fraunhofer HHI)]

The contribution proposes to add a flag to VUI of an enhancement layer that allows signalling that the tile boundaries of all base layers are aligned with tile boundaries of that enhancement layer. This flag can be used in a decoder as guide for a parallelization strategy that allows parallel decoding of base and enhancement layer tiles.

The location for this flag was questioned.  Should be conditionally present.  Perhaps in an SPS extension.

Would the value of the flag be the same for all enhancement layers?  Not necessarily.

Question raised about upsampling filter accessing samples outside the tile.  In MV-HEVC disparity vectors might access outside the tile.

Support was expressed for the concept.

Further study encouraged to understand the decoding process impacts with out-of-tile referencing.

JCTVC-L0188 Unification of scalable and multi-view extensions with HLS only changes [K. Ugur, M. M. Hannuksela, J. Lainema, D. Rusanovskyy (Nokia)]

In this contribution, a high-level syntax only scalable extension standardization track for HEVC is proposed, in addition to a higher performing track that includes low level changes. It is futher proposed to unify the high level syntax only tracks for scalable and multi-view coding in a single extension of HEVC. The goal is to support both scalable and multiview use-cases with no low level changes to HEVC in a single extension, so that HEVC hardware encoders/decoders could be re-used with firmware updates.

Revision 2 of the contribution includes the following changes:

- It attempts to reflect the decisions of the joint BoG report on extension high-level syntax, as documented in JCTVC-L0441v1.

- The capability of using long-term reference pictures across layers was removed from the proposed straw-man text to be discussed separately as part of contribution JCTVC-L0170.

- The proposed specification text changes were divided into two annexes: Annex F contains the proposed common syntax, semantics, and decoding processes for all types of scalable extensions and scalability mechanisms (particularly reference index based and IntraBL-based). Annex G contains the proposed common syntax, semantics, and decoding processes for HLS-only scalable extensions. This division enables IntraBL-based scalable extension to be specified e.g. as Annex H and refer to the common syntax specified in Annex F.

Question raised about using the dependency information in VPS extension during the reference pic list construction process, which differs from the MV-HEVC method.  This proposed method is unique to this contribution, and not included in a separate contribution.    It was asserted this is an editorial only change, but this was questioned by an expert.  Chair suggests that this aspect is reviewed outside of this BoG.

- Support expressed for item 4, non-reference pictures at the highest decoded temporal sub-layer are marked as “unused for reference” immediately after their decoding to enable reduction of the DPB usage.

The following are the technical proposals unique to this contribution, and were reviewed by the BoG:

- Activation for picture and sequence parameter sets for individual layers was added similarly to the SVC and MVC.

- Agree.

- Non-reference pictures at the highest decoded temporal sub-layer are marked as “unused for reference” immediately after their decoding to enable reduction of the DPB usage.

- Agree. BoG recommends to revisit in track for version 1.

- The DPB size and other characteristics (e.g. sps_max_num_reorder_pics) is indicated for each layer separately in the active (layer) sequence parameter set for that layer. The DPB operates separately for each layer except for the bumping process of the output order DPB which operates across layers. The bumping process outputs consecutively, in ascending nuh_layer_id order, the pictures having the same smallest POC value among all the pictures (marked as “needed for output”) in the DPB.

- This modifies the MV-HEVC design, so should be further discussed in that context.

- The view dependency change SEI message of MV-HEVC was changed to equivalent layer dependency SEI message, which applies to all types of scalability.

- Agree.  Should be updated to reflect decision on JCTVC-L0446.