| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0236 |
| --- | --- |

| Title: | JCT-3V BoG report: MVC+D contributions |  |  |
| --- | --- | --- | --- |
| Status: | BoG report input to JCT-3V |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Shinobu Hattori (Sony) | Email: | Shinobu.Hattori@jp.sony.com |
| Source: | BoG on MVC+D contributions |  |  |

_____________________________

# Introduction

This BoG was established to review the contribution related to MVC+D specification at 3rd JCT-3V meeting. The BoG meeting took place on January 18th 2013 from 14:00 to 17:45.

The following 8 contributions were reviewed.

MVC+D related contributions (8):

| JCT3V number | Title | Authors |
| --- | --- | --- |
| JCT3V-C0031 | MVC+D: on target output views | M. M. Hannuksela, D. Rusanovskyy (Nokia) |
| JCT3V-C0057 | AHG7: Bitstream extraction for MVC+D | Y. Chen, Y.-K. Wang(Qualcomm) |
| JCT3V-C0069 | AHG4: Interlace coding on ATM Encoder | Chin-Chieh Lin, Fang-Chu Chen[ITRI] |
| JCT3V-C0071 | AHG4: MVC+D syntax mismatch with ATM 6.0 (frame coding) | Chin-Chieh Lin, Fang-Chu Chen[ITRI] |
| JCT3V-C0121 | MVC+D: Clarification on Depth acquisition information SEI message | Xiaofeng Yang, Carol Zhang, Roy YUE |
| JCT3V-C0156 | MVC+D HLS: On unpaired MVD and field coding | M. M. Hannuksela, D. Rusanovskyy (Nokia) |
| JCT3V-C0158 | MVC+D HLS: depth sampling characteristics | M. M. Hannuksela, D. Rusanovskyy (Nokia) |
| JCT3V-C0162 | MVC+D HLS: on depth acquisition information and depth representation information SEI messages | M. M. Hannuksela (Nokia) |

# Reviewed Proposals

## JCT3V-C0031: MVC+D on target output views [Nokia]

This contribution proposes to indicate whether a depth view or a texture view or both for each target output view is included in the 3DVC operation point. It is asserted that such indication requires syntax changes in the sequence parameter set 3DVC extension, the video usability information extension, the 3DVC view scalability information SEI message, and the 3DVC scalable nesting SEI message. Related changes are proposed for the main-level decoding process and for the sub-bitstream extraction process.

In addition, the contribution proposes the following changes:

- It is proposed to remove the capability of signalling texture-only operation points in the 3DVC view scalability information SEI message.

- It is proposed to add the information whether a texture view is coded with AVC/MVC NAL unit header and if a depth view is coded with AVC/MVC NAL unit header in the operation point signalling of the 3DVC view scalability information SEI message.

The contribution enables signaling of operation point for texture only, depth only or texture and depth. It enables flexibility of views to be operated depending on the application and services. The contribution proposes modifications to sequence parameter set 3DVC extension, VUI extension, 2 SEI messages to realize the signaling. The changes are similar to the flags already adopted in MVC+D which indicates whether the bitstream contain texture and/or depth for each view_id.

Through the proposed changes, the contribution enables packaging of access unit in case the bistream is composed of combination of field texture with frame coded depth which is unpaired depth but the time stamp aligns.

The sub-bitstream extraction process of unpaired depth is enabled which the similar concept is also proposed in JCT3V-C0057. The proposed text are slightly different, however the two proposals achieves the same output.

The SEIs are modified similar to the modification in sequence parameter set 3DVC extension to enable the operation points.

The syntax for the SEI message has been modified to incorporate the 3D-AVC usage – to enable usage of the same SEI messages in both the specification – semantics currently does not mention the presence of 3D AVC, however, they will be over-written in 3D-AVC specification.

Comments:

It was commented that the contribution is suggested to be considered in the Finland NB comments.

It was commented that the related contribution is provided in JCT3V-C0156 – which is more of clean ups.

It was commented that the contribution would allow depth view only operation points.

It was commented that no decoder complexity is added – supportive to introduce unpaired texture and depth operation points.

Recommendation:

Recommend to introduce unpaired texture and depth operation points.

The syntax and semantics except for the sub-bittream extraction process were agreed  See comments for JCT3V-C0057

The decoding process changes were also agreed.

## JCT3V-C0057: AHG7: Bitstream extraction for MVC+D [Qualcomm]

In MVC+D, texture views and depth views can have different view dependencies, and a target output view may contain only texture or only depth. In the current MVC+D bitstream extraction process, both the texture view and the depth view, when present, of a dependent view are included in the sub-bitstream when a target output view depends on either the texture view or the depth view, even when one of the two is needed neither for decoding nor for output. It is proposed that the bitstream extraction process be modified such that unnecessary depth views or texture views are not included in the output sub-bitstream.

The contribution proposed sub-bitstream extraction of texture only or depth only target views. Two separate conditions are introduced – each for texture and depth. The contribution incorporates the changes made to MVC specification at previous MPEG meeting.

The contribution is related to JCT3V-C0031.

Comments:

The proponent of the related proposal JCT3V-C0031 commented that the difference between JCT3V-0031proposal is that JCT3V-0031proposal introduced input parameter - texture and depth view list, however there are no contradictions between the two contributions.

Recommendation:

Recommend to introduce the sub-bitstream extraction process as proposed.

Recommend to merge descriptions with JCT3V-0031 and include it in the MVC+D specification.

Merged text to be provided.

## JCT3V-C0069: AHG4: Interlace coding on ATM Encoder [ITRI]

This document describes the issues and development status of ATM interlace support for consideration as MVC+D reference software.

The contribution reports on integrating interlace coding on MVC+D software – ATM. The contribution reported problems in ATM which needs to be resolved in order to support interlace coding in ATM.

As current status of integration work, the contribution reported that software is capable of coding all of test sequences as field, however, one sequence is indicating some blocking effect – which the reason is currently investigated.

Only the encoder is worked on currently. The decoder needs to be worked on as future work.

Future work was introduced to finalize the integration – related contribution JCT3V-C0071.

The contribution proposes coding configuration for the HP profile of ATM based on the integaration.

Comments:

Some participants commented that the proposed coding configuration should be specified in ATM software description – and the MVC+D ATM software description should be a separated document as independent document.

Recommendation:

Recommend to create an independent software description for MVC+D which includes proposed coding configuration. ITRI will provide the initial draft.

## JCT3V-C0071: AHG4: MVC+D syntax mismatch with ATM 6.0 (frame coding) [ITRI]

This contribution describes the syntax mismatches between ATM 6.0 and MVC+D specification[1].

The contribution reports that through the integration of interlace coding on ATM (as reported in JCT3V-C0069), the miss-match of the software and the MVC+D specification was found.

One of the miss-match requires specification changes as follows:

The current specification has a bug which disables encoding of chroma information etc (in SPS) for the MVC+D profile – profile_idc is missing in the condition.

Comments:

One participant commented that the reported bug is correct and should be fixed in the AVC specification as reported.

It was also commented that other miss-matches are already being worked on currently to fix the ATM software together with the proponent.

Recommendation:

Recommend to change the subclause 7.3.2.1.1 in AVC specification to include profile_idc == 138 to encode chroma information etc in sequence parameter set.

Recommend to continue the work to fix the ATM software miss-match.

## JCT3V-C0121: MVC+D: Clarification on Depth acquisition information SEI message [Huawei]

Camera parameters are coded in Depth acquisition information SEI message for view synthesis prediction (VSP). However the semantics of some camera related flag are not so clear. This contribution proposes to clarify the semantics of these flags. The proposed solution saves some bits in most cases by only adding minor descriptions to the current specification.

The contribution proposed simple modification to the current depth acquisition information SEI message. The contribution is to simplification the semantics of the parameters.

The modifications are proposed for following two parameters: 1) focal length, 2) principal point

For the focal length, the contribution pointed out that both directions of vertical and horizontal of focal_length should always be signaled together. It proposes to change the semantics of the existing flag to indicate that in case only one of either vertical or horizontal direction, it infers that they have the same values.

For the principal point, the contribution propose to infer the value to half of the image size in case the flag is not set to true which indicates signaling of specific values.

The contribution is related to JCT3V-C0162. JCT3V-C0162 proposes to remove the depth acquisition information SEI message – which indicates that proposed changes are not needed.

Comments:

One participant commented that there may not be necessary to have two different flags for vertical and horizontal direction focal length. One single flag to indicate signaling of both information may be enough – since it is usually the case the post-processing will need both of the information.

Some participants commented that the changes proposed in the contribution is minor such that this contribution should be first considered before considering JCT3V-C0162 which includes more complex proposal.

Recommendation:

Revisit – together with JCT3V-C0162.

## JCT3V-C0156: MVC+D HLS: On unpaired MVD and field coding [Nokia]

This contribution proposes asserted clean-ups and bug fixes related to the unpaired multiview-video-plus-depth (MVD) support and field coding. It also proposes that when an access unit contains coded depth field view components, they shall be of the same parity.

The contribution is clean ups and bug fixes of current MVC+D specification related to access unit structure of unpaired field texture and depth.

4 changes were proposed:

1) Delete some text in SPS MVC ext which disenables unpaired MVD,

2) Editorial paragraph reconstruction which defines the access unit structure,

3) Clarify the VOIdx for textre and depth view in SPS 3DVC extension,

4) Modification to disallow the flexibility of mixed field/frame coding within access unit similar to MVC constraints.

Comments:

Through the review, some modification in proposed text were made for Proposal 2) and Proposal 4).

For Proposal 2), the proposed text unintentionally allowed depth frame view component to be composed with second field of texture field view component – which is prohibited in the current MVC+D specification.

For Proposal 4), the proposal added the constraints to the profile section of the specification. Several comments were made that the constraints should be included in the main part of the specification – the reason being that the same constraints for the MVC is defined in the main part.

Recommendation:

Recommend the revised Proposal 2) and Proposal 4) text.

Recommend to include all prosed text changes (replacing text for Proposal 2) and Proposal 4) with revised text) to be included in MVC+D specification.

Revised text to be provided by the proponent.

## JCT3V-C0158: MVC+D HLS: depth sampling characteristics [Nokia]

This contribution proposes to enable indications of depth sampling grid location as well as depth sample size relative to those of the texture as video usability information (VUI). It is also proposed to enable indicating, as supplemental enhancement information (SEI) message, a suggested depth output time relative to the output time of the texture view components of the same access unit. It is suggested that these pieces of information can be assist a decoder side post-processing unit to perform depth-image-based rendering (DIBR) properly.

The contribution includes 2 proposals:

1) Add a metadata to indicate sample size and grid location of depth in VUI.

2) Define a new SEI message to indicate the time stamp of depth.

Comments:

Overall, supportive comments were made that both of information are useful.

It was commented that since the sample aspect ratio relative to the physical size for the texture case, the depth sample aspect ratio may also be defined in the same way.

It was commented that the similar information may be derived from the camera parameter – in case the camera parameter is provided, the proposed information may likely not be used.

It was commented that texture views may have miss-alignment in time stamps also (between views).

It was commented that the depth sample information proposed to be defined in VUI may be more suitable to define in the SEI message – no other depth specific VUI information is yet defined.

Recommendation:

Recommend the revise Proposal 1) to be defined as a new SEI message and included in MVC+D specification.  Revised text to be provided.

Recommend to introduce Proposal 2) in the MVC+D specification as proposed.

## JCT3V-C0162: MVC+D HLS: on depth acquisition information and depth representation information SEI messages [Nokia]

The contribution proposes to merge the depth acquisition information SEI message and depth representation information SEI message into a single SEI message. Moreover, several changes to the syntax and semantics of both messages are proposed.

The contribution is clean ups and bug fixes of the depth acquisition information SEI message and depth representation SEI message. The current syntax and semantics for depth acquisition information SEI includes redundant information and missing semantics.

Following proposals were made – only 1 proposal has spec text (Proposal 1)

- Enable currently defined multiview acquisition information SEI to be used for both texture and depth

- Remove the unnecessary constraint in depth acquisition information SEI, which disenables miss-alignment of camera position between texture and depth. Depth sensor camera may not always be in the same position as texture camera.

- Bug fixes of semantics for some of the parameters in depth acquisition information SEI.

- Missing semantics – units for DMin/DMax and ZNear/Zfar

- Clean ups and bug fixes in depth representation information SEI (unnecessary parameters, unclear semantics etc)

Comments:

It was commented that in case of unpaired texture and depth case, the correct mapping of texture views and depth views must be defined in the semantics of multiview acquisition information SEI message.

It was also commented that DMin and DMax may need to be defined in texture sample since it is used for texture view synthesis.

Overall, the proposed changes were agreed except Proposal 1) and Proposal 4).

Some participant commented that some more time is needed to review the proposal.

It was commented that since the changes are large and only part of the spec text are available at this moment, the thorough review of the members will be necessary.

Recommendation:

Revisit – Proposal 1) It was decided that some more time is needed to think whether it is suitable use the current multiview acquisition infornatuib SEI message for both texture and depth – especially in relation with related contribution JCT3V-C0121.

Revisit – Proposal 4) It was decided that some more time is needed to think whether DMin and DMax unit should be in units of depth sample or texture sample.

For other proposals – conceptually the proposed changes are agreed however, the exact text changes are to be provided.

BoG recommends following two options on how to proceed with the specification editing;

1) Set the editing period after Wed to include all the spec text for 7 proposals – which will delay the ITU consent.

2) Submit the most updated spec at Wed (include all the spec text possible by then) and submit ITU ballot comment for the remaining text changes.