| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0240 |
| --- | --- |

| Title: | Report: Subjective testing results on comparing warp-based with depth-based synthesis from coded data at same bit-rate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Nikolce Stefanoski | Email: | stefanos@disneyresearch.com |
| Source: | Disney Research Zurich |  |  |

_____________________________

# Abstract

According to proposal JCT3V-C0033, synthesis results generated from coded multi-view video + warp (MVW) data were assessed in a subjective test conducted during the meeting. Synthesis results were compared to corresponding synthesis results obtained from multi-view video + depth (MVD) data that is part of the 3D-HEVC anchors. Warp data was used that is coded at a bit rate which is as close as possible to the bit-rate of the anchor depth. In both cases (MVW and MVD) the same coded texture data is used. In the testing, 9 subjects participated. The subjective testing showed that a majority of votes supports the thesis that both types of supplemental data (warp and depth) lead to synthesis results of same quality. This statement is true for the high bit-rate and the low bit-rate case. Even in the remaining votes (votes excluding votes for ‘same quality’) of the high bit-rate case, there is no clear preference observable (19% warp, 19% depth). In the remaining votes of the low bit-rate case, sequences generated with help of warp data are preferred to sequences generated with help of depth data (36.5% warp, 11.1% depth).

# Introduction

In this document, subjective testing results are reported, which compare the quality of stereo sequences generated from multi-view video + warp (MVW) data with corresponding sequences generated from the multi-view video + depth (MVD) data. Synthesis is done from coded MVW and MVD data, which is coded at as close as possible bit rates. Coded MVD data consists of the current 3D-HEVC anchors. For more details on the coding and the used data, the reader is referred to document JCT3V-C0033.

In the testing, 9 subjects participated. Subjects had to assess the relative quality between stereo sequences generated by synthesis from MVW and MVD data. They were asked to answer which sequence they prefer by voting for one of the following possibilities: i) don’t know, ii) same, iii) warp, iv) depth. One test was done based on content which was compressed at high bit rate (QP 25) and a second test was done based on content compressed at a low bit rate (QP 40).

# Results

shows the voting results for each sequence separately. The same results are also reported in Table 1. In , two histograms are shown for each sequence. The histogram indicated as QP25, shows the histogram of all votes obtained for the high bit rate sequences. The histogram indicated as QP40, shows the histogram of all votes obtained for the low bit rate sequences. Figure 2 shows histograms which count votes over all sequences of the QP25 and QP40 case, respectively.

Figure 1: Histograms of voting results for each sequence separately.

Figure 2. Histograms of voting results over all sequences.

Figure 1 shows histograms of the voting results for each sequence separately. The same results are also reported in Table 1. For 5 from 7 sequences of the QP25 case (high bit-rate case), a majority voted that both sequences (generated with help of warp and depth data, respectively) have the same quality. In case of the UndoDancer sequence, the sequences generated with help of warp data was preferred by a majority of 6 from 9 votes and 1 subject preferred the sequence generated from depth data. In case of the Newspaper sequence, a majority of 4 from 9 voted for the sequence generated with help of depth data and 3 from 9 voted for the sequences generated with help of warp data.

For each of the sequences of the QP 40 case (low bit rate case), a majority of all subjects voted either that both sequences have same quality or preferred the sequences generated with help of warp data. In case of Poznan_Street, GT_Fly, Keno and Balloons both sequences where considered by a majority as the same, while in case of Poznan_Hall2, Undo_Dancer and Newspaper1 the sequence generated with help of warp data were preferred.

According to Figure 2 and Table 1, in the high bit-rate case, overall a majority of 58.7% considered that both types of sequences are of same quality, 19% preferred sequences generated from warp data, and again 19% preferred sequences generated from depth data. Hence, it is observable that in the high bit-rate case a majority considers that both types of sequences have the same quality. In the remaining votes, there is no clear preference for sequences of a certain type of supplementary data observable.

In the low bit-rate case, a majority of 44.4% considered that both types of sequences are of same quality, 36.5% preferred sequences generated from warp data, while 11.1% preferred sequences generated from depth data. Obviously, also in the low bit rate case a majority considers that both types of sequences have the same quality. However, in the remaining votes sequences generated with help of warp data are preferred to sequences generated with help of depth data.

Table 1: Summary of voting results.

# Summary

The subjective test showed that a majority of votes supports the thesis that both types of supplemental data (warp and depth) lead to synthesis results of same quality. This statement is true for the high as well as for the low bit-rate case. Even in the remaining votes (votes excluding votes for ‘same quality’) of the high bit-rate case there is no clear preference observable (19% warp, 19% depth). In the remaining votes of the low bit-rate case sequences generated with help of warp data are preferred to sequences generated with help of depth data (36.5% warp, 11.1% depth).

# Patent rights declaration(s)

Disney Research Zurich may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).