| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1) on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Shinya Shimizu | Email: | shimizu.shinya@lab.ntt.co.jp |
| Source: | JCT-3V CE1 Coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 1 (CE1) of the development of 3D extensions for HEVC and AVC. This Core Experiment investigates coding methods based on view synthesis prediction, which uses texture and depth information of the independent view to construct a synthesized prediction signal for coding the dependent views.

# Participants

| Participant | Contact | Email | Proponent |
| --- | --- | --- | --- |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | P/C |
| Mitsubishi Electric | Dong Tian | tian@merl.com | P/C |
| RWTH Aachen University | Fabian Jäger | jaeger@ient.rwth-aachen.de |  |
| MediaTek | Yu-Lin Chang Yi-Wen Chen Yu-Wen Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | Xin Zhao Jewon Kang | xinzhao@qti.qualcomm.com jewonk@qti.qualcomm.com | P/C |
| Nokia | Dmytro Rusanovskyy | Dmytro.Rusanovskyy@nokia.com | P/C |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | C |
| LG Electronics | Sehoon Yea | sehoon.yea@lge.com |  |
| Samsung | Jin Young Lee | jinyoung79.lee@samsung.com |  |
| ETRI | Gun Bang | gbang@etri.re.kr |  |
| KyungHee University | Kyung Yong Kim | kimky@khu.ac.kr |  |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
| KDDI | Kei Kawamura | kei@kddilabs.jp | C |
| GIST | Yunseok Song Yo-Sung Ho | ysong@gist.ac.kr hoyo@gist.ac.kr | C |
| HHI | Heribert Brust | heribert.brust@hhi.fraunhofer.de |  |

(P = proponent, C = cross checker)

# Tools under Investigation

The tools to be investigated in this Core Experiment use the decoded texture and depth view components of the already coded views to compute a prediction signal for the texture and depth view components of the coding target views. This process is done by means of disparity compensation, where the disparity values are derived from the decoded depth information.

## CE1.a: View Synthesis Prediction for 3D-ATM

A block-based VSP using a backward warping is supported in the 3D-ATM. The warping process is performed by 4x4 block basis disparity compensation, where disparity vector is derived from the maximum depth value within depth samples on four corners of the block.

The tools to be investigated can be categorized into the following 3 categories. All separable tools should be evaluated separately according to the following categorization. Reports on combined tools are also encouraged.

- Block size for warping

  - 8x8 (JCT3V-C0169)

- Removal of VSP reference picture from the reference picture list (JCT3V-C0053, JCT3V-C0130)

- Sub-MB level VSP skip (JCT3V-C0053)

On the test of item (b), the length of the reference picture lists is increased by 1 in order to insert one VSP reference picture.

Regarding the item (c), the impacts on the flexibility on the choice of the reference view for B-view and the benefits of the additional motion compensation are claimed. In order to investigate such aspects, the test on the IBP inter-view configuration is also conducted in addition to the CTC.

### JCT3V-C0053: Results on GVSP

GVSP is a way to enable sub-MB skip/direct using VSP reference pictures. In addition to a vsp_flag at MB level, a smaller partition, up to 8x8 blocks, can be signaled whether it is predicted from a VSP reference picture. It is also proposed to remove VSP picture from the reference picture list.

This contribution should be evaluated in terms of the tools in categories (b) and (c).

### JCT3V-C0130: 3D-CE1.a: Inter-view skip/direct mode with sub-partition scheme

The inter-view skip/direct mode is to utilize the depth from the current view and pick up sample predictors from a reference view by using a backward warping. No sample level vector is used, but a maximum depth value in the 4x4 is used for the whole 4x4 block. This contribution also proposes a sub-MB level skip/direct mode and the removal of the VSP reference picture.

This contribution should be evaluated in terms of the tools in categories (b) and (c).

### JCT3V-C0169: CE1.a-related: Simplification of BVSP in 3DV-ATM

This contribution proposes to use 8x8 block basis BVSP in order to reduce the computational complexity. A 8x8 block based warping will help the limits of the block size for the bi-prediction and the number of 4x4 motion compensation.

This contribution should be evaluated in terms of the tools in categories (a).

## CE1.h: View Synthesis Prediction for 3D-HTM

A block-based VSP using a backward warping was adopted into 3D-HTM at Geneva meeting, which is done with a two-step disparity compensation. In the first step, a virtual depth map is fetched for a CU by calling an updated NBDV function. In the second step, VSP compensation is performed on a 4x4 block basis and prediction samples are generated. For VSP compensation, a disparity vector is derived from the derived virtual depth map, where a maximum depth values in each 4x4 block is selected. In addition, a refinement on disparity vector returned by NBDV function is applied on both VSP coded neighbor block and regular coded neighbor block (non-VSP mode).

The tools to be investigated can be categorized into the following 3 categories. All separable tools should be evaluated separately according to the following categorization. Reports on combined tools are also encouraged.

- Block size for the second disparity compensation

  - 8x8

  - Adaptive (no smaller than 4x8 or 8x4) (JCT3V-C0100)

- The representative depth value derivation for each block

  - Max among the corner samples (JCT3V-C0100, JCT3V-C0131)

  - Adaptive (JCT3V-C0112)

- Signalling of VSP

  - Reference picture index (JCT3V-C0100, JCT3V-C0104)

### JCT3V-C0100: CE1.h: Backward Projection based View Synthesis Prediction using Derived Disparity Vector

Adaptive block size, no less than minimum PU size for inter prediction, on the warping stage of BVSP is proposed in order to align with the HEVC design. The use of maximum depth value among the corner samples is also proposed. In this contribution, VSP is activated by signaling a reference picture index in addition to the merge/skip index.

This contribution should be evaluated in terms of the tools in categories (a), (b), and (c).

### JCT3V-C0104: CE1.h: View Synthesis Prediction using Forward Warping

In this contribution, VSP is activated by signaling a reference picture index in addition to the merge/skip index.

This contribution should be evaluated in terms of the tools in categories (c).

### JCT3V-C0112: CE1.h related: Adaptive method for Depth Oriented Neighboring Block Disparity Vector

This contribution proposes many ways to derive a disparity vector for a block by using depth map in order to improve NBDV. The same disparity vector derivation will be applied on the process of BVSP.

This contribution should be evaluated in terms of the tools in categories (b).

### JCT3V-C0131: 3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval

This contribution proposes to improve NBDV with virtual depth map retrieval. The refinement is performed by using the maximum depth value among the corner samples of the corresponding block. The same disparity vector derivation will be applied on the process of BVSP.

This contribution should be evaluated in terms of the tools in categories (b).

# Core Experiment Conditions

## Software

Experiments in CE1.a will use the ATM 7.0 software and experiments in CE1.h will use the HTM 6.0 software, as recommended in JCT3V-C1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences

Test sequences will be used according to common test conditions (CTC) [1].

## Coding Conditions

### CE1.a

The experiments are conducted under CTC [1].

For the tool (c) in the section 2.1, the additional experiments are conducted using the IBP interview prediction structure.

### CE1.h

The experiments are conducted under CTC [1].

## Evaluation Criteria

Evaluation will be done according to CTC [1].

Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the overall simulation results.

- As view synthesis prediction requires depth data to encode texture views, the total bitrate (including depth rate) should be compared against all reported PSNR measures (for decoded and synthesized views). 
The updated Excel sheet should be used for reporting the appropriate simulation results.

Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

Proponents are encouraged to report details about decoder complexity in terms of number of operations and memory consumption.

## Timeline

2013/01/31		Finalize CE description
2013/02/14		Release 3D-ATM 7.0

2013/02/22		Release 3D-HTM 6.0

2013/04/06		Make source code and simulation results available for all CE1 participants, not only for the cross-checkers

2013/04/13		Register documents for the JCT-3V meeting

2013/04/13		Upload simulation and cross check results to the JCT-3V document repository

2013/04/20-26	The 4th JCT-3V meeting

# Reference

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-C1100, Geneva, CH, 2013.