| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C1103 |
| --- | --- |

| Title: | Description of Core Experiment 3 on Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
|  | S. Kamp Monzastraße 4c 63225 Langen Germany | Tel: Email: | +49 6103 766271 steffen.kamp@eu.panasonic.com |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to inter-component prediction. During the 3rd JCT-3V meeting, contributions have been proposed taking benefit of inter-component predictions. In particular, JCT3V-C0137 proposed to remove MPI while adding a new merge candidate. MPI has been kept in the reference software. The goal of this CE is to test the combination of MPI jointly with the new merge candidate adopted from JCT3V-C0137, in the context of the 3D-HTM 6.0, under the common test conditions.

# Participants List

| Participant | Person, Email address | Status (P/C/-) |
| --- | --- | --- |
| HHI |  |  |
| MediaTek | Y.-W. Chen (yiwen.chen@mediatek.com) J.-L. Lin (jl.lin@mediatek.com) Y.-W. Huang (yuwen.huang@mediatek.com) S. Lei (shawmin.lei@mediatek.com) | P |
| Orange Labs | J. Jung (joelb.jung@orange.com) E. Mora (elie.mora@orange.com) | C |
| Panasonic | S. Kamp (steffen.kamp@eu.panasonic.com) | C |

Table 1: participant list

# Experiment description

Adopted proposal (in the 3D-HTM, working draft, and CTCs):

JCT3V-C0137 - [MediaTek] - Removal of parsing dependency and picture buffers for motion parameter inheritance

This contribution is related to previous contribution JCT3V-B0085.

It is proposed to replace motion parameter inheritance (MPI) by adding one additional candidate, called texture merging candidate, into the merge candidate, set for the depth merge/skip mode coding.

Similar to the concept of inter-view merging candidate, the proposed texture merging candidate directly reuse the motion vectors (MVs) and reference index of the corresponding texture block as a merging candidate in depth coding.

As shown in Figure 1, the corresponding texture block is selected as the 4x4 block (C3 block) located to the right bottom of the center of the current PU in the corresponding texture picture.

Figure 1. The derivation of corresponding texture block

Table 2 shows the proposed merge candidate set for texture and depth.

| Merge candidate set in the proposed scheme |  |
| --- | --- |
| Texture | Depth |
| 1. Inter-view candidate 2. Spatial candidate 3. Temporal candidate 4. Additional candidate | 1. Texture candidate 2. Spatial candidate 3. Temporal candidate 4. Additional candidate |

Table 2: proposed merge candidate set.

MPI tool: in the 3D-HTM, but not in the CTC.

MPI description:

The basic idea behind the motion parameter inheritance (MPI) mode is that the motion characteristics of the video signal and its associated depth map should be similar, since they are both projections of the same scenery from the same viewpoint at the same time instant. Therefore, in order to enable efficient encoding of the depth map data, a new coding mode that allows inheritance of the treeblock subdivision into CUs and PUs and their corresponding motion parameters from the video signal has been introduced. Since the motion vectors of the video signal have quarter-sample accuracy, whereas for the depth map signal only full-sample accuracy is used, in the inheritance process the motion vectors are quantized to their nearest full-sample position. It can be adaptively decided for each block of the depth map, whether the motion data are inherited from the co-located region of the video signal or if new motion data are transmitted (cp. Figure). For signalling the MPI coding mode, the merge/skip mode syntax is used. The list of possible merge candidates has been extended in a way that, for depth map coding, the first merge candidate refers to merging with the corresponding block from the associated video signal.

Figure 2: Illustration of the concept of motion parameter inheritance.

Independent of the partitioning of the video picture into its CUs, the MPI mode can be used at any level of the treeblock hierarchy for the depth map. If the MPI mode is indicated at a higher level of the depth map coding tree, corresponding to a CU size that is larger than the CU size that is used for the video signal, the CU/PU subdivision, together with the corresponding motion data, is inherited from the video signal. This makes it possible to specify once for a whole treeblock, typically corresponding to 64 × 64 image samples, that the whole partitioning of this region into its CUs and PUs for the video signal is also applied to the depth map signal. In the other case, if MPI is indicated at a level of the coding tree that corresponds to the same or a smaller CU size than the CU size that is used for the video signal, only the motion data are inherited from the video signal. Since, when using MPI, not only the partitioning and the motion vectors, but also the reference picture indices are inherited from the video signal, it has to be ensured, that the depth maps that correspond to the video reference pictures are also available in the reference picture buffer for the depth map signal. If the region of the video signal, that the motion data and partitioning are to be inherited from includes PUs that are intra coded or not available, the components of the motion vectors and reference indices for these regions are set equal to 0.

Table 3: Summary of the experiments

| Experiment | Tool involved |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
|  | MPI | Texture Merging candidate |  |  |
| Anchor | OFF | ON | - | - |
| Test 1 | ON | ON | HHI ? |  |

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCT3V-C1100. 3D-HTM 6.0 will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | February 13th | Final description of proposals sent to the coordinators |
| --- | --- | --- |
| T2 | February 15th | Upload of Final CE description document |
| T3 | Availability of 3D-HTM6.0 February 22nd | Start of tool integration |
| T4 | April 3rd | Completion of tool integration, test configuration announcement Start of testing |
| T5 | April 10th | Testing complete Reporting of all results to CE3 participants complete |
| T6 | Official document submission deadline April 13th | Input documents uploaded |
| T7 | T6 + 3 days April 16th | CE3 summary report uploaded |

Table 4: Time-line