| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0024 |
| --- | --- |

| Title: | CE4: Summary Report on Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of the Core Experiment CE4 on Residual Prediction. During the 3rd JCT-3V meeting, one contribution has been proposed to further improve the coding efficiency of residual prediction in 3D-HEVC and it was considered for further study in a Core Experiment (JCT3V-C0049). The goal of this CE is to evaluate it in the context of the 3D-HTM 6.0 under the common test conditions.

All of the experiments are related to 3D-HTM only. 8 companies were involved in the CE with 1 CE proposals and 2 CE-related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Ying Chen | lizhang@qti.qualcomm.com cheny@qti.qualcomm.com | P |
| MediaTek | Jian-Liang Lin Jicheng An Kai Zhang Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com jicheng.an@mediatek.com Kai.zhang@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com |  |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp |  |
| LG | Sehoon Yea | sehoon.yea@lge.com | C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
| Intel | Yi-Jen Chiu Zhipin Deng Wenhao Zhang | yi-jen.chiu@intel.com zhipin.deng@intel.com wenhao.zhang@intel.com |  |
| Samsung | Jin Young Lee | jinyoung79.lee@samsung.com |  |
| Nokia | Dmytro Rusanovskyy | Dmytro.Rusanovskyy@nokia.com |  |

# Tools under investigation

## Advanced residual prediction for multiview coding [JCT3V-D0177]

This proposal is a follow-up of JCT3V-C0049. Inter-view residual prediction (IVRP) is enabled in the current 3D-HTM design to code the residue of dependent texture views more efficiently. In this proposal, an advanced residual prediction (ARP) is proposed to further improve the coding efficiency of inter-view residual prediction. In ARP, motion of the current block of picture in current view is applied to the corresponding block in a reference view picture to generate residual predictor. Moreover, an adaptive weighting factor is applied to the residue signal so that the prediction error is further reduced. Meanwhile, the conditional signalling of ARP which depends on the reference picture types is removed. It is shown that the proposed method achieves compression efficiency gain of 0.8% and 1.3% for coded views in terms of BD rate compared to 3D-HTM anchor w/ and w/o IVRP enabled, respectively.

Two methods are proposed to further reduce the complexity of ARP.

- Simplification #1: it is proposed to use fixed reference pictures for all blocks coded with ARP. Compared to the original ARP, the coding loss introduced by such simplification is less than 0.05% for synthesized views.

- Simplification #2: Combined with simplification #1, bi-linear filter instead of 8/4-tap filters for non-base texture views is further applied when the weighting factor is unequal to 0.

It is claimed that analysis shows that the proposed method achieves more than 2.5 times of the coding gain compared to the current IVRP while require only insignificant increase of implementation complexity compared to IVRP. Detailed complexity analysis is also described in this proposal.

This proposal was crosschecked by LG and detail information can be referred to JCT3V-D0093.

# Experiments and results

Table 1: Summary of simulation results compared to 3D-HTM w/o IVRP

| Test cases |  |  |  | Simulation Results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Video 1 |  | Video 2 |  | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Dec time |
| IVRP in 3D-HTM | -1.2% |  | -1.3% |  | -0.5% | -0.5% | -0.4% | 112% |
| ARP w/o simplifications | -3.7% |  | -3.7% |  | -1.3% | -1.2% | -1.1% | 101% |
| Simplification #1 | -3.5% |  | -3.4% |  | -1.3% | -1.2% | -1.0% | 102% |
| Simplification #2 | -3.3% |  | -3.2% |  | -1.1% | -1.0% | -0.9% | 99% |

Table 2: Summary of simulation results compared to 3D-HTM with IVRP

| Test cases |  |  |  | Simulation Results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Video 1 |  | Video 2 |  | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Dec time |
| ARP w/o simplifications | -2.5% |  | -2.4% |  | -0.8% | -0.8% | -0.6% | 90% |
| Simplification #1 | -2.3% |  | -2.1% |  | -0.7% | -0.7% | -0.6% | 90% |
| Simplification #2 | -2.1% |  | -1.9% |  | -0.6% | -0.6% | -0.5% | 88% |

# Related proposals

## Simplified residual prediction with luma clipping and chroma removal (JCT3V-D0063)

This proposal introduces simplification of inter-view residual prediction. Two methods are proposed:

- Method #1: the luma residual samples are clipped in 8 bit range to reduce memory size for storing residual signal. It is reported that there is no coding loss.

- Method #2: the chroma residual prediction is completely removed.

It is reported there is negative coding performance differences with the above two methods compared to 3D-HTM 5.0.

This proposal was crosschecked by MediaTek and detail information can be referred to JCT3V-C0185.

## Limited inter-view residual prediction by CBF checking (JCT3V-D0148)

In current 3D-HEVC, IVRP is always enabled when the temporal inter-view candidate with 2Nx2N partition is selected. In this contribution, it is proposed to control the on/off of IVRP by checking the coded block flag (CBF) of the reference residual block. When the transform unit (TU), which covers the center sample of the reference block in reference view, does not contain non-zero coded block flag (CBF), IVRP is disabled. It is reported that negligible coding gain, i.e., 0.02% bitrate reduction for synthesized views is achieved and the decoding time is saved by 9%.

This proposal is crosschecked by Sharp in JCT3V-D0253 which shows the decoding time saving is 7%.