| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0025 |
| --- | --- |

| Title: | 3D-CE5: Summary report on inter-view/motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Sehoon Yea | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of proposals and cross-check results of CE5 is reported.

# Introduction

A summary of proposals and cross-check results of CE5 is reported.

## Mandates

# Mandates

## CE5

The mandates of this core experiment are as follows:

- To further investigate ways to improve merge list construction and the interaction between VSP merge candidate [JCT3V-C0045, JCT3V-C0148]

- To compare different solutions for the motion compensation of interview prediction: [JCT3V-C0118]

## Participants

Participants with proposals at this meeting in CE5 are as follows:

## CE5

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Jewon Kang | lizhang@qualcomm.com jewonk@qti.qualcomm.com | P/C |
| INRIA | Thomas Guionnet Laurent Guillo Christine Guillemot | Thomas.Guionnet@inria.fr Laurent.Guillo@inria.fr Christine.Guillemot@inria.fr | P/C |
| LG | Taesup Kim Jin Heo Moonmo Koo Sehoon Yea | taesup.kim@lge.com jin78.heo@lge.com moonmo.koo@lge.com sehoon.yea@lge.com | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com | C |
| Sharp | Tadashi Uchiumi Tomohiro Ikai | uchiumi.tadashi@sharp.co.jp ikai.tomohiro@sharp.co.jp | C |
| Samsung | Jin Young Lee | jinyoung79.lee@samsung.com | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |
| MERL | Dong Tian | tian@merl.com | C |
| Hisilicon | Yongbing Lin | yblin@huawei.com | C |
| Panasonic | Steffen Kamp | Steffen.Kamp@eu.panasonic.com | - |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |

# List of input documents

## CE5.a

Two input contributions were gathered in this category, among which was 1 proposal. The input contributions are:

- Direct/Skip mode simplification

JCT3V-D0192 (Qualcomm)

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Qualcomm | JCT3V-D0192 | CE5.a related: Direct/skip mode simplification for 3D-AVC | Proposal |
| MediaTek | JCT3V-D0262 | 3D-CE5.a related: Crosscheck on Direct/skip mode simplification for 3D-AVC (JCT3V-D0192) | Cross-Check |

## CE5.h

A total of 26 input contributions were gathered in this category, among which were 14 proposals. The input contributions can be roughly categorized as follows:

- AMVP/Merge List Construction

Merge candidate derivation: D0178(Qualcomm & INRIA)

Additional interview merge candidate: D0109(MediaTek)

AMVP candidate list construction: D0122(Samsung)

- Interview SAO

Interview SAO: D0091 (LG)

- Illumination compensation

Removal of parsing dependency for illumination compensation: D0060(Sharp) D0152(MediaTek)

Illumination compensation simplification: D0061(Sharp) D0062(Sharp) D0096(LG) D0106(MediaTek) D0114(Samsung)

- Others

Disparity vector rounding: D0135(LG)

Reference view for interview motion prediction and interview residual prediction: D0145(MediaTek)

Interview prediction using image deformation characeteristics: D0248(LG)

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Sharp | JCT3V-D0060 | 3D-CE5.h related: Removal of parsing dependency for illumination compensation | Proposal |
| Sharp | JCT3V-D0061 | 3D-CE5.h related: Illumination compensation regression improvement and simplification | Proposal |
| Sharp | JCT3V-D0062 | 3D-CE5.h related: Combination of JCT3V-D0060 and JCT3V-D0061 | Proposal |
| LG | JCT3V-D0086 | 3D-CE5.h related: Crosscheck of removal of parsing dependency for illumination compensation (JCT3V-D0152) | Cross-Check |
| LG | JCT3V-D0091 | CE5.h related : Inter-view SAO process in 3DV coding | Proposal |
| LG | JCT3V-D0096 | CE5.h The results on simplified illumination compensation | Proposal |
| LG | JCT3V-D0099 | 3D-CE5.h related: Cross-check of Illumination compensation regression improvement and simplification(JCT3V-D0061) | Cross-Check |
| MediaTek | JCT3V-D0106 | 3D-CE5.h related: On complexity reduction of bi-prediction for illumination compensation | Proposal |
| MediaTek | JCT3V-D0109 | 3D-CE5.h related: Additional inter-view merging candidate | Proposal |
| Samsung | JCT3V-D0114 | 3D-CE5.h related: Simplification of illumination compensation | Proposal |
| Samsung | JCT3V-D0122 | 3D-CE5.h related: AMVP candidate list construction | Proposal |
| Samsung | JCT3V-D0128 | 3D-CE5.h related: Cross check of Unification of disparity vector rounding (JCT3V-D0135) | Cross-Check |
| LG | JCT3V-D0135 | 3D-CE5.h related: Unification of disparity vector rounding | Proposal |
| MediaTek | JCT3V-D0145 | 3D-CE5.h related: Unification of the reference view for inter-view motion prediction and inter-view residual prediction | Proposal |
| MediaTek | JCT3V-D0152 | 3D-CE5.h related: Removal of parsing dependency for illumination compensation | Proposal |
| Qualcomm & INRIA | JCT3V-D0178 | CE5.h: Merge candidates derivation from vector shifting | Proposal |
| LG | JCT3V-D0248 | CE5.h : Inter-view prediction using image deformation characteristics between multi-view images | Proposal |
| Sharp | JCT3V-D0249 | 3D-CE5.h related: cross-check of AMVP candidate list construction (JCT3V-D0122) | Cross-Check |
| Sharp | JCT3V-D0254 | 3D-CE5.h related: cross-check of Unification of the reference view for inter-view motion prediction and inter-view residual prediction (JCT3V-D0145) | Cross-Check |
| Sharp | JCT3V-D0255 | 3D-CE5.h related: cross-check of Removal of parsing dependency for illumination compensation (JCT3V-D0152) | Cross-Check |
| Sharp | JCT3V-D0256 | 3D-CE5.h related: cross-check of Additional inter-view merging candidate (JCT3V-D0109) | Cross-Check |
| Sharp | JCT3V-D0258 | CE5.h: cross-check of The results on simplified illumination compensation (JCT3V-D0096) | Cross-Check |
| MediaTek | JCT3V-D0261 | 3D-CE5.h: Crosscheck on Merge candidates derivation from vector shifting (JCT3V-D0178) | Cross-Check |
| Qualcomm | JCT3V-D0274 | CE5.h related: Cross check on Removal of parsing dependency for illumination compensation (JCT3V-D0060) | Cross-Check |
| Qualcomm | JCT3V-D0275 | CE5.h related: Cross check on Combination of JCT3V-D0060 and JCT3V-D0061 (JCT3V-D0062) | Cross-Check |
| Qualcomm | JCT3V-D0279 | CE5.h related: Cross check on Inter-view SAO process in 3DV coding (JCT3V-D0091) | Cross-Check |

# Summary of proposals & results

## CE5.a related

- JCT3V-D0192 [Qualcomm] CE5.a related: Direct/skip mode simplification for 3D-AVC (Cross-Checked by JCT3V-D0262)

In the current ATM software for the skip/direct MBs, when the inter-view motion vector is not available, the median value of motion vectors of the neighboring blocks is used. This contribution proposes to derive the MVP for the skip/direct MBs to be the motion vector of the first available spatial neighbour, including the reference index. Experimental results show that the proposed simplified MVP scheme gives an overall coding gain of 0.06% and synthesized coding gain of 0.08% when compared to the CTC results.

## CE5.h results

### Merge List Construction

JCT3V-D0178 is the CE5.h results for merge list construction. JCT3V-D0109 is a 3D-CE5.h related contribution about merge list construction.

- JCT3V-D0178 [Qualcomm & INRIA] CE5.h: Merge candidates derivation from vector shifting (Cross-Checked by JCT3V-D0261)

As a harmonized follow-up proposal of JCT3V-C0045 and JCT3V-C0148, it is proposed to add two more candidates. Each candidate is derived with a horizontally shifted disparity vector being used for inter-view motion prediction and if it doesn’t provide an available candidate, then it is derived from a horizontally shifted spatial neighbouring candidate, if it contains a disparity motion vector. Compared to the current HTM design, the proposed method achieves compression efficiency gain of 0.2% in terms of BD rate for both BVSP on and off cases.

- JCT3V-D0109 [MediaTek] 3D-CE5.h related: Additional inter-view merging candidate (Cross-Checked by JCT3V-D0256)

In this contribution, an additional inter-view merging candidate is proposed to be added in the merging candidate list for the texture coding in a dependent view. For the pruning process, the additional inter-view candidate is only compared with the original inter-view candidate for redundancy removal. The simulation results reportedly show that the proposed additional inter-view merging candidate could achieve 0.2%, 0.1% and 0.1% BD-rate savings on average for video1, video2, and overall results, respectively, compared to HTM-6.0 while the encoding and decoding time are almost unchanged.

#### Comparison for contributions related to the Merge List Construction

| Proposals | Video 1 | Video 2 | Video only | Synthesized only | Coded & Synthesized | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D0178(CE5.h) | -0.2% | -0.2% | -0.1% | -0.1% | -0.15% | 117.58% | 100.28% |
| D0109(CE5.h related) | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 99.1% | 100.8% |

### Interview prediction using image deformation characeteristics

- JCT3V-D0248 [LG] CE5.h : Inter-view prediction using image deformation characteristics between multi-view images (Cross-Check by JCT3V-D0xxx)

This contribution proposes an inter-view prediction method that considers image deformations in multi-view videos by different view point. The plane objects such as walls and floors look horizontally scaled or sheared in other views. However, the current HTM 6.0 uses traditional block-based translational motion model for inter-view prediction where the deformations should be signalled by residual data. In this contribution, an inter-view prediction method that searches horizontal scaling and shearing parameters with the disparity vector and transmits to decoder. The disparity compensation process of the decoder makes a horizontally scaled and sheared prediction image using the decoded parameters. Proposed method reportedly showed -0.1%, -0.2%, and -0.1% for V1, V2 and video only cases, respectively. In case of for Undo_Dancer test sequence, the largest bitrate savings -1.6%, -1.2%, -0.4% for V1, V2, and video only cases, respectively.

## CE5.h related

### AMVP Candiate List Construction

- JCT3V-D0122 [Samsung] 3D-CE5.h related: AMVP candidate list construction (Cross-Checked by JCT3V-D0249)

In this contribution, it is proposed to fix the number of the candidates in AMVP to two, so we do not have to change the syntax in HEVC for the 3D extension parts. In order to reduce the number of the candidates from three to two, the first option in the proposed method is to remove the inter-view motion vector candidate and then choose two candidates among the spatial and temporal candidates. The second option is to use the inter-view motion vector candidate and then choose one candidate among the spatial and temporal candidates. The results demonstrate that both options have no coding loss and reduce both the encoding and decoding times.

### Interview SAO Process

- JCT3V-D0091 [LG] CE5.h related : Inter-view SAO process in 3DV coding (Cross-Checked by JCT3V-C0279)

In this contribution, SAO process in the 3D HEVC (HTM 6.0) is migrated to that in the current HEVC (HM 9.1). The main modification is that SAO parameters are derived from LCU-based optimization (interleaving mode) rather than picture-based optimization (APS mode). Inter-view SAO process is proposed and modeled on the top of it to confirm the benefit of sharing base view’s SAO parameters on the dependent views through RD-optimization, and reportedly shows 0.3% and 0.3% gains on view 1 and 2, respectively.

#### Comparison for the Interview SAO Process in JCT3V-D0091

| Proposals | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & Synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial migration | 0.3% | -0.4% | -0.3% | 0.1% | 0.2% | -0.1% | 99.8% | 90.8% | 102.8% |
| Interview SAO | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 99.9% | 94.7% | 95.0% |

### Illumination compensation

#### Removal of parsing dependency for illumination compensation

- JCT3V-D0060 [Sharp] 3D-CE5.h related: Removal of parsing dependency for illumination compensation (Cross-Checked by JCT3V-D0274)

In the current 3D-HEVC HTM, illumination compensation (IC) has a parsing problem that decoding an IC utilize flag, or ic_flag needs all motion parameters derivation in CU. This design could decrease CABAC throughput because that motion parameters derivation could stop CABAC stage which should ideally continue to operate independently. This contribution proposes a method to remove the parsing dependency by sending ic_flag independent of ref_idx when merge mode is utilized. The experimental results shows the proposed simplification shows 0.1 % loss in total video.

- JCT3V-D0152 [MediaTek] 3D-CE5.h related: Removal of parsing dependency for illumination compensation (Cross-Checked by JCT3V-D0255)

This contribution presents a method to remove the parsing dependency for illumination compensation in 3D-HEVC. In the current HTM, illumination compensation causes a parsing dependency on the collocated picture. To remove the dependency, two modifications are proposed when both inter-view and inter-time reference pictures are in the reference lists. First, ic_flag is only signalled for a CU with a 2Nx2N AMVP coded PU. Second, ic_flag of a merge coded PU is inherited from the selected merging candidate. It is reported that the proposed method causes on average 0.1% increases in BD-rates for video1 and video2.

#### Simplification for illumination compensation

- JCT3V-D0061 [Sharp] 3D-CE5.h related: Illumination compensation regression improvement and simplification (Cross-Check by JCT3V-D0099)

In the current 3D-HEVC HTM, illumination compensation (IC) utilizes the linear regression model in which parameters are derived by least square error (LSE) algorithm. However these LSE cost is known not robust in real world where a lot of outliner and noise exists. In this contribution, the parameter cost, which is optimized to IC, is added. Additionally we simplify the IC prediction process and IC parameter derivation process by using constant shift operation rather than adaptive shift operation and by restricting its application to 2Nx2N and L0 only.  The experimental results show the proposed regression improvement shows 0.2 % gain in total video. The simplification doesn’t change the coding efficiency. The combination of the regression improvement and the simplification shows 0.2% gain in total video.

- JCT3V-D0062 [Sharp] 3D-CE5.h related: Combination of JCT3V-D0060 and JCT3V-D0061  (Cross-Check by JCT3V-D0275)

This contribution reports combination of illumination contribution proposals, 3D-CE5.h related: Removal of parsing dependency for illumination compensation (JCT-D0060) and 3D-CE5.h related: Removal of parsing dependency for illumination compensation (JCT-D0061). The combination of JCT-D0060 and JCT-D0061 reportedly shows 0.1 % gain in total video while JCT-D0060 and JCT-D0061 show 0.1 % loss and 0.2 % gain in total video respectively. It is reported that the coding efficiency of two proposals is almost orthogonal. The author asserts this combination bring a number of simplification with small gain.

- JCT3V-D0096 [LG] CE5.h The results on simplified illumination compensation (Cross-Check by JCT3V-D0258)

In the 3rd JCT-3V meeting, an offset model for Illumination Compensation(IC) was proposed to simplify the IC process. The offset model requires no multiplication in the parameter estimation/prediction process while retaining comparable coding efficiency with the linear model. In this proposal, results of the simplified IC using the offset model both for texture and depth coding is presented. It has BD-rate change 0.0% compared to current IC model on video only result with 100.3% encoding time, and 98.0% decoding time.

- JCT3V-D0106 [MediaTek] 3D-CE5.h related: On complexity reduction of bi-prediction for illumination compensation (Cross-Check by JCT3V-D0xxx)

In current 3D-HEVC reference software, HTM-6.0, when the two pieces of motion information used for bi-prediction refer a same reference block, encoder/decoder non-normatively changes the inter prediction direction from bi-prediction to uni-prediction to avoid the unnecessary computation caused by the redundant motion compensation process. However, with this non-normative change, HTM-6.0 could generate a bitstream which does not conform to the 3D-HEVC text when illumination compensation (IC) is enabled due to the unequal clipping operations between uni-prediction and bi-prediction. To resolve this issue, in this contribution, it is proposed to normatively align the operations between uni-prediction and bi-prediction when IC is enabled. Experimental results reportedly show that the proposed method causes no coding efficiency loss while the unnecessary computation of bi-prediction can be removed regardless of whether IC is enabled.

- JCT3V-D0114 [Samsung] 3D-CE5.h related: Simplification of illumination compensation (Cross-Check by JCT3V-D0xxx)

This contribution proposes four methods for the illumination compensation (IC) in 3D-HEVC. Firstly, the scale factor derivation in the IC seems to have relatively high complexity because there are many multiplications. Therefore, it is proposed to remove multiplicative terms from the scale factor derivation. Secondly, it is proposed to apply the IC to only 2Nx2N partition to reduce the complexity. Thirdly, when deriving the top-left sample of the reference block for IC parameter estimation, if the disparity vector points a factional sample position, left or above integer sample position is always selected. Thus, in order to improve coding efficiency, it is proposed to point the nearest integer sample position by applying the rounding offset to the top-left sample derivation. Finally, after IC parameter derivation, some combinations of the scale factor and the offset, such as the case where the scale factor is equal to 1 and the offset is equal to 0, do not need to perform the IC. Therefore, it is proposed to selectively perform IC by checking derived IC parameters. The proposed methods reportedly show an average of 0.2% and 0.6% bit-saving for video 1 and video 2, respectively, 0.2% bit-saving for video PSNR vs. video bitrate, and 0.1% bit-saving for video PSNR vs. total bitrate and synth PSNR vs. total bitrate.

### Others

- JCT3V-D0135 [LG] 3D-CE5.h related: Unification of disparity vector rounding (Cross-Check by JCT3V-D0128)

This proposal unifies rounding operation of disparity vector with respect to rounding offset. The modification consist of adding rounding offset for 3 parts; DoNBDV, B-VSP, IC process. Experimental results showed an average coding gain of 0.1% compared to HTM-6.0.

- JCT3V-D0145 [MediaTek] 3D-CE5.h related: Unification of the reference view for inter-view motion prediction and inter-view residual prediction (Cross-Check by JCT3V-D0254)

This contribution proposes to unify the reference view for the inter-view motion prediction and inter-view residual prediction by providing a syntax to identify the picture of reference view from the reference lists and prohibiting the reference view to be changed on every slice in a same picture. The proposed method has benefits for the management of decoded motion and residual data, and provides more flexibility for reference view selection, with little impact on the coding efficiency.

# Closing Remarks

It is recommended the CE be continued and investigate proposed designs in further details.