| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0026 |
| --- | --- |

| Title: | CE6 Summary Report: Depth intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philipp Merkle | Email: | philipp.merkle@hhi.fraunhofer.de |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document summarizes the proposals and cross check results of CE6, including tools under test according to JCT3V-C1106 as well as related input contributions.

# Introduction

In this document a summary of proposals and cross check results for CE6 is reported. The goal of this Core Experiment (CE) is to investigate tools for depth intra coding, covering AVC-based proposals under sub-experiment CE6.A and HEVC-based proposals under sub-experiment CE6.H. For the 4th JCT-3V meeting all tools under test and related contributions are in sub-experiment CE6.H.  The proposed tools are evaluated in terms of coding efficiency and computational complexity. The objective of tools under test in CE6.H was simplification, harmonization and improvement of existing depth intra coding methods, such as DMM, region boundary chain, and SDC.

Participants in CE6 are as follows:

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| HHI | P. Merkle | philipp.merkle@hhi.fraunhofer.de | X | X |
| Aachen University | F. Jäger | jaeger@ient.rwth-aachen.de | X | X |
| LG | J. Heo | jin78.heo@lge.com | X | X |
| Samsung | I. Lim J.-Y. Lee | ilsoon.lim@samsung.com jinyoung79.lee@samsung.com | X | X |
| MediaTek | K. Zhang J. An J.-L. Lin Y.-W. Chen Y.-L. Chang Y.-W. Huang | Kai.Zhang@mediatek.com Jicheng.An@mediatek.com jl.lin@mediatek.com yiwen.chen@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | X | X |
| Ghent University-iMinds | S. van Leuven | sebastiaan.vanleuven@ugent.be |  | X |
| GIST | Y. Song Y.-S. Ho | ysong@gist.ac.kr hoyo@gist.ac.kr |  | X |
| Hisilicon | X. Zheng | zhengxiaozhen@huawei.com |  | X |
| Zhejiang University | P. Lu | yiqiu8932@gmail.com |  |  |
| Qualcomm | X. Zhao | xinzhao@qti.qualcomm.com |  | X |

# List of input documents

## CE6.H proposals

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Aachen Univ. LG | JCT3V-D0032 | CE6.H: Results on CABAC Context Reduction for Simplified Depth Coding (SDC) | Proposal |
| Ghent Univ. | JCT3V-D0037 | CE6.H: Cross check of CABAC Context Reduction for SDC (JCT3V-D0032) | Cross check |
| LG | JCT3V-D0141 | 3D-CE6.h: Concatenate binarization for residual index coding | Proposal |
| Samsung | JCT3V-D0157 | 3D-CE6.h: Cross check of LG's Concatenate binarization for residual index coding (JCT3V-D0141) | Cross check |
| HHI | JCT3V-D0035 | CE6.H: Results on DLT for DMM deltaDC coding | Proposal |
| Qualcomm | JCT3V-D0203 | 3D-CE6.h related: Cross check on DLT for DMM deltaDC coding (JCT3V-D0035) | Cross check |
| Samsung | JCT3V-D0147 | 3D-CE6.h: Results on Signalling of Depth Lookup Table | Proposal |
| LG | JCT3V-D0154 | 3D-CE6.h Cross check of Signalling of Depth Lookup Table (JCT3V-D0147) | Cross check |
| MediaTek | JCT3V-D0151 | 3D-CE6.h results on an efficient coding method for DLT in 3DVC | Proposal |
| HHI | JCT3V-D0269 | CE6.H: Cross check of efficient DLT coding in SPS (JCT3V-D0151) | Cross check |

## CE6.H related proposals

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Aachen Univ. | JCT3V-D0033 | CE6.H related: Results on Complexity Reduction for Simplified Depth Coding (SDC) | Proposal |
| MediaTek | JCT3V-D0264 | Ce6.H related: cross-check of results on complexity reduction for simplified depth coding (JCT3V-D0033) | Cross check |
| Aachen Univ. | JCT3V-D0034 | CE6.H related: Results on Fixes for Context Initialization and Unary Coding for SDC | Proposal |
| Qualcomm | JCT3V-D0205 | 3D-CE6.h related: Cross check on fixes for context initialization and unary coding for SDC (JCT3V-D0034) | Cross check |
| HHI | JCT3V-D0036 | CE6.H related: Results on aligning 3D-HTM software with 3D-HEVC draft specification for Wedgelet pattern construction | Proposal |
| Aachen Univ. | JCT3V-D0040 | CE6.H related: Cross check of aligning 3D-HTM software with 3D-HEVC draft specification for Wedgelet pattern construction (D0036) | Cross check |
| Aachen Univ. | JCT3V-D0054 | CE6.H related: Results on Updating Mechanism for Coding of Depth Lookup Table (Delta-DLT) | Proposal |
| Ghent Univ. | JCT3V-D0055 | CE6.H related: Cross Check of Updating Mechanism for Coding of Depth Lookup Table (Delta-DLT) (JCT3V-D0054) | Cross check |
| LG | JCT3V-D0066 | CE6.h related: Removal of DC from SDC Mode | Proposal |
| MediaTek | JCT3V-D0263 | 3D-CE6.h related: Crosscheck on Removal of dc in SDC from LG | Cross check |
| LG | JCT3V-D0067 | CE6.h related: Removal of Overlap between DMM1 and DMM3 | Proposal |
| Hisilicon | JCT3V-D0285 | CE6.h releated: Crosscheck on Removal of Overlap between DMM1 and DMM3 (D0067) | Cross check |
| Zhejiang Univ. | JCT3V-D0076 | CE6.H related: Results on Fixes for Region Boundary Chain Coding in 3D-HEVC draft specification | Proposal |
|  |  |  | Cross check |
| Zhejiang Univ. | JCT3V-D0077 | CE6.H related: Results on Modified Binarization for Region Boundary Chain Coding | Proposal |
|  |  |  | Cross check |
| MediaTek | JCT3V-D0110 | 3D-CE6.h related: Sample-based simplified depth coding | Proposal |
| Sharp | JCT3V-D0257 | 3D-CE6.h related: cross-check of Sample-based simplified depth coding (JCT3V-D0110) | Cross check |
| Intel | JCT3V-D0281 | 3D-CE6.h related: Crosscheck on Sample-based SDC from MTK (JCT3V-D0110) | Cross check |
| Samsung | JCT3V-D0123 | 3D-CE6.h related: DMM simplification and signalling | Proposal |
| HHI | JCT3V-D0271 | CE6.H related: Cross check of DMM simplification and signalling (JCT3V-D0123) | Cross check |
| Samsung | JCT3V-D0124 | 3D-CE6.h related: SDC with DLT in intra slices | Proposal |
|  |  |  | Cross check |
| LG | JCT3V-D0136 | 3D-CE6.h related: Improvement of SDC coding | Proposal |
| Samsung | JCT3V-D0159 | 3D-CE6.h related: Cross check of LG's Improvement of SDC coding (JCT3V-D0136) | Cross check |
| LG | JCT3V-D0142 | 3D-CE6.h related: Improved DLT coding method | Proposal |
| Samsung | JCT3V-D0133 | 3D-CE6.h related: Cross check of Improved DLT coding method (JCT3V-D0142) | Cross check |
| Samsung LG | JCT3V-D0150 | 3D-CE6.h related: Results on Signalling and DPCM coding of Depth Lookup Table | Proposal |
| Intel | JCT3V-D0282 | 3D-CE6.h related: Crosscheck on Signalling and DPCM coding of Depth Lookup Table from SamSung (JCT3V-D0150) | Cross check |
| MediaTek | JCT3V-D0153 | 3D-CE6.h related: A flexible DLT signaling method in 3D-HEVC | Proposal |
| Hisilicon | JCT3V-D0290 | CE6.h releated: Crosscheck on flexible DLT signaling method in 3D-HEVC | Cross check |
| MediaTek | JCT3V-D0155 | 3D-CE6.h related: Direct simplified depth coding in 3D-HEVC | Proposal |
| LG | JCT3V-D0100 | Crosscheck of MTK's CE6.h related proposal | Cross check |
| LG | JCT3V-D0163 | CE6.h related SDC with WVSO metric | Proposal |
|  |  |  | Cross check |
| GIST | JCT3V-D0169 | CE6.h related: Results on disabling of DMM 4 | Proposal |
|  |  |  | Cross check |
| ZTE | JCT3V-D0172 | 3D-CE6.h related: Differential coding method for DLT in 3D-HEVC | Proposal |
|  |  |  | Cross check |
| Hisilicon | JCT3V-D0173 | CE6.h related: Removal of division operation for DMM and SDC | Proposal |
| Qualcomm | JCT3V-D0204 | 3D-CE6.h related: Cross check on removal of division operation for DMM and SDC (JCT3V-D0173) | Cross check |
| Hisilicon | JCT3V-D0174 | CE6.h related: Extension of reference samples sub-sampling to all block-sizes in DMM and SDC | Proposal |
| Intel | JCT3V-D0283 | 3D-CE6.h related: Crosscheck on Extension of reference samples sub-sampling to all block-sizes in DMM and SDC (JCT3V-D0174) | Cross check |
| SCU Hisilicon | JCT3V-D0175 | 3D-CE6.h related: Fast intra prediction mode selection for intra depth map coding | Proposal |
| Intel | JCT3V-D0284 | 3D-CE6.h related: Crosscheck on Fast intra prediction mode selection for intra depth map coding (JCT3V-D0175) | Cross check |
| Qualcomm | JCT3V-D0182 | CE6.h related: On Signaling of DLT for depth coding | Proposal |
| Aachen Univ. | JCT3V-D0273 | CE6.H related: Cross check of Signaling of DLT for depth coding (D0182) | Cross check |
| Qualcomm | JCT3V-D0183 | CE6.h related: Simplified DC predictor for depth intra modes | Proposal |
| HHI | JCT3V-D0270 | CE6.H related: Cross check of simplified DC predictor for depth intra modes (JCT3V-D0183) | Cross check |
| Qualcomm | JCT3V-D0184 | CE6.h related: Wedgelet modes unification | Proposal |
| Hisilicon | JCT3V-D0288 | CE6.h releated: Crosscheck on Wedgelet modes unification | Cross check |
| Intel | JCT3V-D0190 | CE6.h related: SDC without per-pixel prediction | Proposal |
| MediaTek | JCT3V-D0266 | 3D-CE6.h related: Crosscheck of Intel's JCT3V-D0190 on simplified SDC mode | Cross check |
| Intel | JCT3V-D0193 | CE6.h related: Clean up for 64x64 SDC | Proposal |
| Hisilicon | JCT3V-D0289 | CE6.h releated: Crosscheck on Clean up for 64x64 SDC | Cross check |
| PKU Qualcomm | JCT3V-D0195 | CE6.h related: Unification of new intra modes in 3D-HEVC | Proposal |
| LG | JCT3V-D0299 | CE6.h related: Cross check on Unification of new intra modes in 3D-HEVC | Cross check |

# Summary of proposals

All the proposals are related to the specific depth intra coding methods SDC, DLT, DMM, and RBC (region boundary chain). Contributions marked by a ‘*’ combine different methods and therefore appear in more than one category. Table entries in parentheses indicate optional parts of a proposal.

## All depth intra modes

- Three contributions propose simplifications related to the prediction of partition DC value:

| Proposal | SDC | DMM | RBC |
| --- | --- | --- | --- |
| JCT3V-D0173: Remove division | X | X |  |
| JCT3V-D0174: Sub-sampling for all block-sizes | X | X |  |
| JCT3V-D0183: only 2-4 reference samples | X | X | X |

- One contribution (JCT3V-D0195) proposes unifying the syntax of depth intra mode signaling for SDC, DMM, and RBC.

## SDC

- Two contributions are about the tool under test on “Context reduction and binarization for SDC”: JCT3V-D0032 proposes a context reduction and JCT3V-D0141 a concatenate binarization scheme for residual indices.

- Three contributions (JCT3V-D0110, JCT3V-D0136, JCT3V-D0155*) propose fixing a bug related to signal reconstruction for the Planar mode in SDC. It might be necessary to consider this fix for some proposals related to SDC, as it has some impact on the coding performance and behaviour.

- Three contributions (JCT3V-D0033, JCT3V-D0155*, JCT3V-D0190) propose simplifications and complexity reductions related to the prediction of partition DC values in SDC.

- One contribution (JCT3V-D0066) proposes removing the DC mode from SDC.

- One contribution (JCT3V-D0124*) proposes restricting SDC to intra slices.

- One contribution (JCT3V-D0193) proposes a clean-up for undefined decoder behaviour of 64x64 SDC.

## DLT

A total of nine contributions are about DLT modifications, including two contributions (JCT3V-D0147, JCT3V-D0151) related to the tool under test on “Signaling and binarization of DLT”:

| Proposal | signaling in |  |  | signaling/binarization of DLT values | prediction/update of DLT values |
| --- | --- | --- | --- | --- | --- |
|  | SPS | slice hdr | VPS |  |  |
| JCT3V-D0147 |  | X |  |  |  |
| JCT3V-D0151 | (X) | X |  | RCBM |  |
| JCT3V-D0054 |  | X |  | (RCBM) | delta-DLT updates for P/B slices |
| JCT3V-D0124* | (X) | X |  |  |  |
| JCT3V-D0142 |  |  |  | DPCM |  |
| JCT3V-D0150 |  | X |  | DPCM |  |
| JCT3V-D0153 | X | X |  |  | DLT reference index in P/B slices |
| JCT3V-D0172 | X |  |  | differential coding |  |
| JCT3V-D0182 |  |  | X | differential coding | inter-view prediction of DLT vallues |

## DMM

- One contribution (JCT3V-D0035) is about the tool under test on “DLT for DMM deltaDC coding”, proposing to use DLT indices instead of depth values for delta DCs in DMM.

- Four contributions propose simplifications related to the DMM intra modes:

| Proposal | DMM1 | DMM2 | DMM3 | DMM4 |
| --- | --- | --- | --- | --- |
| JCT3V-D0067 |  |  | modify |  |
| JCT3V-D0123 |  | (remove) | remove | remove |
| JCT3V-D0169 |  |  |  | remove |
| JCT3V-D0184 | modify | (remove) | modify |  |

## RBC

- One contribution (JCT3V-D0077) proposes modifying the binarization for RBC.

## Other

- Two contributions are about encoder optimizations: JCT3V-D0163 proposes WVSO for SDC and JCT3V-D0175 proposes a fast mode selection for DMM and RBC.

- Three contributions are about aligning software and draft specification:  JCT3V-D0034 for SDC, JCT3V-D0036 for DMM and JCT3V-D0076 for RBC.

# Performance evaluation

Table 1. Result for 3-view scenario under CTC

|  |  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JCT3V-D0032 | T1 | 0.0% | -0.1% | -0.1% | 100.2% | 100.3% | n/a |
| JCT3V-D0141 | T2 | 0.0% | -0.1% | -0.1% | 99.4% | 89.6% | 101.0% |
|  | T3 | 0.0% | -0.1% | -0.2% | 100.2% | 99.2% | 96.0% |
| JCT3V-D0035 | T1 | 0.0% | 0.0% | -0.1% | 100.0% | 99.0% | 99.8% |
|  | T2 | 0.0% | 0.0% | -0.2% | 100.0% | 99.7% | 100.1% |
| JCT3V-D0147 | T1 | 0.0% | 0.0% | 0.0% | 100.4% | 100.3% | n/a |
|  | T2 | 0.0% | 4.7% | 4.7% | 99.4% | 100.0% | n/a |
| JCT3V-D0151 | T3 | 0.0% | 0.0% | 0.0% | 100.0% | 99.1% | 99.5% |
|  | T4 | 0.0% | 0.0% | 0.0% | 99.9% | 100.5% | 100.1% |
|  | T5 | 0.0% | 1.6% | 1.6% | 99.7% | 99.0% | 98.5% |
| JCT3V-D0033 |  | 0,0% | 0,0% | 0,0% | 100,1% | 98,9% | n/a |
| JCT3V-D0034 | 6.1* | 0,0% | 0,0% | -0,1% | 100,2% | 100,0% | n/a |
| JCT3V-D0036 | 6.1 | 0.0% | 0.0% | -0.1% | 99.8% | 98.4% | 99.7% |
| JCT3V-D0054 | A-6.1 | 0.0% | 0.2% | 0.2% | 100.0% | 101.2% | n/a |
|  | B-6.1 | 0.0% | 0.1% | 0.1% | 100.1% | 100.4% | n/a |
| JCT3V-D0066 |  | 0.0% | 0.0% | -0.1% | 99.7% | 99.9% | n/a |
| JCT3V-D0067 |  | 0.0% | 0.0% | -0.1% | 101.3% | 100.0% | 101.2% |
| JCT3V-D0077 |  | 0.0% | 0.0% | -0.1% | 89.6% | 100.5% | n/a |
| JCT3V-D0110 |  | 0.0% | -0.2% | -0.2% | 99.4% | 100.8% | 98.1% |
| JCT3V-D0123 | T1 | 0.0% | 0.0% | -0.1% | 96.8% | 99.1% | 100.0% |
|  | T2 | 0.0% | 0.0% | 0.0% | 94.7% | 98.4% | 100.0% |
| JCT3V-D0124 | T1 | 0.0% | 0.1% | 0.0% | 99.0% | 98.6% | 100.0% |
|  | T2 | 0.0% | 0.1% | -0.1% | 99.1% | 99.8% | 100.0% |
| JCT3V-D0136 |  | 0.0% | -0.2% | -0.2% | 101.6% | 105.6% | 108.4% |
| JCT3V-D0142 | old sheet | 0.0% | 0.0% | 0.0% | 84.7% | 81.8% | 75.4% |
| JCT3V-D0150 |  | 0.0% | 0.0% | 0.0% | 99.8% | 100.0% | n/a |
| JCT3V-D0153 |  | 0.0% | 0.0% | 0.0% | 100.3% | 100.6% | 100.6% |
| JCT3V-D0155 |  | 0.0% | 0.0% | -0.2% | 100.4% | 97.9% | 100.1% |
| JCT3V-D0163 |  | -0.1% | 0.0% | -0.2% | 99.6% | 104.0% | 110.4% |
| JCT3V-D0169 |  | 0.0% | 0.0% | -0.2% | 99.6% | 100.2% | 100.5% |
| JCT3V-D0172 |  | 0.0% | 0.0% | 0.0% | 97.9% | 101.9% | 93.4% |
| JCT3V-D0173 | S1 | 0.0% | 0.0% | -0.1% | 100.0% | 102.4% | 101.7% |
|  | S2 | 0.0% | 0.0% | 0.0% | 100.2% | 100.8% | 101.0% |
| JCT3V-D0174 | SDC | 0.0% | 0.0% | 0.0% | 99.9% | 98.5% | 99.7% |
|  | SDC DMM | 0.0% | 0.0% | -0.1% | 100.1% | 97.4% | 100.3% |
| JCT3V-D0175 |  | 0.0% | 0.0% | -0.1% | 97.7% | 100.8% | 101.4% |
| JCT3V-D0182 |  | 0.0% | 0.0% | 0.0% | 98% | 98% | 97% |
| JCT3V-D0183 |  | 0.0% | -0.1% | -0.2% | 98% | 95% | 96% |
| JCT3V-D0184 |  | 0.0% | 0.0% | 0.0% | 97% | 95% | 92% |
| JCT3V-D0190 | 6.1 | 0.0% | 0.0% | 0.0% | 97.6% | 96.6% | 93.0% |
| JCT3V-D0193 | S1-6.1 | -0.1% | 0.1% | 0.0% | 98.2% | 96.6% | 93.9% |
|  | S3-6.1 | 0.0% | 0.0% | -0.1% | 99.9% | 98.3% | 97.2% |
| JCT3V-D0195 |  | 0.0% | -0.1% | -0.1% | 102% | 102% | 99% |

Table 2. Result for 3-view scenario under all-intra configuration

|  |  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JCT3V-D0032 | T1 | 0.0% | 0.1% | 0.1% | 99.4% | 97.4% | 98.5% |
| JCT3V-D0141 | T2 | 0.0% | 0.0% | -0.1% | 99.6% | 99.1% | 99.2% |
|  | T3 | 0.0% | 0.0% | 0.0% | 99.5% | 95.3% | 96.9% |
| JCT3V-D0035 | T1 | 0.0% | 0.0% | 0.0% | 100.2% | 99.5% | 100.2% |
|  | T2 | 0.0% | 0.0% | 0.0% | 99.8% | 99.8% | 100.1% |
| JCT3V-D0147 | T1/2 | 0.0% | 0.3% | 0.3% | 99.5% | 97.6% | n/a |
| JCT3V-D0151 | T3 | 0.0% | 0.0% | 0.0% | 100.0% | 99.7% | 100.3% |
|  | T4/5 | 0.0% | 0.1% | 0.0% | 99.6% | 100.5% | 99.5% |
| JCT3V-D0033 |  | 0,0% | 0,0% | 0,0% | 99,3% | 98,7% | n/a |
| JCT3V-D0034 | 6.1* | 0,0% | 0,0% | 0,1% | 100,3% | 100,5% | n/a |
| JCT3V-D0036 | 6.1 | 0.0% | 0.0% | 0.0% | 100.2% | 97.8% | 99.0% |
| JCT3V-D0054 | A-6.1 | 0.0% | 0.0% | 0.0% | 99.8% | 100.4% | n/a |
|  | B-6.1 | 0.0% | 0.0% | 0.0% | 99.7% | 100.4% | n/a |
| JCT3V-D0066 |  | 0.0% | 0.0% | -0.1% | 106.7% | 99.8% | n/a |
| JCT3V-D0067 |  | 0.0% | 0.0% | 0.0% | 94.0% | 100.0% | 75.6% |
| JCT3V-D0077 |  | 0.0% | 0.0% | 0.0% | 98.7% | 99.0% | n/a |
| JCT3V-D0110 |  | 0.0% | -0.2% | -0.1% | 100.6% | 99.7% | 99.3% |
| JCT3V-D0123 | T1 | 0.0% | 0.0% | 0.2% | 81.5% | 98.4% | 100.0% |
|  | T2 | 0.0% | 0.1% | 0.3% | 74.0% | 98.7% | 100.0% |
| JCT3V-D0124 | T1 | n/a | n/a | n/a | n/a | n/a | n/a |
|  | T2 | n/a | n/a | n/a | n/a | n/a | n/a |
| JCT3V-D0136 |  | 0.0% | -0.2% | -0.1% | 101.7% | 99.5% | 97.6% |
| JCT3V-D0142 |  | n/a | n/a | n/a | n/a | n/a | n/a |
| JCT3V-D0150 |  | 0.0% | 0.1% | 0.1% | 98.6% | 97.2% | n/a |
| JCT3V-D0153 |  | n/a | n/a | n/a | n/a | n/a | n/a |
| JCT3V-D0155 |  | 0.0% | -0.1% | -0.1% | 99.8% | 97.2% | 99.6% |
| JCT3V-D0163 |  | 0.0% | 0.7% | -0.2% | 94.7% | 104.6% | 99.0% |
| JCT3V-D0169 |  | n/a | n/a | n/a | n/a | n/a | n/a |
| JCT3V-D0172 |  | n/a | n/a | n/a | n/a | n/a | n/a |
| JCT3V-D0173 | S1 | 0.0% | 0.0% | 0.0% | 99.7% | 101.8% | 99.6% |
|  | S2 | 0.0% | 0.0% | 0.0% | 100.0% | 102.4% | 100.5% |
| JCT3V-D0174 | SDC | 0.0% | 0.0% | 0.0% | 99.9% | 100.9% | 99.6% |
|  | SDC DMM | 0.0% | 0.0% | 0.0% | 100.2% | 98.9% | 99.3% |
| JCT3V-D0175 |  | 0.0% | 0.0% | 0.0% | 76.5% | 99.6% | 100.7% |
| JCT3V-D0182 |  | 0.0% | 0.0% | 0.0% | 95% | 97% | 98% |
| JCT3V-D0183 |  | 0.0% | -0.1% | -0.1% | 99% | 100% | 100% |
| JCT3V-D0184 |  | 0.0% | 0.0% | 0.3% | 103% | 99% | 97% |
| JCT3V-D0190 | 6.1 | 0.0% | 0.0% | 0.0% | 92.1% | 90.4% | 88.5% |
| JCT3V-D0193 | S1-6.1 | 0.0% | 0.6% | 0.3% | 97.9% | n/a | 96.3% |
|  | S3-6.1 | 0.0% | 0.0% | 0.0% | 98.5% | 94.3% | 92.9% |
| JCT3V-D0195 |  | 0.0% | -0.1% | 0.1% | 101% | 98% | 92% |