| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0035 |
| --- | --- |

| Title: | CE6.H: Results on DLT for DMM deltaDC coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Karsten Müller, Thomas Wiegand | Email: | philipp.merkle@hhi.fraunhofer.de, karsten.mueller@hhi.fraunhofer.de, thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results on the tool under test in CE6.H for studying possible further improvements of deltaDC coding for DMM by combining it with DLT.

# Introduction

The additional depth intra coding modes in 3DV-HTM, referred to as DMM, are adapted to the specific characteristics of depth maps by applying non-rectangular block partitions for approximating the signal of a depth block containing an edge. All four modes use a method for residual adaptation in the pixel domain, referred to as deltaDC method. This method consists of estimating the optimum offset to the predicted constant value for each of the two partitions of a DMM block by a VSO-based minimum distortion search. For reconstructing the signal of the block, these offset values are transmitted in the bit stream.

Furthermore, a depth lookup table (DLT) is used for the simplified depth coding (SDC) scheme in 3DV-HTM. At the encoder the values of the DLT are derived by analyzing a pre-defined number of frames from the input video sequence. Consequently, each DLT index maps to a valid depth values based on the original uncompressed depth map. The DLT values are signaled to the decoder in the SPS. Consequently, for SDC coding partition offset values are estimated and signaled to the decoder as DLT indices instead of full precision depth values.

The motivation for the proposed tool under test in CE6 is to study possible further improvements of deltaDC coding for DMM described in by combining it with DLT.

# Proposed method

According to section 3.2 of the CE6 description in JCT3V-C1106 [1], combining deltaDC coding of DMM with DLT is studied in two tests.

## Test1:

The representation of partition offset values transmitted in the bitstream is changed from full depth precision sample values to DLT indices. Consequently these indices need to be re-mapped to depth sample values for prediction. The signaling of partition offset values is not changed.

## Test2:

In addition to the modifications for Test1, the signaling of partition offset values is changed to the method used within SDC, which basically means replacing the contexts and binarization scheme.

# Experimental results

According to the CE6 description in JCT3V-C1106 [1] and the common test conditions in JCT3V-C1100 [2], the proposed methods are evaluated for random access (CTC) and all-intra configuration.

## Test1:

Table 1. Test1 results for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 100.2% | 97.7% | 100.1% |
| Kendo | 0.0% | 0.0% | -0.1% | 99.8% | 100.3% | 98.5% |
| Newspapercc | 0.1% | 0.1% | -0.2% | 99.9% | 97.3% | 99.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 100.3% | 100.4% | 100.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 99.9% | 100.4% | 100.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 99.7% | 100.1% | 100.8% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 100.2% | 96.6% | 99.7% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 99.9% | 98.4% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 100.0% | 99.4% | 100.1% |
| average | 0.0% | 0.0% | -0.1% | 100.0% | 99.0% | 99.8% |

Table 2. Test1 results for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.2% | 100.5% | 98.9% | 100.4% |
| Kendo | 0.0% | 0.0% | 0.2% | 100.1% | 98.9% | 99.9% |
| Newspapercc | 0.0% | 0.0% | -0.4% | 100.0% | 100.3% | 99.9% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 100.0% | 99.7% | 99.2% |
| PoznanHall2 | 0.0% | 0.0% | 0.1% | 99.9% | 97.0% | 101.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 100.0% | 100.8% | 100.3% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 100.6% | 100.6% | 100.4% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 100.2% | 99.4% | 100.1% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 100.1% | 99.5% | 100.2% |
| average | 0.0% | 0.0% | 0.0% | 100.2% | 99.5% | 100.2% |

## Test2:

Table 3. Test2 results for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | 100.2% | 96.0% | 99.2% |
| Kendo | 0.0% | -0.1% | -0.1% | 99.8% | 100.4% | 98.6% |
| Newspapercc | 0.0% | 0.0% | -0.2% | 100.0% | 97.4% | 99.6% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 100.3% | 100.4% | 99.9% |
| PoznanHall2 | -0.3% | -0.3% | -0.3% | 100.0% | 101.2% | 102.0% |
| PoznanStreet | 0.1% | 0.0% | 0.1% | 99.7% | 100.3% | 101.1% |
| UndoDancer | 0.0% | 0.0% | -0.6% | 100.1% | 102.2% | 100.5% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 100.0% | 97.9% | 99.1% |
| 1920x1088 | -0.1% | -0.1% | -0.2% | 100.0% | 101.0% | 100.9% |
| average | 0.0% | 0.0% | -0.2% | 100.0% | 99.7% | 100.1% |

Table 4. Test2 results for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.2% | 99.8% | 98.5% | 100.9% |
| Kendo | 0.0% | 0.0% | 0.2% | 100.0% | 99.5% | 99.9% |
| Newspapercc | 0.0% | 0.0% | -0.3% | 100.0% | 101.0% | 100.1% |
| GhostTownFly | 0.0% | 0.0% | -0.1% | 99.6% | 100.4% | 99.3% |
| PoznanHall2 | 0.0% | 0.0% | 0.2% | 99.7% | 97.6% | 99.9% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 99.5% | 101.3% | 100.3% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 100.1% | 100.5% | 100.3% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 99.9% | 99.7% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 99.7% | 100.0% | 99.9% |
| average | 0.0% | 0.0% | 0.0% | 99.8% | 99.8% | 100.1% |

# Draft specification

The proposed methods require changing the specification of the “3D-HEVC Test Model 3” in JCT3V-C1005 [3].

## Test1:

The method described in section 2.1 requires the following changes to the specification in section Error: Reference source not found:

…

For X being replaced by 0 and 1, the values of the prediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ] 
numNeigh += 1

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ] 
numNeigh += 1

  - The variables predDcVal specifying the predicted constant partition values for partition X is derived as follows.

      - predDcVal = ( numNeigh ! = 0 ) ? ( sumNeigh / numNeigh ) : ( 1 << ( BitDepthY  − 1 ) )

  - The variable dcOffset dcPartition specifying the DC offset of partition X is derived as follows.

    - If dcOffsetAvailFlag is equal to 1, the following applies:

- dcOffsetScale = intraChainFlag ? Clip3( 1, ( 1 << BitDepthY ) − 1, Round( 2 (QP’Y /10)- 2 ) ) : 1

- dcOffset = dcOffset * dcOffsetScale

      - If intraChainFlag is equal to 1, the following applies:

- dcPartition = predDcVal + (dcOffset * Clip3( 1, ( 1 << BitDepthY ) − 1, Round( 2 (QP’Y /10)- 2 ) ) )

      - Otherwise (intraChainFlag is equal to 0), the following applies:

        - If dlt_flag[ nuh_layer_id ] is equal to 1, the following applies:

- dcPartition = Idx2DepthValue[ DepthValue2Idx[ predDcVal ] + dcOffset ]

        - Otherwise (dlt_flag[ nuh_layer_id ] is equal to 0), the following applies:

- dcPartition = predDcVal + dcOffset

    - Otherwise ( dcOffsetAvailFlag is equal to 0), dcOffset dcPartition is set equal to 0 predDcVal.

  - The predicted sample values predSamples are derived as follows for x = 0..nT−1 and for y = 0..nT−1.

    - When partitionPattern[ x ][ y ] is equal to X, the following applies.

        - predSamples[ x ][ y ] = predDcVal + dcOffset dcPartition

…

## Test2:

The method described in section 2.2 requires the following additional changes to the specification in sections H.7.3.9.1, H.9.3.1, and H.9.3.2:

| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| --- | --- |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + j ] ) |  |
| dmm_dc_1_abssign_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + j ] != 0 ) |  |
| dmm_dc_1_sign_flagabs[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abssign_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + j ] != 0 ) |  |
| dmm_dc_2_sign_flagabs[ x0 + i ][ y0 + j ] | ae(v) |

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | dmm_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_predtex_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_1_abs dmm_dc_2_abs |  | 0 | 1 | 2 |
|  | edge_code | Error: Reference source not found | 0 | 1 | 2 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | mvp_l0_idx mpv_l1_idx | Error: Reference source not found |  | 0..1 | 2..3 |
|  | sdc_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag dmm_dc_1_sign_flag dmm_dc_2_sign_flag |  | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 dmm_dc_1_abs dmm_dc_2_abs | Table Error: Reference source not found-4 | 0..19 | 20..39 | 40..59 |
|  | sdc_pred_mode | Error: Reference source not found | 0..2 | 3..5 | 6..8 |

Table Error: Reference source not found-2 – Values of variable initValue for dmm_dc_1_abs and dmm_dc_2_abs ctxIdx

| Initialization variable | dmm_dc_1_abs dmm_dc_2_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-3 – Values of variable initValue for sdc_residual_sign_flag, dmm_dc_1_sign_flag, and dmm_dc_2_sign_flag ctxIdx

| Initialization variable | sdc_residual_sign_flag dmm_dc_1_sign_flag dmm_dc_2_sign_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 255 | 255 | 0 | 252 | 55 | 0 |

Table Error: Reference source not found-4 – Values of variable initValue for sdc_residual_abs_minus1, dmm_dc_1_abs, and dmm_dc_2_abs ctxIdx

| Initialization variable | sdc_residual_abs_minus1 dmm_dc_1_abs dmm_dc_2_abs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 255 | 255 | 64 | 0 | 2 | 0 | 64 | 0 | 6 | 0 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 67 | 0 | 8 | 0 | 61 | 0 | 7 | 0 | 47 | 0 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 243 | 255 | 12 | 0 | 14 | 0 | 33 | 0 | 243 | 255 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 2 | 0 | 66 | 0 | 0 | 0 | 63 | 0 | 1 | 0 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| initValue | 12 | 0 | 50 | 0 | 14 | 0 | 27 | 0 | 0 | 0 |
|  | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 239 | 255 | 14 | 9 | 27 | 0 | 0 | 0 | 239 | 255 |

Table Error: Reference source not found-5 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| dmm_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| dmm_mode | 0 | FL, cMax = 7 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| wedge_full_tab_idx | 0 | FL, cMax = wedgeFullTabIdxBits[ log2PbSize ] (defined in Error: Reference source not found) | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| wedge_predtex_tab_idx | 0 | FL, cMax = wedgePredTexTabIdxBits[ log2PbSize ] (defined in Error: Reference source not found) | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| dmm_delta_end_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| dmm_delta_end_abs_minus1 | 0 | FL, cMax = 3 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| dmm_delta_end_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | UEG0 [Ed. (GT) To be specified] | 0 |  | 0 |
|  | 1 |  | 0 |  | 1 |
|  | 2 |  | 0 |  | 2 |
| dmm_dc_1_sign_flag dmm_dc_2_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| edge_code | 0 | TU, cMax = 6 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| ic_flag | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 2 |  | 0 | Error: Reference source not found | 1 |
| mvp_l0_idx mvp_l1_idx | 1 | FL, cMax = 1 + iv_mv_pred_flag[ nuh_layer_id ] | 1 | Error: Reference source not found | 0 |
|  | 2 |  | 1 | Error: Reference source not found | 2 |
| sdc_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| sdc_residual_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 2 |
|  | 2 |  | 0 | Error: Reference source not found | 4 |
| sdc_residual_sign_flag dmm_dc_1_sign_flag dmm_dc_2_sign_flag | 0 | FL, cMax = 1 | 0 |  | 0 |
|  | 1 |  | 0 |  | 2 |
|  | 2 |  | 0 |  | 4 |
| sdc_residual_abs_minus1 dmm_dc_1_abs dmm_dc_2_abs | 0 | FL, cMax = dltFlag ? Floor( Log2( num_depth_values_in_dlt[ nuh_layer_id ] ) : BitDepthY | 9 | Table Error: Reference source not found-4 | 0 |
|  | 1 |  | 9 | Table Error: Reference source not found-4 | 20 |
|  | 2 |  | 9 | Table Error: Reference source not found-4 | 40 |
| sdc_pred_mode | 0 | U [Ed. (GT): in implementation 0 and 1 are switched. ] [Ed. (GT): Could be TU, cMax = 3] | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |

Table Error: Reference source not found-6 – Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| dmm_flag | 0 | na | na | na | na | na |
| dmm_mode | 0 | 0 | 0 | 0 | 0 | 0 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_delta_end_flag | 0 | na | na | na | na | na |
| dmm_delta_end_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| edge_code | 0 | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| mvp_l0_idx, mvp_l1_idx | 0 | 1 | na | na | na | na |
| sdc_flag | 0,1,2 (subclause G.9.3.3.1.1) | na | na | na | na | na |
| sdc_residual_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_sign_flag dmm_dc_1_sign_flag dmm_dc_2_sign_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_abs_minus1 dmm_dc_1_abs dmm_dc_2_abs | 0,10 (subclause Error: Reference source not found) |  |  |  |  |  |
| sdc_pred_mode | 0 | 1 | 2 | na | na | na |

Table Error: Reference source not found-7 – Specification of ctxIdxInc using segment index

| Syntax element | IntraPredMode | SdcSegmentIndex | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_residual_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_sign_flag dmm_dc_1_sign_flag dmm_dc_2_sign_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_abs_minus1 dmm_dc_1_abs dmm_dc_2_abs | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 10 |

# Cross check

The cross check of both tests was done by Qualcomm. They did not find any issues with the source code and the simulation results perfectly matched.

# Conclusion

The results show that the tested methods achieve a small BD-rate gain (0.1-0.2%) for CTC and no gain for all-intra configuration. The tested methods have no impact on encoder and decoder complexity. The changes to the draft text are small, allowing for further harmonizing the specification of SDC and DMM. In conclusion, deltaDC coding of DMM is slightly improved by combining it with DLT. Therefore, an additional test was carried out, combining the changes of Test2 with the modified contexts and binarization scheme tested as part of CE6 and proposed in JCT3V-D0032. Assuming JCT3V-D0032 as the reference, the results show that no gain (0.0% for CTC and all-intra, detailed results can be found in the annex) is achieved for using DLT with deltaDC. In summary, the impact of DLT on DMM deltaDC coding is small and Test1 configuration might be preferred over Test2, as being independent from the contexts and binarization scheme of SDC.

# References

- P. Merkle, "Description of Core Experiment 6 (CE6) on Depth Intra Coding", JCT3V-C1106, Geneva, Switzerland, January 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-C1100, Geneva, Switzerland, January 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3,” JCT3V-C1005, Geneva, Switzerland, January 2013.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

Table A1. Test2 using JCT3V-D0032: results for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 99.7% | 100.0% | 99.9% |
| Kendo | 0.0% | 0.0% | -0.1% | 99.8% | 97.8% | 99.8% |
| Newspapercc | 0.0% | 0.0% | -0.1% | 99.7% | 98.9% | 98.1% |
| GhostTownFly | 0.0% | 0.0% | 0.1% | 99.7% | 99.1% | 100.3% |
| PoznanHall2 | 0.0% | 0.0% | 0.1% | 100.0% | 100.7% | 99.1% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 100.0% | 99.0% | 98.0% |
| UndoDancer | 0.0% | 0.0% | 0.2% | 100.3% | 107.6% | 100.8% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 99.7% | 98.9% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 100.0% | 101.6% | 99.5% |
| average | 0.0% | 0.0% | 0.0% | 99.9% | 100.4% | 99.4% |

Table A2. Test2 using JCT3V-D0032: results for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.2% | 99.7% | 98.9% | 96.4% |
| Kendo | 0.0% | 0.0% | 0.2% | 100.4% | 99.5% | 99.1% |
| Newspapercc | 0.0% | 0.0% | -0.3% | 100.0% | 98.8% | 99.5% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 100.2% | 100.2% | 102.4% |
| PoznanHall2 | 0.0% | 0.0% | 0.2% | 99.9% | 95.7% | 98.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 100.1% | 100.5% | 101.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 100.1% | 99.5% | 101.8% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 100.0% | 99.1% | 98.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 100.1% | 99.0% | 101.0% |
| average | 0.0% | 0.0% | 0.0% | 100.1% | 99.0% | 99.9% |