| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0201 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0041 |

| Title: | MV-HEVC/SHVC HLS: On access unit definition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

In draft MV-HEVC and SHVC specifications, an access unit is defined to contain one coded picture (with a particular value of nuh_layer_id). Another approach is to define an access unit to contain all pictures with the same POC value (and with any value of nuh_layer_id). This contribution compares the two approaches and asserts that the current access unit definition in MV-HEVC and SHVC is more favourable (i) in terms of the number of specification text changes in HEVC version 1, (ii) for HRD operation, and (iii) for providing encoders flexibility in standards scalability and in determination of layer-specific prediction structures. It is therefore proposed to make no changes in the access unit definition of the current draft MV-HEVC and SHVC specifications.

# Discussion

## Options to Define an Access Unit

An access unit has been defined as follows in HEVC:

access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded picture.

NOTE 1 – In addition to containing the VCL NAL units of the coded picture, an access unit may also contain non-VCL NAL units. The decoding of an access unit always results in a decoded picture.

An access unit (AU) can be defined in scalable extensions in two ways:

- “AU = one coded picture”

In a first approach, which is currently in use in the MV-HEVC and SHVC draft specification texts, the definition of the access unit is kept unchanged compared to that in HEVC (as reprinted above). In other words, in MV-HEVC and SHVC an access unit contains one coded picture (with a particular value of nuh_layer_id). In MV-HEVC, a coded picture is defined essentially identically to a view component in MVC.

- “AU = all pictures of the same POC”

In a second approach an access unit is defined to contain all pictures or view components with the same POC value. There are two slightly different options how such an access unit definition could be specified:

- By specifying a coded picture to consist of VCL NAL units of a particular nuh_layer_id value, an access unit to consist of a coded picture with nuh_layer_id equal to 0 and zero or more coded picture pictures with a non-zero nuh_layer_id.

- By defining a coded picture to consist of VCL NAL units of nuh_layer_id equal to 0 (only), a layer picture to consist of VCL NAL units of a particular non-zero nuh_layer_id, an access unit to consist a coded picture and zero or more layer pictures.

## Analysis

It is believed that the approaches outlined in the previous sub-section have the following differences:

- The amount of the required changes in HEVC v1 specification.

It is believed that the first approach is fully implemented already in the current MV-HEVC and SHVC draft specifications.

For the second approach, all occurrences of HEVC v1 with the assumption of an access unit containing one coded picture only should be rewritten. For example, there is phrasing, such as the following, in the HRD specification:

“When the coded picture in access unit 0 has NalUnitType equal to…”

“Let picture n be the coded picture or the decoded picture of access unit n.” (The phrase “picture n” is subsequently used in many occasions.)

Furthermore, the second approach would require specifying a number of definitions again.

In option 2b, both coded pictures and layer pictures should be consistently referred to in decoding processes.

- HRD operation.

The CPB operates on access unit basis. Consequently, in the first approach (“AU = one coded picture”), a coded picture is removed from the CPB immediately after its decoding and the corresponding decoded picture is inserted into the DPB at that time. In the second approach, all coded pictures of the same POC value are decoded before their removal from the CPB, at which time the corresponding decoded pictures are inserted into the DPB.

Due to more frequent removal of coded pictures from the CPB, the CPB occupancy level is believed to be more stable in the first approach. Moreover the CPB operation of the first approach is believed to resemble actual multi-loop decoding operation.

In the second approach (“AU = all pictures of the same POC”), decoder implementations have to consider the storage for those decoded pictures of the AU that have been decoded earlier than the CPB removal time of the AU. This may have an implication that decoder implementations have to reserve additional decoded picture storage buffers (the amount of which is probably equal to the max number of layers in the used profile – 1) in addition to those needed for the DPB operation.

The CPB operation of the second approach could be made close to that of the first approach, when decoding unit (DU) level HRD operation is applied to remove parts of an AU from the CPB at different times (e.g. one DU can be a coded picture). However, all decoded pictures of an AU are input to the DPB at the same time (which is the removal time of the last DU of the AU) – hence, the same requirement for additional decoded picture storage buffers as in the AU-based CPB operation for the second approach remains.

- Flexibility for standards scalability and layer-specific prediction structures.

In standards scalability, the base layer may be encoded under profile and level and with coding tools that allow only certain types of prediction hierarchies. For example, if MPEG-2 base layer is in use, conventional non-hierarchical temporal scalability is in use, such as IBP, IBBP, or IBBBP, where the B pictures are predicted from the adjacent I and P pictures. However, it is typically preferable (for compression efficiency) to use hierarchical prediction structures with HEVC.

Even if both the base layer were coded with HEVC, it would be desirable to let the encoder to have the flexibility to choose a different GOP length between layers in order to:

- Improve the compression efficiency by having longer GOPs in the enhancement layer than in the base layer.

- Reduce the DPB memory use by having shorter GOPs in the enhancement layer than in the base layer.

The improvement in compression efficiency was studied in JCTVC-L0171.

Out of the two options to define an access unit, only the first one (“AU = one coded picture”) provides the flexibility of having different prediction structures or GOP sizes in different layers.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).