| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0205r1 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0042r1 |

| Title: | MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kemal Ugur Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes:

- For MV-HEVC and SHVC (both reference-index-based and textureRL-based), the VPS extension syntax is proposed to be extended to include an indication whether a reference layer is used for sample prediction, syntax prediction, or both.

- For MV-HEVC and reference-index-based SHVC, the variables and decoding processes related to the inter-layer reference picture set and inter-layer reference pictures in reference picture lists are proposed to apply only to those layers that are indicated to be references for sample prediction in the VPS extension.

- For MV-HEVC and reference-index-based SHVC, it is proposed to use one slice_reserved_flag (named alt_collocated_indication_flag) to specify whether a reference index (to a reference picture list) or an inter-layer reference index (to a list of reference pictures that are indicated to be references for motion prediction in the VPS extension) is present in case both are allowed by VPS and SPS level signalling. When alt_collocated_indication_flag is equal to 1, the inter-layer reference index is present in the slice segment header.

- It is proposed to remove sps_inter_layer_mfm_enable_flag from the SPS extension syntax specified in the SHVC test model. For specifying the decoding processes, alt_collocated_indication_flag is proposed to be used instead of sps_inter_layer_mfm_enable_flag.

Revision 1 of the contribution includes an alternative for the proposed slice segment header syntax.

# Introduction

Contributions JCTVC-L0071 and JCTVC-L0278 proposed indicating inter-layer texture prediction and inter-layer syntax prediction separately. Inspired by the mentioned contributions, this contribution proposes VPS extension syntax for indicating types of prediction for each reference layer in an extensible manner.

The design approach taken in this contribution is motivated by the following observations and opinions:

- The SHVC Test Model (JCTVC-L1007) includes the sps_inter_layer_mfm_enable_flag in the SPS extension, while all other syntax elements related to inter-layer prediction dependencies reside in the VPS extension. In this contribution, it is considered to be a better design choice to include all inter-layer prediction dependency indications in the VPS extension syntax. That way, the same SPS could potentially be used for a greater number of layers and some bitrate savings could potentially be reached.

- The reference picture lists can be shorter if inter-layer reference pictures not used as reference for sample prediction are excluded from reference picture lists. Consequently, a rate-distortion gain can potentially be obtained in reference index coding.

- In reference-index-based scalability, the only type of syntax prediction is motion prediction through the temporal motion vector prediction (TMVP) mechanism. This contribution takes the approach to modify the slice-header-level indication for the inter-layer collocated picture in case of reference-index-based scalability. Thus, reference picture lists could potentially exclude those pictures that are not used for sample prediction but could be used as collocated pictures. Such cases can particularly occur when coding multiview-plus-depth content, where texture pictures are used as collocated pictures for motion prediction of depth pictures.

# Proposed Specification Text

## Proposed Changes in MV-HEVC and SHVC WD

### VPS Extension Syntax and Semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | ue(v) |
| } |  |
| } |  |

[Ed. (MH): The semantics below is currently specific to reference-index-based scalability, where motion prediction is the only type of syntax prediction. For texture-RL based SHVC, the semantics referring to “collocated picture” should be generalized to refer to any type of syntax prediction.]

direct_dependency_type[ i ][ j ] specifies whether a picture with nuh_layer_id equal to layer_id_in_nuh[ j ] is included in the initial reference picture lists of a picture with nuh_layer_id equal to layer_id_nuh[ i ] and whether a picture with nuh_layer_id equal to layer_id_in_nuh[ j ] is available as a collocated picture used for temporal motion vector prediction for a picture with nuh_layer_id equal to layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive.

The variables NumDirectRefLayers[ i ], NumSamplePredRefLayers[ i ], NumMotionPredRefLayers[ i ], and RefLayerId[ i ][ j ], SamplePredEnabledFlag[ i ][ j ] and MotionPredEnabledFlag[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = 0, NumDirectRefLayers[ i ] = 0, NumSamplePredRefLayers[ i ] = 0, 
	NumMotionPredRefLayers[ i ]; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ]  = =  1 ) {
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]
			SamplePredEnabledFlag[ i ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1) & 1)
			NumSamplePredRefLayers[ i ] += SamplePredEnabledFlag[ i ][ j ]
			MotionPredEnabledFlag[ i ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1) & 2) >> 1 )
			NumMotionPredRefLayers[ i ] += MotionPredEnabledFlag[ i ][ j ]
		}

### Reference picture set and list related changes in Annex G (MV-HEVC and SHVC WD)

- G.7.4.7.2	Reference picture list modification semantics

The specifications in subclause Error: Reference source not found apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPocTotalCurr is replaced by:

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)	(G-1)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPocTotalCurr++
NumPocTotalCurr += NumSamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]

…

- G.8.1.1	Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0, j = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	if( SamplePredEnabledFlag[ LayerIdInVps[ nuh_layer_id ] ][ i ] ) {
		RefPicSetInterLayer[ ij ]  = the picture with picture order count equal to PicOrderCnt and
			nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
		RefPicSetInterLayer[ ij++ ] is marked as "used for long-term reference"
	}
}

- G.8.1.2	Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.

for( i = 0; i < NumSamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

…

- G.8.3.4	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(G-2)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumSamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 

}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(G-3)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(G-4)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i< NumSamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(G-5)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

### Indication of inter-layer collocated picture

A first alternative of the slice segment header syntax uses one slice_reserved_flag to indicate whether a collocated picture for TMVP is an inter-layer reference picture.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| extraSliceHeaderBitPos = 0 |  |
| if( sps_temporal_mvp_enabled && NumMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] && num_extra_slice_header_bits > 0 ) { |  |
| alt_collocated_indication_flag | - u(1) |
| extraSliceHeaderBitPos++ |  |
| } |  |
| for( i = extraSliceHeaderBitPos0; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | - u(1) |
| … |  |
| if( slice_temporal_mvp_enabled_flag && alt_collocated_indication_flag = = 0 ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| else if( slice_temporal_mvp_enabled_flag ) |  |
| if( NumMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| … |  |

A second alternative of the slice segment header syntax:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| AltCollocatedIndicationFlag = 0 |  |
| if( nuh_layer_id > 0 && NumMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ) { |  |
| alt_collocated_indication_flag | u(1) |
| AltCollocatedIndicationFlag = alt_collocated_indication_flag |  |
| } |  |
| if( AltCollocatedIndicationFlag ) { |  |
| if( NumMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| … |  |

alt_collocated_indication_flag equal to 0 specifies that, when slice_temporal_mvp_enabled_flag is equal to 1, a collocated picture for temporal motion vector prediction is indicated using collocated_from_l0_flag, if present, and collocated_ref_idx. alt_collocated_indication_flag equal to 1 specifies that, when slice_temporal_mvp_enabled_flag is equal to 1, a collocated picture for temporal motion vector prediction is indicated using collocated_ref_layer_idx. When alt_collocated_indication_flag is not present, it is inferred to be equal to 0.

…

collocated_ref_layer_idx specifies the collocated picture for temporal motion vector prediction as specified in subclause 8.5.3.2.7. When alt_collocated_indication_flag is equal to 1 and NumMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] is equal to 1, collocated_ref_layer_idx is inferred to be equal to 0. collocated_ref_layer_idx shall be in the range of 0 to NumMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1, inclusive.

- 8.5.3.2.7	Derivation process for temporal luma motion vector prediction

…

- Depending on the values of alt_collocated_indication_flag, collocated_ref_layer_idx, slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

- If alt_collocated_indication_flag is equal to 1, colPic is derived as follows

for( currIdx = 0, i = NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1; i >= 0; i-- )
	if( MotionPredEnabledFlag[ LayerIdInVps[ nuh_layer_id ] ][ i ] ) {
		if( currIdx  = =  collocated_ref_layer_idx )
			colPic = the picture with picture order count equal to PicOrderCnt and
				nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
		currIdx++
	}

- Otherwise, if slice_type is equal to B and collocated_from_l0_flag is equal to 0, colPic is set equal to RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), colPic is set equal to RefPicList0[ collocated_ref_idx ].

## Proposed Changes in SHVC Test Model Text

- G.7.3.2.2.1	Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_inter_layer_mfm_enable_flag | u(1) |
| } |  |

All occurrences of sps_inter_layer_mfm_enable_flag are replaced by alt_collocated_indication_flag.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).