| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0207r1 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0044r1 |

| Title: | MV-HEVC/SHVC HLS: Step-wise layer access (STLA) pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted in this contribution that it would be beneficial to enable activation of layer SPSs at non-IDR pictures for example to:

- Provide the encoder the flexibility to change coding modes controlled by syntax elements in the SPS separately for the enhancement layer than for the base layer, but not require the encoder to code an IDR picture across all layers when new active layer SPS is taken into use.

- Enable changing the spatial resolution of the enhancement layer, for example to reflect the resolution of the source pictures for encoding, without a need to code an IDR picture across all layers.

This contribution proposes:

- A step-wise layer access (STLA) picture (STLA_N_LP and STLA_W_RADL), which is otherwise identical to an IDR picture with nuh_layer_id greater than 0 but needs not be aligned across layers.

- In revision 1 of the contribution, an alternative (referred to as “alternative 1”) is proposed, which can be summarized as follows:

- When an IDR picture has nuh_layer_id equal to 0, all other pictures in the same access unit shall be IDR pictures.

- IDR pictures with nuh_layer_id greater than 0 may be present in access unit where the picture with nuh_layer_id equal to 0 is a non-IDR picture.

- As IDR pictures can be non-aligned in alternative 1, no STLA picture are necessary and hence not included in the alternative 1 proposal.

- BLA pictures shall have nuh_layer_id equal to 0.

- If alternative 1 is accepted, no new NAL unit types are proposed in this contribution.

- Otherwise, if proposal 2 is accepted, STLA_N_LP and STLA_W_RADL are proposed to take the nal_unit_type values BLA_N_LP and BLA_W_RADL, respectively.

- Otherwise, STLA_N_LP and STLA_W_RADL are proposed to take nal_unit_type values RSV_IRAP_VCL22 and RSV_IRAP_VCL23, respectively.

- For alternative 1 only, it is proposed to include slice_pic_order_cnt_lsb (in the slice segment header) for IDR pictures with nuh_layer_id greater than 0.

# Motivation for the Proposals

- A step-wise layer access (STLA) picture (STLA_N_LP and STLA_W_RADL), which is otherwise identical to an IDR picture with nuh_layer_id greater than 0 but needs not be aligned across layers.

Motivation: A STLA picture may activate a new layer SPS, while a CRA picture with nuh_layer_id greater than 0 never activates a new layer SPS.

STLA pictures allow different RAP periods in different layers. For example, more frequent RAP pictures can be used in the base layer, where they are cheaper due to a smaller spatial resolution. Such flexibility was allowed in SVC too. Without STLA pictures, different RAP periods could be allowed only through CRA pictures, which cannot be used to activate a new layer SPS.

It would be beneficial to enable activation of layer SPSs at non-IDR pictures for example to:

- Provide the encoder the flexibility to change coding modes controlled by syntax elements in the SPS separately for the enhancement layer than for the base layer, but not require the encoder to code an IDR picture across all layers when new active layer SPS is taken into use.

- Enable changing the spatial resolution of the enhancement layer, for example to reflect the resolution of the source pictures for encoding, without a need to code an IDR picture across all layers.

- BLA pictures shall have nuh_layer_id equal to 0.

Motivation: At a splice point, the first access unit of the latter coded video sequence shall have an IDR picture or a BLA picture at the base layer in order to enable correct decoding. A simple requirement “BLA pictures shall have nuh_layer_id equal to 0” is sufficient to be able to omit decoding of non-base layers until they contain a RAP picture. Consequently, splicers only need to change the NAL unit type of base-layer NAL units when performing the splicing operation.

See contribution JCTVC-M0206 / JCT3V-D0043 for a discussion on starting the decoding of a bitstream from a CRA picture with nuh_layer_id equal to 0, which is an equivalent case for decoding a coded video sequence starting with a BLA picture with nuh_layer_id equal to 0.

- If proposal 2 is accepted, STLA_N_LP and STLA_W_RADL are proposed to take the nal_unit_type values BLA_N_LP and BLA_W_RADL, respectively. Otherwise, STLA_N_LP and STLA_W_RADL are proposed to take nal_unit_type values RSV_IRAP_VCL22 and RSV_IRAP_VCL23, respectively.

Motivation: There are only two reserved NAL unit type values for IRAP pictures. According to proposal 2 the BLA NAL unit type values are not in use when nuh_layer_id is greater than 0, which makes them available for the proposed STLA NAL unit types.

# Proposed Specification Text

## STLA pictures

The proposed specification text changes for proposal 1 (STLA pictures) are presented with purple below. In red and blue changes in subclauses 8.1 and 8.1.1 are from contribution JCTVC-M0206 / JCT3V-D0043 and presented here, as the proposed STLA picture affects the same subclauses. Proposals 2 and 3 of this contribution were considered trivial and hence the specification text for them is not presented.

- 3	Definitions

…

- 3.X	step-wise layer access (STLA) picture: A IRAP picture for which each NAL unit has nal_unit_type equal to STLA_N_LP or STLA_W_RADL and nuh_layer_id greater than 0.

…

- 8.1	General decoding process

…

The decoding process is specified such that all decoders will produce numerically identical cropped decoded pictures. Any decoding process that produces identical cropped decoded pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

When the current picture is an IRAP picture that is the first picture in the bitstream, the variable LayerInitialisedFlag[ 0 ] is set equal to 1, the variable LayerInitialisedFlag[ i ] is set equal to 0 for all values of i from 1 to 63, inclusive.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular nuh_layer_id is an IDR picture, a STLA picture, a BLA picture, the first picture with that particular nuh_layer_id in the bitstream in decoding order, or the first picture with that particular nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture, the variable  HandleCraAsBlaFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

When the current picture is an IRAP picture and LayerInitialised[ nuh_layer_id ] is equal to 0 and LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ] ][ j ], where j is in the range of 0 to NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1, inclusive, LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1.

When the current picture has nuh_layer_id equal to 0, the decoding process for a coded picture with nuh_layer_id equal to 0 specified in subclause  is invoked.

- 8.1.1	Decoding process for a coded picture with nuh_layer_id equal to 0

When the current picture is a BLA picture that has nal_unit_type equal to BLA_W_LP or is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable UseAltCpbParamsFlag to a value, UseAltCpbParamsFlag is set equal to the value provided by the external means.

–	Otherwise, the value of UseAltCpbParamsFlag is set equal to 0.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular nuh_layer_id in the bitstream in decoding order, or the first picture with that particular nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture, the variable  HandleCraAsBlaFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

…

- F.7.4.2.4.2	Order of VPS, SPS and PPS RBSPs and their activation

- ...

- When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a nuh_layer_id value and it is referred to by activation of a PPS RBSP for that nuh_layer_id value (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), it is activated for that nuh_layer_id value. This SPS RBSP is called the active layer SPS RBSP for that nuh_layer_id value until it is deactivated by the activation of another SPS RBSP for the same layer. An SPS RBSP, with that particular value of sps_seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means. An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from an IDR, STLA or BLA picture having that nuh_layer_id value, inclusive, until either the next IDR, STLA or BLA picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier. The nuh_layer_id value the NAL unit containing the SPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same SPS RBSP may be the active layer SPS for more than one nuh_layer_id value.

…

## Alternative 1 (“non-aligned IDR”)

In addition to the changes proposed in JCTVC-M0206 for layer-wise decoding start-up, alternative 1 requires the following change.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| … |  |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).