| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0048 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Signalling and derivation of inter-layer RPS for HEVC 3DV and scalable extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Jianle Chen Krishna Rapaka Ying Chen Xiang Li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-908-803-3888 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a change to the signalling and derivation of inter-layer RPS for the following purposes:

- The inter-layer RPS subset can be used to detect loss of pictures used for inter-layer picture reference.

- In SHVC it can be restricted that at most one reference layer picture is used in inter layer prediction for coding a picture so that up to one picture resampling process is needed to code a picture. In this case, when pictures from multiple directly dependent layers are present, and the one of the highest layer is used for inter prediction, the information is not explicitly signaled in slice header.

JCTVC-M0269 contains basically the same content as this document.

# Problem statement

Currently, in one AU (assuming that an AU consists of all pictures associated with the same output time), if a picture of an enhancement has no picture for a layer that is a direct dependent layer of the current layer, the current derivation process for the inter-layer RPS subset RefPicSetInterLayer would also include such unavailable lower-layer picture. However, there is no way for decoders to know whether an entry in RefPicSetInterLayer corresponding to no picture in the DPB is lost during transmission or was not in the original bitstream.

In the current SHVC as well as MV-HEVC working draft, it is allowed to use multiple reference layer pictures for inter layer prediction process. Compared to employing one reference layer picture for inter layer prediction, this feature is supposed to provide coding performance improvement for MV-HEVC. However, using multiple inter layer reference pictures is unlikely to improve the coding performance of SHVC. Note that in SVC for each picture at most one inter-layer reference picture is used. Meanwhile, the complexity of SHVC decoding process is increased when multiple reference layer pictures are used for inter-layer prediction because the picture resampling process may need to be performed multiple times to decode an enhancement layer picture. Therefore, it is desirable to have a design that enables easily constraining the number of inter-layer reference pictures to be up to one as in SVC, while at the same time keeping the high-level syntax design aligned with MV-HEVC.

The proposal in this document tries to address both the above-mentioned issues.

# Proposal

Additions are highlighted and removals are strikethrough.

## Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| highest_dep_layer_used_flag | u(1) |
| } |  |

...

highest_dep_layer_used_flag equal to 1 specifies that the highest dependent layer is used for inter-layer prediction for each picture that uses inter-layer prediction. highest_dep_layer_used_flag equal to 0 specifies that the highest dependent layer may or may not be used for inter-layer prediction for each picture that uses inter-layer prediction.

## General slice segment header syntax and semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] > 0 ) |  |
| num_inter_layer_ref_pics | ue(v) |
| for( i = 0; i < num_inter_layer_ref_pics; i++ ) |  |
| if( i > 0 ¦ ¦ !highest_dep_layer_used_flag ) |  |
| ref_layer_idx_delta[ i ] | ue(v) |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

...

num_inter_layer_ref_pics specifies the number of pictrues used by the current picture for inter-layer prediction. When not present, the value of num_inter_layer_ref_pics is inferred to be equal to 0. The value of num_inter_layer_ref_pics shall be in the range of 0 to NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ], inclusive.

All slices of a picture shall have the same value of num_inter_layer_ref_pics.

NOTE 1 – The value of num_inter_layer_ref_pics equal to 0 indicates that no inter-layer prediction is used for the current picture. The value may be restricted to be less than or equal to 1, e.g. by an SHVC profile, such that up to one picture is used for inter-reference reference.

ref_layer_idx_delta[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th inter-layer reference picture referred to by the current picture. When highest_dep_layer_used_flag is equal to 1 and num_inter_layer_ref_pics is greater than 0, the vlaue of ref_layer_idx_delta[ 0 ] is inferred to be equal to 0.

The variable RefPicLayerId[ i ] is derived as follows:

for( i = 0; i < num_inter_layer_ref_pics; i++) {
	currLayerIdx = LayerIdInVps[ nuh_layer_id ]
	if( i = = 0 )
		if( highest_dep_layer_used_flag )
			refLayerIdx[ 0 ] = NumDirectRefLayers[ currLayerIdx ] − 1
		else
			refLayerIdx[ 0 ] = NumDirectRefLayers[ currLayerIdx ] − 1 − ref_layer_idx_delta[ 0 ]
	else
		refLayerIdx[ i ] = refLayerIdx[ i − 1 ] + ref_layer_idx_delta[ i ] + 1
	RefPicLayerId[ i ] = RefLayerId[ currLayerIdx ][ refLayerIdx[ i ] ]
}

All slices of a picture shall have the same value of ref_layer_idx_delta.

## Reference picture list modification semantics

The specifications in subclause Error: Reference source not found apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPocTotalCurr is replaced by:

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ])
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)	(G-1)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ])
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ])
		NumPocTotalCurr++
NumPocTotalCurr += NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]
NumPocTotalCurr += num_inter_layer_ref_pics

## Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows:

for( i = 0; i < num_inter_layer_ref_pics NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	if( there is a picture picX in the DPB that has the same PicOrderCntVal as the current picture and
			nuh_layer_id equal to RefPicLayerId[ i ] ) {
		RefPicSetInterLayer[ i ]  = picX
		RefPicSetInterLayer[ i ]  = the picture	 with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ] 
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ]  = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer.

## Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies:

for( i = 0; i < num_inter_layer_ref_pics NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short- term reference"

## Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-2)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
	for( i = 0; i < num_inter_layer_ref_pics; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 

}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-3)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-4)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i< NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ; rIdx++, i++ )
	for( i = 0; i < num_inter_layer_ref_pics; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-5)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).