| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0049 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On DPB operations in HEVC 3DV and scalable extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com -858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution discusses DPB management operations in HEVC 3DV and scalable extensions, including decoded picture storage, marking, output and removal. With the assumption of multi-loop decoding in SHVC and the need for the support of spatial scalability, DPB management involving decoded pictures of different sizes becomes necessary. Currently, the mechanism for DPB management and the definition of DPB size (which is important for level definitions) in SHVC for spatial scalability are not clearly specified.

Two fundamentally different approaches are possible. One is AU-level (with the assumption that AU is defined to consist of all pictures pertaining to a particular output time), the other is picture-level. AU-level DPB management is asserted to be simple from design, implementation and editing points of view, while picture-level DPB management can be more optimal in memory requirement, at least for some non-typical scenarios.

Both approaches are summarized with some preliminary text changes provided. It is suggested that the group discusses these topics and tries to make a preliminary conclusion on which direction to go. In case no essential conclusion can be made at this point of time, it is suggested that some straw-man designs for both directions are made as some break-out group activity and the straw-man designs are documented in some kind of live document similarly as being done for VPS designs earlier.

Document JCTVC-M0270 contains basically the same content.

# Problem statement

In multi-layer video coding involving multiple spatial resolutions for different layers, particularly when multi-loop decoding is applied, as it is in SHVC, wherein at least two more layers need to be fully reconstructed during the decoding process, storage of decoded pictures of different layers into the decoded picture buffer or buffers would be needed.

In SVC, single-loop decoding is applied, thus only the highest layer needs to be fully reconstructed and only the decoded picture of the highest layer in each access unit needs to be stored.

In MVC, different views are required to have the same spatial resolution, thus multiple decoded pictures in each access unit need to be stored, but they all have the same resolution.

In MVC+D, different texture views are required to have the same spatial resolution and different depth views are required to have the same spatial resolution but can be different from the resolution of the texture views. Multiple decoded pictures (texture or depth view components) in each access unit need to be stored with different spatial resolutions.

Currently, the mechanism for DPB management operations, the definition of DPB size (which is important for level definitions) in SHVC for spatial scalability are not clearly specified.

# Preliminary AU-level DPB management design

For this approach, it is editorially more convenient to define layer component for the representation of an AU in one layer, and to define a picture to consist of all layer components in one AU. We assume these in the following descriptions.

The AU-level DPB management design is summarized as follows:

- Storing a decoded layer component into DPB happens immediately after a layer component is decoded.

- A decoded layer component is not removed from the DPB unless all decoded layer components in the AU are no longer needed for either prediction reference or output, and all layer components of each AU (referred to as decoded picture) are removed from the DPB at the same time.

- Marking of a decoded layer component as "unused for reference" is marking of the whole decoded picture as "unused for reference".

Changes for the preliminary design are highlighted in yellow, made directly on top of the HEVC version 1 specification. As can be seen, few changes are needed in the Annex C.

- C.3	Operation of the decoded picture buffer (DPB)

- C.3.1	General

The specifications in this subclause apply independently to each set of DPB parameters selected as specified in subclause Error: Reference source not found.

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers may contain a decoded picture that is marked as "used for reference" or is held for future output. The processes specified in subclauses , Error: Reference source not found and Error: Reference source not found are sequentially applied as specified below.

Each of the picture storage buffers contains all the decoded layer components of an access unit. Each entry of the picture storage buffer (decoded picture) is the entire decoded picture.

A decoded picture is marked as “unused for reference” if and only if all layer components in the access unit are marked as “unused for reference”.

A picture is an IDR picture if all layer components have nal_unit_type equal to IDR_W_DLP or IDR_N_LP.

A picture is a CRA picture if all layer components have nal_unit_type equal to CRA_NUT.

A picture is a BLA picture if all layer components have nal_unit_type equal to BLA_W_LP, BLA_W_DLP, or BLA_N_LP.

- C3.2	Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows:

–	The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS of each layer component of the current access unit is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for each layer component of the preceeding access unit, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

# Preliminary picture-level DPB management design

For this method, it is editorially more convenient to define picture as in the current SHVC WD, equivalent to layer component in above. We assume this in the following descriptions.

The picture-level DPB management design makes use of sub-DPBs and can have two options, as summarized below:

- Option 1: Each sub-DPB contains pictures of one layer only.

- Option 2: Each Sub-DPB contains picture storage buffers for storing decoded pictures of one spatial resolution of all layers with that spatial resolution.

Changes for option 1 of the preliminary design provided below, and are highlighted in yellow, made directly on top of SHVC WD1. As can be seen, relatively more changes are needed in the Annex C.

        - General

The decoded picture buffer contains sub-DPBs corresponding to each layer in the TargetDecLayerIdList, and each sub-DPB contains picture storage buffers. The number of picture storage buffers for sub-DPB associated with nuh_layer_id equal to 0 is derived from the active SPS for nuh_layer_id equal to 0. The number of picture storage buffers for each sub-DPB with non-zero nuh_layer_id value is derived from the active layer SPS for that non-zero nuh_layer_id value. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. The process for output and removal of pictures from the DPB as specified in subclause F.3.1.1.2 is invoked, followed by the invocation of the process for picture decoding, marking, additional bumping, and storage as specified in subclause F.3.1.1.3. The "bumping" process is specified in subclause Error: Reference source not found and is invoked as specified in subclauses F.3.1.1.2 and F.3.1.1.3.

        - Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows.

The decoding process for RPS as specified in subclause 8.3.2 is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture in decoding order with same nuh_layer_id value as the current picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in all the sub-DPBs in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in all the sub-DPBs in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found, and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture and when one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

–	The number of pictures in the sub-DPB associated with layer nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture in the sub-DPB associated with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures in the sub-DPB associated with with nuh_layer_id equal to currLayerId in the DPB is greater than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

        - Picture decoding, marking, additional bumping, and storage

The processes specified in this subclause happen instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture.

For each picture in the sub-DPB associated with nuh_layer_id value equal to currLayerId that is marked as "needed for output" the associated variable PicLatencyCount[ currLayerId ] is set equal to PicLatencyCount[ currLayerId ] + 1.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the sub-DPB associated with currLayerId, and the following applies:

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output" and its associated variable PicLatencyCount[ currLayerId ] is set equal to 0.

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

The current decoded picture is marked as "used for short-term reference".

When one or more of the following conditions are true, the "bumping" process specified in subclause F.3.1.1.3 is invoked repeatedly until none of the following conditions are true.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	sps_max_latency_increase_plus1[ HighestTid ] is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] that is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId..

# Discussions

The AU-level DPB management approach has the following advantages compared to the picture-level approach:

- The specification change for the operation of DPB is minimum on top of HEVC version 1 specification.

- It applies to layered codecs with multiple-loop decoding regardless of whether pictures in multiple layers have different spatial resolutions or other unaligned characteristics, such as bit depth, and chroma sampling format.

- Definitions of DPB size and related level constraints would be easy.

The picture-level DPB management has the following advantages compared to the AU-level approach:

- Can achieve optimal memory consumption when different access units in a coded video sequence contain different number of layers and when some pictures in an AU may become no longer needed for either reference or output earlier than other pictures in the same AU. This typically happens in SHVC to support combined temporal and spatial scalability and is less useful for multiview or 3D video codecs.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).