| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0051_v2 |
| --- | --- |

| Title: | AHG 13: Restriction of inter-layer prediction on tile boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gerhard Tech Yago Sanchez Karsten Suehring Robert Skupin Thomas Schierl | Email: | gerhard.tech@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to restrict inter-layer prediction for a particular tile in a dependent layer to a particular tile in a reference layer. The constraint enables coding of tiles of dependent layer, before the complete reference layer has been coded and is therefore in particular suitable for low-delay applications. To enable the tile constraint a flag in the SPS is proposed. For MV-HEVC bitstream restrictions apply when the constraint is enabled. For 3D-HEVC the decoding process is modified when the constraint is enabled.

Version 2 of the document has been revised. A missing constraint related to TMVP has been added.

# Basic idea

Parallel decoders can decode multiple layers in parallel. Without any restriction, a decoder is required to finalize decoding of all reference layers in 3D- and MV-HEVC, since any position in the reference layer could be utilized for the prediction of the dependent layer. This contribution proposes to allow an earlier decoding of tiles in the dependent layer, based on restriction of inter-layer prediction.

All predictions for a particular tile in the dependent layer are constraint to use data from the same reference layer tile. The reference layer tile is the tile in the reference layer that includes the block that is collocated to the current block in the tile of the dependent layer. The constraint prevents dependencies between the dependent layer tile and other tiles not equal to the reference layer tile.

For low delay applications this approach enables the encoding and decoding of tiles in the dependent layer, before decoding or encoding of the reference layer is finished.

For MV-HEVC only a constraint on disparity motion vectors and the loop filter is necessary. For 3D-HEVC a modification of the decoding process is proposed.

# Changes in MV-HEVC

Whether the constraint applies is signalled by a flag in the SPS extension. For MV-HEVC three bitstream constraints are proposed, when the flag is equal to 1.

- Motion vectors used for inter-layer prediction are constraint in a way that the reference samples are located in the same reference layer tile.

- When the collocated picture is in a layer different to the layer of the current picture, syntax elements shall be constraint such that the collocated motion vector only needs to be derived, when the position of the collocated block is in the (collocated) reference picture tile.

- Moreover loop filtering over tile boundaries shall be disabled in the reference layer.

Proposed Syntax and semantics changes

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_layer_pred_tile_constraint_flag | u(1) |
| inter_view_mv_vert_constraint_flag | u(1) |
| } |  |

inter_layer_pred_tile_constraint_flag specifies whether inter-layer prediction at tile boundaries is constrained in the CVS.

–	If inter_layer_pred_tile_constraint_flag is equal to 1, the following applies:

–	Motion vectors used for inter-layer prediction shall be constraint that in the fractional sample interpolation process all sample positions within the reference sample array that are used to derive prediction sample values are within the tile in the reference picture that includes the luma sample with a position relative to the top-left luma sample of the reference picture equal to the position of top-left sample of the current luma prediction block relative to the top-left sample of the current picture.

–	When the collocated picture is a picture of a layer different to the current layer, syntax elements shall be constraint such that the collocated motion vector only needs to be derived when the position of the top-left sample of the collocated block relative to the top left-sample of the collocated picture is in the tile in the collocated picture that includes the luma sample with a position relative to the top-left sample of the collocated picture equal to the position of the top-left sample of the current luma prediction block relative to the top-left sample of the current picture.

Note  The above constraint can be fulfilled by e.g. not using a collocated picture in a different layer, or by e.g. restricting merge_idx, mvp_lX_flag at tile boundaries.

–	The syntax element loop_filter_across_tiles_enabled_flag of all reference layers used for inter-layer prediction of the current layer shall be equal to 0.

–	Otherwise, ( inter_layer_pred_tile_constraint_flag is equal to 0 ), no constraints are signalled by this flag.

When not present the inter_layer_pred_tile_constraint_flag is inferred to be equal to 0.

# Changes in 3D-HEVC

Unlike MV-HEVC, where a bitstream constraint is sufficient in 3D-HEVC several parts of the decoding process need to restricted when inter_layer_pred_tile_constraint_flag is equal to 1. In particular, TMVP, NBDV, backward disparity derivation, inter-view prediction motion vector prediction, inter-view residual prediction, illumination compensation and the fractional sample interpolation process need to be slightly modified. Required changes to the draft text can be found in an additional document provided with this contribution.

TMVP

When the proposed constraint is enabled, reference blocks in the reference view that are not located within the reference layer tile are unavailable.

Neighbouring block disparity derivation

The same applies for reference blocks utilized for neighbouring block disparity derivation (NBDV). When the proposed constraint is enabled the "temporal" NBDV disparity candidate is not available, when the reference picture is an inter-layer reference picture and the reference block is not within the reference layer tile.

Backward disparity derivation

The disparity vector derived from NBDV is utilized to locate depth samples in a reference depth layer to derive a refined disparity vector. When the proposed constraint is enabled, depth sample positions are clipped to be located within the reference depth layer tile.

Inter-view motion vector prediction

The refined disparity vector is utilized to locate a reference block in an inter-view reference picture. When the proposed constraint is enabled, the refined disparity vector is clipped to the boundaries of the reference layer tile.

Inter-view residual prediction

The refined disparity vector is utilized to locate reference residual samples in an inter-view reference picture. When the proposed constraint is enabled, reference residual samples location are clipped such that they are located within the reference layer tile.

Fractional sample interpolation process

View synthesis prediction uses the fractional sample interpolation process. Moreover derived disparity vectors are used as motion vectors in this process for Inter-view sample prediction. When the proposed constraint is enabled, reference sample locations are clipped such that they are located within the reference layer tile.

Illumination compensation

Illumination compensation utilizes neighbouring samples of a reference block to calculate a compensation factor. Due to restrictions described above the reference block is located within the reference layer tile. When none of the neighbouring samples is located in reference layer tile, illumination compensation is not available.

# Conclusion

A tile restriction has been proposed to restrict inter-layer prediction for a particular tile in a dependent layer to a particular tile in a reference layer. The constraint enables decoding of tiles of dependent layer, before the complete reference layer has been decoded and is therefore in particular suitable for low-delay applications.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).