| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D00053 |
| --- | --- |

| Title: | Inter-layer reference picture set initialization for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes to have inter-layer reference picture lists be initialized in the order of closeness to the current picture. In the current design, inter-layer reference pictures are put in an ascending order, from lower layer to higher layer. However, it is against the convention of putting closer reference pictures to a current picture in the head of reference picture lists.

This contribution proposes the same contents as JCTVC-M0150 and a further improvement for MV-HEVC. For MV-HEVC, the inter-layer reference pictures can be put in a descending order of ViewId.

# Introduction

Tables 1 and 2 show the currently defined inter-layer reference picture set marking process and derivation process of related variables (NumDirectRefLayers[i] and RefLayerId[i][j]). As can be observed from Table 2, inter-layer reference pictures are put in an ascending order (from lower layer to higher layer) by the current marking process,. However, it is against the convention of putting closer reference pictures to a current picture in the head of reference picture lists. For this reason, we propose to change the order of inter-layer reference pictures to a descending order (from higher to lower layers).

G.8.1.2  Marking process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ currLayerId ] ]; i++ ) {
	rlPic = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ currLayerId ][ i ] ]
	RefPicSetInterLayer[ i ] = rsPic as derived in the clause G.8.1.4 with rlPic given as input
	rsPic is marked as "used for long-term reference"
}

Table 1. Marking process for inter-layer reference picture set (G.8.1.2 [1])

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = 0, NumDirectRefLayers[ i ] = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

Table 2. Derivation process of NumDirectReftLayers[i] and RefLayerId[i][j] (F.7.4.3.1.1 [2])

# Proposed inter-layer reference picture set initialization

We propose to initialize a reference picture set in such a way that closer inter-layer reference pictures to the current layer are listed before farther inter-layer reference pictures in each reference picture list. This follows the convention of placing closer reference pictures in reference picture lists in the version 1 HEVC and legacy H.264 specifications. The proposed change to F.7.4.3.1.1 is indicated in Table 3, where the index ‘j’ is put in a descending order from ‘i-1’ to 0.

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = i-1, NumDirectRefLayers[ i ] = 0; j >=0; j-- )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

Table 3. Proposed change in the derivation of RefLayerId[i][j]

# Improved inter-layer reference picture set initialization

RefLayerId[ i ][ j ] can be reordered in a descending order of a specific scalability dimension type. When NumScalabilityTypes is equal to 1, RefLayerId[ i ][ j ] is sorted in a descending order of the k-th scalability dimension type, where scalability_mask[ i ] is equal to 1. In MV-HEVC, RefLayerId[ i ][ j ] is reordered in a descending order of ViewId.

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = i-1, NumDirectRefLayers[ i ] = 0; j >=0; j-- )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

sort(RefLayerId[ i ][ j ], ViewId)

where sort(RefLayerId[ i ][ j ], scalability dimension) returns the reordered RefLayerId[ i ][ j ] in a descending order of scalability dimension

Table 4. Proposed change in the derivation of RefLayerId[i][j] by ViewId

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| scalability_type_priority | u(4) |
| … |  |
| } |  |

scalability_type_priority indicates a specific scalability dimension, where RefLayerId[ i ][ j ] is sorted in the descending order of the scalability dimension.

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).